| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0008-v2 |
| --- | --- |

| Title: | JVET AHG Report: High bit depth, high bit rate, and high frame rate coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | Adrian Browne Tomohiro Ikai Dmytro Rusanovskyy Xiaoyu Xiu Yue Yu | Email: | adrian.browne@sony.com ikai.tomohiro@sharp.co.jp dmytror@qti.qualcomm.com xiaoyuxiu@kwai.com yue.yu@oppo.com |
| Source: | AHG8 |  |  |

_____________________________

# Abstract

This document summarizes the activity of AHG8: High bit depth, high bit rate, and high frame rate coding between the 28th meeting in Mainz (20-28 October 2022) and the 29th meeting in Mainz (11-20 January 2023).

# Mandates

The AHG was established with the following mandates:

- Study the benefits and characteristics of VVC coding tools for high bit depth, high bit rate, and high frame rate coding.

- Produce test results with JVET-AA2018 testing conditions for high bit depth and high bit rate coding in coordination with AHG3.

- Contribute to the development of software and conformance testing for operation range extensions in coordination with AHG3 and AHG5.

# Activities

Previously, the AHG has used the main JVET reflector, jvet@lists.rwth-aachen.de, with [AHG8] in message headers, however no correspondence marked as AHG8 was sent between the 28th and 29th meetings.

The area of work for the AHG in this meeting cycle was the generation of crosschecked results for the combined VTM/HM CTC for high bit depths JVET-AA2018.

# Contributions

No contributions relevant to the study of high bit depth, high bit rate or high frame rate coding have been registered for the 29th meeting.

# Benchmarks

The following provide a comparison of VTM19.0 and HM17.0 for 12 and 16 bit content. The tables below report BD-Rate results computed using PSNR only.

## Low QP Range, BD-rate gain on PSNR.

| HDR PQ |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM19.0 over HM17.0 |  |  |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -5.73% | -6.65% | -7.35% | 2932% | 164% |
| PQ422 | -8.96% | -12.80% | -13.44% | 2473% | 158% |
| Overall | -7.34% | -9.73% | -10.39% | 2702% | 161% |
|  |  |  | LDB |  |  |
|  | VTM19.0 over HM17.0 |  |  |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -6.65% | -4.92% | -6.94% | 288% | 160% |
| PQ422 | -7.86% | -12.08% | -13.09% | 366% | 157% |
| Overall | -7.26% | -8.50% | -10.02% | 327% | 158% |
|  |  |  | RA |  |  |
|  | VTM19.0 over HM17.0 |  |  |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -6.28% | -4.63% | -5.92% | 322% | 160% |
| PQ422 | -7.83% | -11.79% | -12.02% | 450% | 158% |
| Overall | -7.05% | -8.21% | -8.97% | 386% | 159% |
| Overall PQ | -7.22% | -8.81% | -9.79% | 1139% | 159% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM19.0 over HM17.0 |  |  |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -4.58% | -6.22% | -6.60% | 3569% | 158% |
| HLG422 | -5.32% | -7.62% | -7.71% | 3021% | 152% |
| Overall | -4.95% | -6.92% | -7.16% | 3295% | 155% |
|  |  |  | LDB |  |  |
|  | VTM19.0 over HM17.0 |  |  |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -6.30% | -8.70% | -8.34% | 393% | 148% |
| HLG422 | -6.04% | -10.29% | -9.16% | 476% | 148% |
| Overall | -6.17% | -9.49% | -8.75% | 435% | 148% |
|  |  |  | RA |  |  |
|  | VTM19.0 over HM17.0 |  |  |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -6.37% | -8.59% | -7.92% | 440% | 150% |
| HLG422 | -6.05% | -9.75% | -8.20% | 594% | 153% |
| Overall | -6.21% | -9.17% | -8.06% | 517% | 152% |
| Overall HLG | -5.78% | -8.53% | -7.99% | 1416% | 152% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM19.0 over HM17.0 |  |  |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -2.64% | -1.83% | -1.85% | 5548% | 145% |
| SVT12 | -4.29% | -3.19% | -3.28% | 5389% | 145% |
| Overall | -3.47% | -2.51% | -2.56% | 5468% | 145% |
|  |  |  | LDB |  |  |
|  | VTM19.0 over HM17.0 |  |  |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -2.52% | -1.14% | -1.13% | 711% | 148% |
| SVT12 | -4.61% | -2.13% | -2.14% | 653% | 145% |
| Overall | -3.57% | -1.64% | -1.63% | 682% | 146% |
|  |  |  | RA |  |  |
|  | VTM19.0 over HM17.0 |  |  |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -2.66% | -1.20% | -1.19% | 852% | 149% |
| SVT12 | -4.71% | -2.21% | -2.22% | 793% | 145% |
| Overall | -3.69% | -1.71% | -1.71% | 822% | 147% |
| Overall RGB | -3.57% | -1.95% | -1.97% | 2324% | 146% |

## Standard QP Range, BD-rate gain on PSNR.

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM19.0 over HM17.0 |  |  |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| Class H1 | -26.50% | -69.30% | -56.97% | 290% | 177% |
| Class H2 | -33.05% | -57.93% | -57.02% | 293% | 161% |
| Overall | -29.78% | -63.62% | -56.99% | 291% | 169% |
|  | AI VTM19.0 over HM17.0 |  |  |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| Class H1 | -12.58% | -68.09% | -54.83% | 2104% | 185% |
| Class H2 | -23.06% | -49.57% | -50.26% | 2530% | 174% |
| Overall | -17.82% | -58.83% | -52.55% | 2307% | 179% |

- Recommendations

The AHG recommends the following:

- To close the group.