| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0014-v2 |
| --- | --- |

| Title: | JVET AHG report: NNVC software development AhG14 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | S. Eadie, F. Galpin, Y. Li, L. Wang, Z. Xie | Tel: Email: | seadie@qti.qualcomm.com franck.galpin@interdigital.com yue.li@bytedance.com liqiangwang@tencent.com xiezhihuang@oppo.com |
| Source: | AHG14 on NNVC software development |  |  |

_____________________________

# Abstract

This report summarizes the activities of the AHG14 on NNVC software development that has taken place between the 28th and 29th JVET meetings.

# Introduction

The mandates for the AhG are as follows:

- Coordinate development of the NNVC software and associated configuration files.

- Prepare and deliver NNVC-3.0 software version and the reference configuration encodings according to the NNVC common test conditions JVET-AB2016. Prepare and release anchor data for all configurations of the software.

- Study and maintain the SADL (Small Adhoc Deep-Learning Library). Identify gaps in functionality and develop improvements as needed.

- Coordinate with NNVC algorithm and software description (JVET-AB2019) editors to identify any mismatches between software and description document, suggest further updates to the description document as appropriate.

- Coordinate with AHG11 on items related to NNVC activities.

# Software development

## Location

NNVC repository is located at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM

NNVC software is based on VTM-11.0 with enabled MCTF including the update from JVET-V0056, GOP32, and enabling deblocking in the RDO.

NNVC-3.0 anchor at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM is used for NNVC performance evaluation.

## Software changes

The following changes were integrated into NNVC-3.0:

- add missing license headers in training scripts [MR17]

- JVET-AB0083: EE1-1.8: More refinements on NN based in-loop filter with a single model (Test 1) [MR 18]

- JVET-AB0068: EE1-1.6: RDO Considering Deep In-Loop Filtering [MR 20]

- Update to SADL v3.0 [MR 19]

- JVET-AB0053: EE1-1.2: Removed attention branch and partitioning input from filter set 1. [MR 21] [MR 23]

The following fixes:

- Add missing license headers in training scripts

- refactor code of filter set #1: correct variable name, remove constant [MR 22]

NNVC 3.0 was tagged December 1st 2022.

NCS-1.0 was tagged September 4th 2022 (first release containing the FilterSets, using NNVC 2.0 as a base).

NNVC 2.0 was tagged August 4th 2022 (add deblocking in RDO).

NNVC 1.0 was tagged May 6th 2021 (VTM-11.0 base with MCTF enabled).

## CTC performance

In this section, NNVC test results following NNVC CTC configuration descried in JVET-AB2016 are summarized.

### Filter set 0

The following tables show NNVC-3.0 NN-based filter set #0 (int16 precision) performance over NNVC-3.0 anchor.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-3.0 anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -9.26% | -13.15% | -18.91% | 160% | 56967% |
| Class A2 | -9.51% | -19.53% | -14.74% | 151% | 51368% |
| Class B | -8.59% | -19.05% | -20.78% | 158% | 57030% |
| Class C | -8.62% | -20.95% | -20.76% | 136% | 48210% |
| Class E |  |  |  |  |  |
| Overall | -8.91% | -18.47% | -19.19% | 151% | 53391% |
| Class D | -9.74% | -20.01% | -21.14% | 141% | 40159% |
| Class F | -3.79% | -12.18% | -11.09% | 191% | 23529% |
|  | Low delay B Main10 |  |  |  |  |
|  | BD-rate Over NNVC-3.0 anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -7.78% | -18.19% | -20.35% | 144% | 56136% |
| Class C | -8.26% | -20.07% | -20.45% | 124% | 45792% |
| Class E | -8.89% | -15.66% | -16.70% | 217% | 47327% |
| Overall | -8.22% | -19.03% | -20.39% | 152% | 50260% |
| Class D | -9.61% | -18.73% | -20.58% | 133% | 39906% |
| Class F | -4.19% | -10.69% | -8.72% | 195% | 23635% |
|  | All Intra Main10 |  |  |  |  |
|  | BD-rate Over NNVC-3.0 anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -6.12% | -14.26% | -17.26% | 196% | 43391% |
| Class A2 | -5.73% | -15.81% | -12.98% | 168% | 33962% |
| Class B | -5.93% | -15.53% | -17.44% | 148% | 35682% |
| Class C | -6.40% | -16.42% | -18.16% | 129% | 21838% |
| Class E | -8.83% | -15.25% | -16.12% | 173% | 38475% |
| Overall | -6.51% | -15.52% | -16.61% | 158% | 33197% |
| Class D | -6.41% | -15.30% | -18.56% | 116% | 19535% |
| Class F | -4.02% | -11.81% | -11.71% | 128% | 29605% |

The following tables show NNVC-3.0 NN-based filter set #0 (int16 precision) performance over NCS-1.0 NN-based filter set #0.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NCS-1.0 filter set #0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -0.12% | 0.89% | -0.28% | 129% | 110% |
| Class A2 | -0.14% | -0.54% | -0.33% | 128% | 108% |
| Class B | -0.29% | -0.50% | -0.50% | 122% | 113% |
| Class C | -0.26% | -0.87% | -0.52% | 115% | 112% |
| Class E |  |  |  |  |  |
| Overall | -0.22% | -0.33% | -0.43% | 122% | 111% |
| Class D | -0.18% | -0.74% | -0.89% | 119% | 102% |
| Class F | -0.08% | -0.36% | -0.21% | 119% | 107% |
|  | Low delay B Main10 |  |  |  |  |
|  | BD-rate Over NCS-1.0 filter set #0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.50% | -0.80% | -0.18% | 119% | 112% |
| Class C | -0.46% | 0.16% | -0.07% | 113% | 115% |
| Class E | -0.50% | 1.70% | -0.26% | 126% | 135% |
| Overall | -0.49% | -0.37% | -0.13% | 118% | 118% |
| Class D | -0.44% | 0.03% | -1.36% | 114% | 107% |
| Class F | -0.05% | -0.56% | 0.17% | 126% | 111% |
|  | All Intra Main10 |  |  |  |  |
|  | BD-rate Over NCS-1.0 filter set #0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 0.00% | 0.00% | 0.00% | 111% | 110% |
| Class A2 | 0.00% | 0.00% | 0.00% | 124% | 113% |
| Class B | 0.00% | 0.00% | 0.00% | 113% | 112% |
| Class C | 0.00% | 0.00% | 0.00% | 106% | 102% |
| Class E | 0.00% | 0.00% | 0.00% | 114% | 106% |
| Overall | 0.00% | 0.00% | 0.00% | 113% | 109% |
| Class D | 0.00% | 0.00% | 0.00% | 105% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 112% | 109% |

### Filter Set 1

The following tables show NNVC-3.0 NN-based filter set #1 (int16 precision) performance over NNVC-3.0 anchor.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-3.0 anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -8.89% | -15.92% | -18.72% | 189% | 39242% |
| Class A2 | -10.23% | -20.25% | -17.44% | 177% | 35733% |
| Class B | -9.05% | -23.01% | -21.66% | 197% | 39779% |
| Class C | -9.92% | -21.77% | -22.30% | 166% | 36427% |
| Class E |  |  |  |  |  |
| Overall | -9.49% | -20.71% | -20.40% | 183% | 37928% |
| Class D | -11.48% | -23.64% | -23.95% | 167% | 32995% |
| Class F | -5.02% | -11.79% | -10.59% | 256% | 16584% |
|  | Low delay B Main10 |  |  |  |  |
|  | BD-rate Over NNVC-3.0 anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -7.83% | -15.97% | -13.81% | 177% | 38000% |
| Class C | -9.14% | -15.25% | -14.54% | 154% | 35569% |
| Class E | -8.79% | -17.14% | -17.21% | 307% | 30032% |
| Overall | -8.51% | -15.65% | -14.14% | 194% | 35048% |
| Class D | -10.82% | -19.44% | -18.51% | 156% | 30052% |
| Class F | -4.94% | -9.35% | -6.37% | 249% | 16689% |
|  | All Intra Main10 |  |  |  |  |
|  | BD-rate Over NNVC-3.0 anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -6.59% | -17.96% | -19.97% | 173% | 28224% |
| Class A2 | -6.84% | -20.30% | -17.54% | 148% | 21252% |
| Class B | -6.82% | -21.91% | -21.71% | 142% | 22586% |
| Class C | -7.61% | -18.66% | -21.87% | 131% | 14756% |
| Class E | -10.20% | -20.84% | -20.64% | 151% | 24992% |
| Overall | -7.52% | -20.08% | -20.58% | 147% | 21469% |
| Class D | -7.46% | -19.01% | -21.05% | 118% | 12569% |
| Class F | -5.28% | -11.80% | -10.96% | 124% | 19230% |

The following tables show NNVC-3.0 NN-based filter set #1 (int16 precision) performance over NCS-1.0 NN-based filter set #1.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NCS-1.0 filter set #1 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 0.03% | 0.04% | -0.25% | 111% | 105% |
| Class A2 | -0.09% | -0.05% | -0.16% | 112% | 105% |
| Class B | -0.05% | 0.06% | 0.08% | 114% | 108% |
| Class C | -0.11% | 0.01% | 0.04% | 103% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.06% | 0.02% | -0.04% | 110% | 105% |
| Class D | -0.05% | -0.18% | -0.09% | 100% | 105% |
| Class F | -0.35% | -0.10% | 0.09% | 105% | 95% |
|  | Low delay B Main10 |  |  |  |  |
|  | BD-rate Over NCS-1.0 filter set #1 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | 0.02% | 0.24% | 107% | 102% |
| Class C | -0.04% | -0.06% | 0.40% | 108% | 109% |
| Class E | -0.18% | 0.13% | -0.13% | 111% | 107% |
| Overall | -0.07% | -0.02% | 0.31% | 108% | 106% |
| Class D | -0.05% | -0.80% | -0.56% | 101% | 104% |
| Class F | 0.00% | -0.26% | -0.21% | 110% | 107% |
|  | All Intra Main10 |  |  |  |  |
|  | BD-rate Over NCS-1.0 filter set #1 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -0.14% | -0.05% | -0.13% | 98% | 88% |
| Class A2 | -0.37% | 0.19% | 0.33% | 106% | 86% |
| Class B | -0.32% | -0.11% | -0.22% | 103% | 86% |
| Class C | -0.24% | 0.23% | -0.15% | 94% | 82% |
| Class E | -0.37% | 0.01% | 0.03% | 97% | 85% |
| Overall | -0.29% | 0.05% | -0.06% | 100% | 85% |
| Class D | -0.15% | 0.55% | 0.42% | 88% | 82% |
| Class F | -1.24% | -0.04% | -0.02% | 104% | 107% |

The Excel files with the complete NNVC results are attached to this report.

# Discussions

The following points were discussed or are still discussed in the group:

- The setting of a better solution for models’ storage in order to avoid repository excess size.

- Upgrade the VTM version to benefit from bug corrections from AhG3.

- To enforce adopted contributions to merge into existing code, including for the training scripts if they exist (see issue with [MR 21/23]).

- Issue raised on the encoder invariance: likely due to an aggressive optimization compilation option affecting all float operations in the encoder.

# Recommendations

The AHG recommends to:

- Continue to develop NNVC software.

- Improve the software documentation.

- Encourage people to report all (potential) bugs that they are finding using GitLab Issues functionality https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/issues

- Encourage people to submit merge requests fixing identified bugs.