| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0023-v3 |
| --- | --- |

| Title: | EE1: Summary of Exploration Experiments on Neural Network-based Video Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | E. Alshina, F. Galpin, Y. Li, M. Santamaria, J. Ström, H. Wang, L. Wang and Z. Xie | Tel: Email: | elena.alshina@huawei.com franck.galpin@interdigital.com yue.li@bytedance.com maria.santamaria_gomez@nokia.com hongtaow@qti.qualcomm.com jacob.strom@ericsson.com liqiangwang@tencent.com xiezhihuang@oppo.com |
| Source: | EE Coordinators |  |  |

_____________________________

# Abstract

This document summarizes Exploration Experiment 1 (EE1) tests performed between the JVET-AB and JVET-AC meetings to evaluate Neural Network-based Video Coding (NNVC) technologies, analyze their performance and complexity aspects. EE1 conducted tests in three categories: NN-based in-loop and post-filters, adaptive resolution coding with “classical” and NN-based filters, NN-based Intra coding. Average across JVET test set compression gain over VVC anchor achieved in those categories for random access configuration is 10.2% (filters), 2.1% (adaptive resolution coding with NN-re-sampler), 1.6% (NN-based Intra).

# Introduction

Group continues evaluation of new promising NN-based video coding technologies, answering questions and addressing suggestions from JVET members made during presentation of NN-based technologies at JVET-AB meeting.

NNVC common SW base (NNVC) and training scripts are highly encouraged to be used by all EE1 proponents. All proponents were asked to report results relatively to the AhG11 anchor NNVC-3.0 [1] and use up-to-date version of results reporting template recommended by AhG 11. It was the case for all EE1 test except test on end-to-end AI picture coding (which was tested using very different test conditions). Some proponents still don’t report MS-SSIM metric, even support for it was added long time ago.

During this round of Exploration Experiment tests were massively cross-checked, which indicates that more and more JVET members become familiar with testing and training NN-based video coding technologies. Massive cross-check was performed for both model training and for inference (coding).

In testing the performance difference up to 0.1% BD-rate is reported. It must be asserted that with NN-based filters enabled in NNVC SW same level of performance deviation is observed (even for quantized to int16 models). It was explained by NNVC SW coordinators that issue is known and likely called by usage of different compliers. Hopefully issue will be solved in AhG14.

The most promising technologies recommended by JVET undergo procedure of cross-check for the training. If NN model was re-trained by cross-checker and then in test performance difference between model from proponent and cross-checker is even larger (up to 0.5 %).

Figure 1 Complexity performance analysis of all EE1 test: Random Access cfg BD-rate vs kMAC/pxl (top), All intra cfg. BD-rate vs kMAC/pxl (middle), Random Access cfgBD-rate vs kMAC/pxl (bottom).

Complexity for all EE1 test has been analyzed and summarized according to AhG 11 recommendations. BD-rate vs computational complexity (kMAC/pxl) and total memory for model parameters is shown in Figure 1.

# Exploration experiments on Enhancement filters

There are two technologies in this category, which are part of NNVC SW, known as filter set #0 and filter set#1. Those technologies are clustering around level of complexity 600~700 kMAC/pxl and demonstrate BD-rate gain vs VVC anchor 9~10%. Filter set#0 filters family got slightly higher performance improvement during this EE1 round (compare to filter set#1 family) w/o increment kMAC/pxl and memory.

Figure 2 Typical representatives of exploration experiment tests on NN-based enhancement filters: BD-rate gain over VVC in random access configuration vs kMAC/px (top); vs total memory size for all models (bottom).

## NNVC filter set#0 based

Table 1 EE1 tests for filter set#0 architecture

| EE1- Test | document | Random access, AhG11 CTC |  |  | All Intra, AhG11 CTC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | Cb | Cr | Y | Cb | Cr |
| NNVC-filter set#0 | JVET-AC0014 | -8.9% | -18.5% | -19.2% | -6.5% | -15.5% | -16.6% |
| EE1-1.1-1 | JVET-AC0194 | -9.2% | -19.6% | -19.2% | -6.6% | -15.3% | -16.4% |
| EE1-1.1-2 | JVET-AC0194 | -9.8% | -21.1% | -20.6% | -7.5% | -16.8% | -17.4% |
| EE1-1.1-3 | JVET-AC0194 | -10.0% | -21.4% | -20.9% | -7.6% | -16.7% | -17.5% |
| EE1-1.2-1 | JVET-AC0195 | -9.2% | -18.5% | -19.2% | -7.0% | -15.6% | -16.7% |
| EE1-1.2-2 | JVET-AC0195 | -10.2% | -21.4% | -20.9% | -8.0% | -16.8% | -17.6% |
| EE1-1.3-1 | JVET-AC0063 | -8.9% | -19.7% | -19.7% |  |  |  |
| EE1-1.4-1 | JVET-AC0064 | -8.9% | -19.4% | -19.4% |  |  |  |
| EE1-1.4-2 | JVET-AC0064 | -8.9% | -20.2% | -19.9% |  |  |  |

In all tests NN parameters precision is Int 16, total number of parameters is 1.9 Million, computational complexity 615 kMAC/pxl (assuming block wise processing).

EE1-1.1 is about filter set#0 model refinement to address change in NNVC CTC (enabling EncDbOpt). Performance of filter set#0 improved by 1.2% (all configuration) in test EE1-1.1-3 called real-iterative training. This training hint looks useful and deserves to be presented and explained to the group in details.

EE1-1.2 consideration of filter application is RDO process. In RDO simplified model is used. Similar encoder only technology is already part of NNVC for filter set #1. Performance improvement from encoder only change is 0.3% (RA cfg). The combination with “normative” modifications from EE1-1.1-3 improves performance of filter set#0 by 1.5%. The best in filter set#0 family test EE1-1.2.2 achieves 10.2% gain over VVC anchor; computational complexity 615 kMAC/pxl, only one model with 1.9 M parameters.

In test EE1-1.3 Chroma components are swapped prior to start NN-based filter, so filter trained for “Cr” component is used for “Cr” (and vice versa). Experiment was done for both filter architectures in NNVC. BD-rate gain is mostly visible for Chroma component (1...2%).

In test EE1-1.4 residual adjustment by reducing residual magnitude by value {1, 2} (content adaptively). Performance improvement looks not that impressive for random access configuration, but in low-delay B configuration the gain of this tool is 0.9% (on top of NNVC filter set#0). This fact indicates the lack of NNVC filters adaptively or not sufficient enough training of them for low-delay use case.

## NNVC filter set#1 based

Table 2 1 EE1 tests for filter set#1 architecture

| EE1-test | document | Total params, M | kMAC/pxl | Random access, AhG11 CTC |  |  | All Intra, AhG11 CTC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Y | Cb | Cr | Y | Cb | Cr |
| filter set#1 | JVET-AC0014 | 6.2 | 664 | -9.5% | -20.7% | -20.4% | -7.5% | -20.1% | -20.6% |
| 1.5-2 float | JVET-AC0089 | 3.1 | 677 | -9.6% | -21.3% | -22.0% | -7.4% | -18.2% | -20.0% |
| 1.5-2 int16 | JVET-AC0089 | 3.1 | 677 | -9.6% | -21.0% | -22.1% | -7.4% | -18.3% | -20.1% |
| 1.5-3 float | JVET-AC0089 | 3.1 | 673 | -9.6% | -21.0% | -21.5% | -7.4% | -18.2% | -19.9% |
| 1.5-3 int16 | JVET-AC0089 | 3.1 | 673 | -9.6% | -20.9% | -21.5% | -7.4% | -18.3% | -19.9% |
| 1.6 int16 | JVET-AC0143 | 6.2 | 664 | -9.5% | -20.7% | -20.4% | -7.5% | -20.0% | -20.8% |
| 1.7-1 int16 | JVET-AC0177 | 7.8 | 664 | -10.1% | -20.7% | -20.4% | -7.5% | -20.1% | -20.6% |
| 1.7-2 int16 | JVET-AC0177 | 7.8 | 664 | -10.0% | -20.7% | -20.4% | -7.5% | -20.1% | -20.6% |
| 1.7-3 int16 | JVET-AC0177 | 6.2 | 664 | -9.6% | -20.7% | -20.3% | -7.5% | -20.1% | -20.6% |
| 1.8 int16 | JVET-AC0118 | 6.2 | 664 |  |  |  | -7.5% | -20.6% | -21.3% |

In test EE1-1.5 same model is used for intra and inter (both Luma and Chroma), NNVC filter set#1 architecture slightly modified by removing attention modules. So, total size of memory needed for NNVC filter set #1 is reduced by half, computational complexity (kMAC/pxl) about the same; compression performance of NNVC filter set#1 preserved (or even slightly better). Architecture with and w/o IBP information as input to Chroma NNVC filter network was tried, performance difference is very small. Cross-check for training is still running. Inference results have been cross-checked. Modification is recommended to be considered for adoption to NNVC (need to complete training cross-check).

Test EE1-1.6 shows that partitioning information can be safely (w/o performance drop) removed from Intra Chroma models in NNVC filter set#1. In the inter chroma model, there is already no partitioning input. The training and inference were cross-checked. After re-training, the BD-rates between proponent and cross-checker deviate up to 0.05% in luma and 0.20% in chroma.

Test EE1-1.7 studies two aspects: 1) adding reference frame as extra input to NNVC filter set#1; 2) flipping input to NNVC filter set#1. Aspect#1 provides gain of 0.6% (but extra model is used and so total memory size for filter set#1 increases (6.2  7.8 million parameters). Aspect#2 provides 0.1% (RA) /0.2% (LDB) gain, doesn’t increase number of multiplications or memory for filter parameters. The best in filter set#1 family test EE1-1.7.1 achieves 10.1% gain over VVC anchor; computational complexity 664 kMAC/pxl, five models with 7.8 M parameters.

In test EE1-1.8 loss function in training was modified: weighted sum of L1 and L2 was used, weight depends on QP. Some minor Chroma gain is observed.

## Not in NNVC NN-filters

Experiment EE1-1.9 is trying to combine network architecture design elements of filter filter set #0 and filter set#1. Gain and complexity are at the same level with filters already included into NNVC.

Filters tested in experiment EE1-1.10 operate at ~30 times smaller computational complexity and ~20 times smaller model size compared to filters in NNVC SW, demonstrating promising 5% BD-rate gain over VVC anchor.

This filter is not yet cross-checked. Key elements of two variants of low complexity NN-based filter design in EE1-1.10 are decomposition if 2D convolution (33) to the sequence of two 1D convolution (31 and 13) separable convolutions with same number of channels (R=K) coupled with linear fusion of adjacent 1x1 convolutions and split of Luma and Chroma layers (Chroma layers uses less channels). Details of simple NN-based filter in EE1-10 are shown of figures below. Note that 3x1 and 1x3 convolutions are both spatial and depth-wise separable.

Figure 3 CP Decomposition + fusion of 1x1 conv layers of JVET-X0140 Baseline Model

Figure 4  Split luma and chroma model + CP Decomposition + fusion of 1x1 conv layer

Test 1.10.1 Test the model with CP decomposition and fusing adjacent 1x1 convolution with following model parameters:

- Number of hidden layers: n = 11

- Feature maps and rank: K = 24, R = 24, M = 72

- Model complexity = 16.2 kMAC/pixel

Test 1.10.2 Test the model with CP decomposition, fusing adjacent 1x1 convolution and split architecture for luma and chroma components with following model parameters:

- Number of hidden layers for luma and chroma: nY = nC = 10

- Feature maps and rank for luma and chroma split (24L, 8C):

  - Luma: KY = 24, RY = 24, MY = 72

  - Chroma: KC = 8, RC = 8, MC = 24

- Model complexity: 17.7 kMAC/pixel

## NN-based post-filters

Post-filter in experiment EE1-1.11 was recommended for the cross-check of training. Retraining was done by proponent. Performance deviation from original proposal after re-training is up 0.5%, this issue is under investigation jointly be proponent and cross-checker.

# Adaptive resolution coding and super-resolution NN-based post-filters

In EE1-2.1 adaptive resolution coding is used in combination with “no NN-based” filters: RPR filters and long-tap post filter (8-tap for luma, 6-tap for chroma). Coding resolution is selected adaptively w/o multi-pass coding. At least 0.5% gain can be achieved by extending coded picture selection by scaling factor 1.25 and 1.5 additionally to 1.0 and 2.0.

Other three tests in this category use NN-based re-sampling filters. Test results for RA configuration (average among all resolutions, not only 4K) are summarized in Table 4. Figure 5 shows example of RD-curves for VVC full resolution coding, adaptive scaling factors 1.0 or 2.0 with RPR and “NN-based filters. Curves are (almost) not crossing, quality level is close enough, bit-rate saving is obvious.

Table 4 Adaptive resolution coding with “non-NN based” (“classical”) and NN-based filters.

| EE1 test | scale | filter | BD-rate-Y | BD-rate-U | BD-rate-V |
| --- | --- | --- | --- | --- | --- |
| 2.1-1 | 1.0, 2.0 | RPR | -0.7% | 0.5% | 0.6% |
| 2.1-2 | 1.0, 2.0, 1.25, 1.5 | RPR | -1.2% | 3.2% | 2.7% |
| 2.1-3 | 1.0, 2.0, 1.25, 1.5 | RPR & long | -1.2% | 2.9% | 2.4% |
| 2.2 | 1.0, 2.0 | NN-based, 469kMAC/xpl | -2.1% | -1.0% | -0.8% |
| 2.3 | 1.0, 2.0 | NN-based, 926kMAC/xpl | -0.2% | 1.3% | 1.8% |
| 2.4 | 1.0, 2.0 | NN-based, 964kMAC/xpl | -1.0% | -0.2% | 0.5% |

Figure 5  Example of RD-curves for RA, Tango: VTM anchor (full size coding), EE1-2.1 (adaptive scale 1.0 or 2.0), RPR filters and EE2.2 (adaptive scale 1.0 or 2.0), NN-based filters

# NN-Intra coding

In test EE1-3.1 one well known end-to-end AI based picture coding (Figure 6) was implemented with SADL, tested together with RDOQ. Test set is KODAK, anchor is not AhG 11 anchor, so results of this test are not shown in EE1 summary. Through this test it was shown that quantization aware training allows solving performance problem at high rate, which shows up if static quantization is used.

Figure 6  Overview of the model architecture. N=128 and M=192.

In test EE1-3.2 studies NN-base Intra, block leaven NN-tool incorporated into VVC hybrid coding. Inference cross-check for this technology was done few meetings ago. This is the time for training cross-check (JVET-AC0290) and combination test with NNVC filter set#0 and NNVC filter set #1. The cross check was divided into three evaluation points. Two are finished and the last one is still running.

Test results are summarized in Table 5. There is no conflict in performance between NN-based Intra and filters, gain is more or less additive. Tool can be recommended to adoption to NNVC if training cross-check is successful.

Table 5 Combination results for NN-based Intra and NNVC filters.

| NN-Intra | NN-filter | # params | kMAC/pxl | BD-rate vs VVC (RA cfg) |  |  | BD-rate vs VVC (all intra cfg) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Y | U | V | Y | U | V |
| YES | no | 1.5 | 7.8 | -1.6% | -1.1% | -1.2% | -3.4% | -3.6% | -3.7% |
| NO | filter set#0 | 1.9 | 615 | -8.9% | -18.5% | -19.2% | -6.5% | -15.5% | -16.6% |
| YES | filter set#0 | 3.4 | 623 | -10.3% | -19.4% | -20.2% | -9.6% | -18.6% | -19.7% |
| NO | filter set#1 | 6.2 | 682 | -9.5% | -20.7% | -20.4% | -7.5% | -20.1% | -20.6% |
| YES | filter set#1 | 7.7 | 690 | -10.9% | -21.6% | -21.4% | -10.5% | -22.9% | -23.3% |

# Cross-check status

All cross-checker reported status of cross-check to EE coordinator in time. Most of cross-checks are registered as input contributions.

| Test | xCheck doc | Subject of the cross-check | Comments |
| --- | --- | --- | --- |
| ## NNVC filter set #0 based |  |  |  |
| Test 1.1 | JVET-AC0278 | Training and inference | Training of EE1-1.1.1 and EE1-1.1.2 have been cross-checked, performance diff 0.0x% (Luma), 0.x% (Chroma) Inference of EE1-1.1.3 has been cross-checked, results match to proponent data |
| Test 1.2 | JVET-AC0279 | Training and inference | Training of EE1-1.2.1 has been cross-checked, performance diff 0.0x% (both Luma and chroma) Inference of EE1-1.2.2 has been cross-checked, results match to proponent data |
| Test 1.3 | JVET-AC0271 | Only inference | exact match |
| Test 1.4 | JVET-AC0272 JVET-AC0286 | Only inference | exact match |
| ## NNVC filter set #1 based |  |  |  |
| Test 1.5 | JVET-AC0273 | Training and inference | Training cross-check approaching to the end; exact match for inference with model from proponent |
| Test 1.6 | JVET-AC0234 | Training and inference (SADL integer) | Training has been cross-checked, performance diff 0.0x% (Luma), 0.x% (Chroma) |
| Test 1.7 | JVET-AC0209 | Training and inference | Training was cross-checked, performance diff 0.0x% |
| Test 1.8 | JVET-AC0215 | Training and inference | Training has been cross-checked, performance diff 0.0x%; exact match for inference with model from proponent |
| ## Not in NNVC NN-filters |  |  |  |
| Test 1.9 | JVET-AC0254 | Training and inference | Training cross-check is on-going; exact match for inference with model from proponent |
| Test 1.10 | JVET-AC0241 | Only inference | Exact match for inference with model from proponent |
| ## NN-based post-filters |  |  |  |
| Test 1.11 |  | Training and inference | Training cross-check is done, performance difference 0.5% under investigation by proponent and |
| ## Super-resolution NN-based post-filters |  |  |  |
| Test 2.1 | JVET-AC0290 | Only inference | very minor deviation for RA but no impact on overall performance |
| Test 2.2 | JVET-AC0221 | Training and inference | Training was cross-checked, performance diff 0.0x% |
| Test 2.3 | JVET-AC0222 | Training and inference | Only inference cross-check, minor deviation in performance |
| Test 2.4 | JVET-AC0223 | Training and inference | Only inference cross-check, minor deviation in performance |
| ## NN-Intra coding |  |  |  |
| Test 3.1 | JVET-AC0257 | Only inference | Only inference, diff is 0.00x% |
| Test 3.2 | JVET-AC0238 JVET-AC0270 JVET-AC0274 | Training and inference | Training cross-check is on-going, half-trained model provides results close 0.1x% to proponent Inference cross-check almost perfect match |

# Recommendations

- Proponents should ensure smooth integration of adopted NN-based tools to NNVC SW (including well described training scripts)

- Adopt improvements in EE1-1.2 for NNVC filter set #0 verified though this round of EE1

- Adopt improvements EE-1.5 and EE1-1.7 for NNVC filter set #1 verified though this round of EE1

- Establish training cross-check for of EE1-1.10 (simple NN-based filter)

- Adopt EE1-1.4 for the next version of NNVC SW as generic technique improving performance of NN-base filters

- Adopt EE1-1.11 for the next version of NNVC SW as an example of post-filter

- Adopt EE1-1.2.2 for the next version of NNVC SW as an example of adaptive resolution coding with NN-based re-sampler

- Adopt EE1-1.3.2 for the next version of NNVC SW as an example of NN-based Intra tool

# References

- JVET-AB2016 Common Test Conditions and evaluation procedures for neural network-based video coding technology