| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0024-v2 |
| --- | --- |

| Title: | EE2: Summary Report on Enhanced Compression beyond VVC capability |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Vadim Seregin Jie Chen Guichun Li Karam Naser Jacob Ström Fan Wang Martin Winken Xiaoyu Xiu Kai Zhang | Email: | vseregin@qti.qualcomm.com jiechen.cj@alibaba-inc.com guichunli@global.tencent.com Karam.Naser@InterDigital.com jacob.strom@ericsson.com wangfan6@oppo.com martin.winken@hhi.fraunhofer.de xiaoyuxiu@kwai.com zhangkai.video@bytedance.com |
| Source: | EE coordinators |  |  |

_____________________________

# Abstract

This document provides a summary report of Exploration Experiment on Enhanced Compression beyond VVC capability. The tests are categorized as intra prediction, inter prediction, screen content coding, transform, and in-loop filtering.

The software basis for this EE is ECM-7.0, released at https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-7.0. ECM-7.0 is used as an anchor in the tests.

Software for EE tests is released in the corresponding branches at https://vcgit.hhi.fraunhofer.de/ecm/jvet-ab-ee2/ECM/-/branches.

Test results can be found in input JVET contributions, cross-check results are uploaded to https://vcgit.hhi.fraunhofer.de/ecm/jvet-ab-ee2/simulation-results if cross-check reports are not submitted as they are optional for EE tests.

# List of tests

|  | Tests | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1 Intra prediction |  |  |  |
| 1.1 | Directional planar prediction | LGE S. Yoo JVET-AC0082 | InterDigital K. Naser JVET-AC0261 |
| 1.2 | Improvements on planar horizontal and planar vertical mode | WILUS K. Kim JVET-AC0105 | InterDigital K. Naser JVET-AC0262 |
| 1.3 | Horizontal and vertical planar modes | Alibaba X. Li JVET-AC0084 | OPPO F. Wang JVET-AC0243 |
| 1.4 | Combination of directional planar prediction methods (Test 1.1 + Test 1.2 + Test 1.3) | LGE S. Yoo WILUS K. Kim Alibaba X. Li JVET-AC0083 | ETRI W. Lim |
| 1.6 | Chroma fusion | vivo C. Zhou JVET-AC0119 | Alibaba J. Chen |
| 1.7 | Adaptive reference region DIMD | Sharp Z. Fan JVET-AC0045 | Xiaomi P. Andrivon |
| 1.8 | Location-dependent DIMD | Nokia S. Blasi JVET-AC0098 | Xiaomi P. Andrivon |
| 1.9 | TIMD with directional blending | Nokia S. Blasi JVET-AC0099 | Xiaomi P. Andrivon |
| 1.10 | Optimizing the use of available decoded reference samples | InterDigital T. Dumas JVET-AC0094 | Kwai H.-J. Jhu JVET-AC0168 |
| 1.11a | Test 1.8 + Test 1.9 | Nokia S. Blasi JVET-AC0100 | Xiaomi P. Andrivon |
| 1.11b | Test 1.8 + Test 1.9 + Test 1.10 | Nokia S. Blasi InterDigital T. Dumas JVET-AC0101 | Xiaomi P. Andrivon |
| 1.11c | Test 1.7 + Test 1.8 + Test 1.9 + Test 1.10 | Sharp Z. Fan Nokia S. Blasi InterDigital T. Dumas JVET-AC0046 | Xiaomi P. Andrivon |
| 1.12a | Gradient and location based CCCM | Nokia R. G. Youvalari JVET-AC0054 | Qualcomm Y.-J. Chang JVET-AC0281 |
| 1.12b | Gradient based CCCM | Nokia R. G. Youvalari JVET-AC0054 | Qualcomm Y.-J. Chang JVET-AC0281 |
| 1.13 | CCCM using non-downsampled luma samples | Kwai H.-J. Jhu JVET-AC0147 | Alibaba X. Li JVET-AC0247 |
| 1.14 | No luma subsampling for CCCM | Qualcomm V. Seregin JVET-AC0147 | Alibaba X. Li JVET-AC0247 |
| 1.15 | Test 1.13 + Test 1.14 | Kwai H.-J. Jhu Qualcomm V. Seregin | withdrawn |
| 2 Inter prediction |  |  |  |
| 2.1 | Decoder-side affine model refinement | Alibaba J. Chen JVET-AC0144 | Kwai X. Xiu |
| 2.2a | Sub-block processing for affine DMVR without subblock downsampling and without linear regression | Qualcomm H. Huang Alibaba J. Chen | withdrawn |
| 2.2b | Sub-block processing for affine DMVR with subblock downsampling and without linear regression | Qualcomm H. Huang Alibaba J. Chen | withdrawn |
| 2.2 | Sub-block processing for affine DMVR | Qualcomm H. Huang Alibaba J. Chen JVET-AC0144 | Xiaomi M. Blestel |
| 2.2d | Sub-block processing for affine DMVR with subblock downsampling and with linear regression | Qualcomm H. Huang Alibaba J. Chen | withdrawn |
| 2.3a | Control-point motion vector refinement for affine DMVR step 1 | Qualcomm H. Huang JVET-AC0144 |  |
| 2.3b | Control-point motion vector refinement for affine DMVR all steps | Qualcomm H. Huang JVET-AC0144 |  |
| 2.3c | Extension of affine DMVR to other modes | Qualcomm H. Huang | withdrawn |
| 2.4a | Test 2.1 + Test 2.2 | Qualcomm H. Huang Alibaba J. Chen JVET-AC0144 | Xiaomi M. Blestel Bytedance L. Zhao |
| 2.4b | Test 2.1 + Test 2.2 + Test 2.3 step 1 | Qualcomm H. Huang Alibaba J. Chen JVET-AC0144 | Xiaomi M. Blestel Bytedance L. Zhao |
| 2.4c | Test 2.1 + Test 2.2 + Test 2.3 | Alibaba J. Chen Qualcomm H. Huang JVET-AC0144 |  |
| 2.5a | Pixel based affine motion compensation (minimum subblock size is conditionally set to be 1×1 when OBMC flag is false, otherwise 4×4) | Qualcomm Z. Zhang JVET-AC0158 | Kwai X. Xiu |
| 2.5b | Pixel based affine motion compensation (minimum subblock size is always set to be 1×1) | Qualcomm Z. Zhang JVET-AC0158 | Kwai X. Xiu |
| 2.6a | ARMC merge candidate list reordering for AMVP-merge mode for low-delay pictures | Qualcomm K. Cui LGE H. Jang JVET-AC0150 | Xiaomi F. Le Léannec |
| 2.6b | Adaptive merge candidate list reordering for AMVP-Merge mode at CU level | Qualcomm K. Cui JVET-AC0150 | Xiaomi F. Le Léannec |
| 2.7a | Enhanced temporal motion information derivation | Bytedance L. Zhao JVET-AC0185 | Alibaba J. Chen |
| 2.7b | Enhanced temporal motion information derivation with SbTMVP template matching using adjacent subblock MVs | Bytedance L. Zhao Tencent L.-F. Chen JVET-AC0185 | Google X. Li |
| 3 Screen content coding |  |  |  |
| 3.1a | Direct block vector mode for chroma prediction (using CU level flag in MODE_INTRA) | Xidian Univ. J.-Y. Huo X. Hao OPPO M. Li JVET-AC0071 | Alibaba X. Li JVET-AC0245 |
| 3.1b | Direct block vector mode for chroma prediction (without signalling) | Xidian Univ. J.-Y. Huo X. Hao OPPO M. Li JVET-AC0071 | Alibaba X. Li JVET-AC0245 |
| 3.2 | Block vector difference sign prediction for IBC blocks | Xidian Univ. J.-Y. Huo X. Hao OPPO M. Li JVET-AC0072 | Alibaba X. Li JVET-AC0246 |
| 3.3a | Block Vector Difference Prediction for IBC blocks: 6 bins (2 sign bins and 4 suffix bins) | Ofinno A. Filippov JVET-AC0104 | Alibaba R.-L. Liao JVET-AC0210 InterDigital K. Naser JVET-AC0265 |
| 3.3b | Block Vector Difference Prediction for IBC blocks: 8 bins (2 sign bins and 6 suffix bins) | Ofinno A. Filippov JVET-AC0104 | Alibaba R.-L. Liao JVET-AC0210 |
| 3.3c | Block Vector Difference Prediction for IBC blocks: 10 bins (2 sign bins and 8 suffix bins) | Ofinno A. Filippov JVET-AC0104 | Alibaba R.-L. Liao JVET-AC0210 |
| 3.3d | Block Vector Difference Prediction for IBC blocks: 12 bins (2 sign bins and 10 suffix bins) | Ofinno A. Filippov JVET-AC0104 | Alibaba R.-L. Liao JVET-AC0210 |
| 3.3e | Test 3.3a + Test 3.1a | Ofinno A. Filippov JVET-AC0104 | Xiaomi M. Radosavljevic JVET-AC0268 |
| 3.3f | Test 3.3a + Test 3.1b | Ofinno A. Filippov JVET-AC0104 | Xiaomi M. Radosavljevic JVET-AC0268 |
| 3.4 | BVP candidates clustering and BVD sign derivation for reconstruction-reordered IBC mode | Ofinno D. Ruiz JVET-AC0060 | Tencent H. Zhang JVET-AC0233 |
| 3.5 | Tests 3.1 + Test 3.2 + Test 3.3 + Test 3.4 | Xidian Univ. J.-Y. Huo X. Hao OPPO M. Li Ofinno A. Filippov Ofinno D. Ruiz JVET-AC0113 | Alibaba R.-L. Liao JVET-AC0211 |
| 3.6a | Combined IBC and intra prediction | Kwai C. Ma Bytedance Y. Wang JVET-AC0112 | OPPO L. Zhang JVET-AC0244 |
| 3.6b | Combined IBC and intra prediction (IBC-CIIP) | Bytedance Y. Wang Kwai C. Ma JVET-AC0112 | OPPO L. Zhang JVET-AC0244 |
| 3.6c | 3.6a + IBC with geometry partitioning (IBC-GPM) + IBC with local illumination compensation (IBC-LIC) | Kwai C. Ma Bytedance Y. Wang | Duplicated of 3.6f |
| 3.6d | 3.6b + IBC with geometry partitioning (IBC-GPM) + IBC with local illumination compensation (IBC-LIC) | Bytedance Y. Wang Kwai C. Ma JVET-AC0112 | OPPO L. Zhang JVET-AC0244 |
| 3.6e | Combination of 3.6a and 3.6b | Kwai C. Ma Bytedance Y. Wang JVET-AC0112 | Google X. Li JVET-AC0202 |
| 3.6f | 3.6e + IBC with geometry partitioning (IBC-GPM) + IBC with local illumination compensation (IBC-LIC) | Kwai C. Ma Bytedance Y. Wang JVET-AC0112 | Google X. Li JVET-AC0202 |
| 3.7 | Chroma IBC method as in VTM-5.0 | Tencent Y. Wang JVET-AC0080 | Google X. Li JVET-AC0133 |
| 3.8 | Test 3.1 + Test 3.7 | Tencent Y. Wang Xidian Univ. J.-Y. Huo X. Hao OPPO M. Li JVET-AC0125 | Google X. Li JVET-AC0206 |
| 4 Transform |  |  |  |
| 4.1a | Modifications of MTS and LFNST for IntraTMP coded block | WILUS D. Kim JVET-AC0115 | OPPO F. Wang JVET-AC0240 |
| 4.1b | Generation of MPM list using the derived intra prediction mode | InterDigital K. Naser JVET-AC0115 | OPPO F. Wang JVET-AC0240 |
| 4.1c | Test 4.1a + Test 4.1b | WILUS D. Kim InterDigital K. Naser JVET-AC0115 | OPPO F. Wang JVET-AC0240 |
| 4.2 | Non-separable primary transform for intra coding | Qualcomm P. Garus LGE S. Kim JVET-AC0130 | InterDigital K. Naser JVET-AC0264 |
| 4.2b | Non-separable primary transform for intra coding with only LFNST training set (to be clarified during the editing period) used in training process for NSPT | Qualcomm P. Garus LGE S. Kim | withdrawn InterDigital K. Naser |
| 5 In-loop filtering |  |  |  |
| 5.1a | Using prediction samples for adaptive loop filter | Kwai C. Ma JVET-AC0162 | Bytedance W. Yin JVET-AC0188 |
| 5.1b | Using residual samples for adaptive loop filter | Kwai C. Ma JVET-AC0162 | Bytedance W. Yin JVET-AC0188 Alibaba J. Chen |
| 5.2 | ALF with diversified extended taps | Bytedance W. Yin JVET-AC0183 | Kwai C. Ma JVET-AC0288 |
| 5.3a | Test 5.1a + Test 5.2 | Kwai C. Ma Bytedance W. Yin JVET-AC0184 | Google X. Li JVET-AC0214 |
| 5.3b | Test 5.1b + Test 5.2 | Kwai C. Ma Bytedance W. Yin JVET-AC0184 | Google X. Li JVET-AC0214 |

# Description of tests

## Intra prediction

### Test 1.1 Directional planar prediction (JVET-AC0082)

Two additional planar prediction modes are added. The horizontal planar prediction block is constructed only using the horizontal interpolation of the planar prediction as shown below:

predx,y=W-1-x*rec-1,y+x+1*recW,-1+(W≫1)≫log2W

The vertical planar prediction is also performed using the top reference samples and the below-left reference sample as follow:

predx,y=H-1-y*recx,-1+y+1*rec-1,H+(H≫1)≫log2H

If the intra mode is planar mode (MPM index 0), a flag is signalled to indicate whether conventional planar or directional planar prediction is utilized. If the mode is not a conventional planar mode, and DIMD first mode is less than the mode 34, then mode is inferred as a horizontal planar mode, otherwise a vertical planar mode.

The horizontal and vertical planar modes are considered as vertical and horizontal modes for transform selection, respectively.

### Test 1.2 Improvements on planar horizontal and planar vertical mode (JVET-AC0105)

Additional horizontal and vertical planar modes are added, two flags are signalled if an intra mode is planar to indicate whether conventional, horizontal, or vertical planar modes are used.

For horizontal and vertical planar modes, a transform derivation step is introduced, where horizontal intra direction for vertical planar mode or vertical intra direction for horizontal planar mode is used to derived MTS and LFNST sets.

### Test 1.3: Horizontal and vertical planar modes (JVET-AC0084)

For horizontal and vertical planar modes, a gradient based decoder side derivation method is proposed to decide which of the two directional planar modes is used. The horizontal gradient for each sample in the adjacent row of the current block and the vertical gradient for each sample in the adjacent column of the current block are calculated. If the sum of the absolute values of the horizontal gradients is greater than the sum of the absolute values of the vertical gradients, the vertical planar mode is used, otherwise, the horizontal planar mode is used.

For MTS and LFNST transform selection, the horizontal and vertical planar modes are modified into vertical and horizontal intra directions, respectively.

### Test 1.4: Combination of directional planar prediction methods (JVET-AC0083)

For horizontal and vertical planar modes, MTS and LFNST transform is derived using vertical and horizontal intra direction for horizontal and vertical planar modes, respectively.

Further, the directional flag is inferred either horizontal planar or vertical planar mode based on the DIMD mode.

Additional planar modes can be used for blocks with size less than or equal to 32x32.

### Test 1.6: Chroma fusion (JVET-AC0119)

In ECM, there is one chroma fusion method that the DM mode and the four default modes can be fused with the MMLM_LT mode as follows:

pred=w0*pred0+w1*pred1+(1≪(shift-1))≫shift

where pred0 is the predictor obtained by applying the non-LM mode, pred1 is the predictor obtained by applying the MMLM_LT mode and pred is the final predictor of the current chroma block. The preset weight combination, {w0, w1} is equal to {1, 3}, {3, 1} or {2, 2}.

A different fusion method is tested:

predi,j= α0∙recL'i,j+α1∙predC'i,j+α2∙midValue

where predi,j represents the final predicted chroma sample, recL'i,j represents the reconstructed luma samples, predC'i,j represents the predicted chroma samples obtained by applying the non-LM mode, midValue=1≪(bit_depth-1).

The model parameters α0, α1 and α2 are derived from the same adjacent samples (two lines) based on the LDL decomposition method used in CCCM.

There are three chroma fusion methods:

| Value | Name |
| --- | --- |
| 0 | Disable |
| 1 | Pre-set weights |
| 2 | Single linear model |
| 3 | Multiple linear model |

### Test 1.7 Adaptive reference region DIMD (JVET-AC0045)

Two modes for DIMD using left (top-left, left, and left-bottom) and above (top-left, top, and top-right) templates for mode derivation are introduced in addition to the current DIMD mode.

### Test 1.8: Location-dependent DIMD (JVET-AC0098)

In the test, the usage of sample-based blending is inferred during the DIMD mode derivation process, by analysing the histograms computed for above and left templates of the current block. The weights used to blend the DIMD predictors are derived based on the histogram calculation process.

By comparing above and left histogram magnitudes (if one if twice larger than the other one) for each DIMD mode involved in the fusion, the decision is made whether it is more dependent on the left, above template, or no directionality observed.

If no directionality is observed, the existing ECM DIMD blending is applied, otherwise higher weights per sample are assigned to the portion of the block closer to above or left boundary as follows.

For a block of size W×H, based on the derived directional property the weights are derived for each DIMD mode i involved in fusion as follows. 5 DIMD modes and planar are used for fusion in ECM.

If direction is above, then:

wix,y= wDimdi+∆i-2∆iy(H-1).

If direction is left, then:

wix,y= wDimdi+∆i-2∆ix(W-1),

where wDimd is the current weight of the DIMD derived in ECM based on the total histogram magnitudes, ∆i is pre-defined, corresponding to the largest deviation of wix,y from wDimdi, and higher values of ∆i result in a higher variation of the weights within the block.

### Test 1.9: TIMD with directional blending (JVET-AC0099)

In the test, two separate TIMD modes are derived using only above and only left templates for an additional prediction mode using TIMD with directional blending.

The left TIMD mode searching process uses the MPM list as in the existed TIMD mode, and the mode with the smallest SATD cost is identified.

For the above TIMD mode derivation, the MPM list with the two additional modes (leftMode-1, leftMode+1) based on the derived left TIMD mode is used, and the mode with the smallest SATD cost is identified.

The two TIMD predictors are fused using directional blending with sample-based weights which have higher weights for the upper portion of the block for the above TIMD mode, and higher weights for the left portion of the block for the left TIMD mode.

For a block of size W×H, the weights for the above and TIMD modes are defined as:

wAx,y= 32+32xW-32yH,

wLx,y= 64- wAx,y.

The weights are further adjusted considering SATD costs of the left and above TIMD modes, and in case one cost is much larger than the other one, the higher weights are assigned to the TIMD mode having lower cost.

The directional TIMD prediction is computed as:

dirTimdPredx,y=wAx,y*timdDirPredAx,y+wLx,y*timdDirPredLx,y+32≫6

A block level flag is introduced to indicate the mode usage which is signalled if TIMD is selected, and 1 RD check is added at the encoder side.

### Test 1.10: Optimizing the use of available decoded reference samples (JVET-AC0094)

In the test, three methods related to intra prediction are included:

- The order of MPMs depends on the availability of the above-right and bottom-left decoded reference samples.

- The DIMD template area is extended towards above-right by block width additional columns and towards bottom-left by block height additional rows, if available.

- Wide-angle intra prediction candidates are added during the TIMD derivation step if they intensively use available above-right or bottom-left decoded reference samples.

Method 1. If above right samples are not available, the last 6, 10, or 12 vertical positive intra directions including wide angles are marked as “reserved”, depending on the value of abs(floorLog2(W) - abs(floorLog2(H)).

If bottom left sample are not available, the first 6, 10, or 12 horizontal positive intra directions including wide angles are marked as “reserved”.

During the derivation of the MPM list the “reserved” list of modes is additional constructed. After adding planar mode to MPM list, left, above, bottom left, above right, and above left neighbor intra directions are checked, and if the mode is already tagged as “reserved” and is not already in the “reserve” list, it is added to the “reserve” list, otherwise it is added to the MPM list if it is not already there. Finally, the “reserved” list is appended to the current MPM list and the remaining part of the derivation of the MPM list is performed.

Method 2. In addition to extending the DIMD template area, chroma DIMD collocated template lines are changed to use the second and third line relative to the current block.

Method 3. If the above right and bottom left reference samples are available, wide angles are added to the TIMD search as shown in Figure 1, in a block of W/H=1 angles normally reserved for W/H=2 and W/H=4 are added to the TIMD search if the sample at the blue position and red position are available reconstructed samples. Similarly, this is done by checking the left and bottom left samples to add angles normally reserved for W/H=1/2 and W/H=1/4. No extra signaling is required.

Figure 1. Wide angle modes added to TIMD search.

### Test 1.11a: Combination of Tests 1.8 and 1.9 (JVET-AC0100)

This test is a combination of location-dependent DIMD (Test 1.8) and TIMD with directional blending (Test 1.9)

### Test 1.11b: Combination of Tests 1.8, 1.9, and 1.10 (JVET-AC0101)

This test is a combination of location-dependent DIMD (Test 1.8), TIMD with directional blending (Test 1.9), and optimizing the use of available decoded reference samples (Test 1.10)/

### Test 1.11c: Combination of Tests 1.7, 1.8, 1.9, and 1.10 (JVET-AC0046)

This test is a combination of adaptive reference region DIMD (Test 1.7), location-dependent DIMD (Test 1.8), TIMD with directional blending (Test 1.9), and optimizing the use of available decoded reference samples (Test 1.10).

### Test 1.12: Gradient and location based convolutional cross-component model for intra prediction (JVET-AC0054)

In the test, gradient and location information is used instead of the 4 spatial neighbor samples in the CCCM filter in the following form. The filter shape is shown in Figure 2.

predChromaVal = c0C + c1Gy + c2Gx + c3Y + c4X + c5P + c6B

Where Gy and Gx are the vertical and horizontal gradients, respectively, and are calculated as:

Gy = (2N + NW + NE) – (2S + SW + SE)

Gx = (2W + NW + SW) – (2E + NE + SE),

and Y and X parameters are the vertical and horizontal locations of the center luma sample.

Figure 2. Spatial filter for the additional CCCM mode.

The mode usage is indicated by a flag signalled when CCCM flag is true.

Two tests are performed:

In Test 1.12a, 7-tap filter C, Gy, Gx, Y, X, P, B is used.

In Test 1.12b, 5-tap filter C, Gy, Gx, P, B is used.

### Tests 1.13-1.14: CCCM using non-downsampled luma samples (JVET-AC0147)

An additional CCCM mode using non-downsampled luma samples is tested, where 3x2 filter consisting of 6-tap spatial terms, four nonlinear terms and a bias term is used shown in Figure 3. The 6-tap spatial terms correspond to 6 neighboring luma samples (i.e., L0, L1, …, L5) around the chroma sample (i.e., C) to be predicted, the four non-linear terms are derived from the samples L0, L1, L2, and L3.

C=i=05αi∙(Li-offsetLuma)+i=69αi∙(((Li-4-offsetLuma)2+β)≫bitDepth)+α10∙β+ offsetChroma

Figure 3. CCCM filter for non-downsampled luma samples.

A block level flag is used to indicate the mode usage, which is signalled if CCCM flag is true, SPS signaling is introduced to indicate whether the CCCM using non-downsampled luma samples is enabled.

In Test 1.13, encoder speed-up logic is involved which adaptively adjusts the number of RD calculations according to the SATD values of the regular intra modes, the default CCCM, and CCCM with non-downsampled luma samples.

In Test 1.14, the additional CCCM mode is added into SATD pass of the existed CCCM modes at encoder side.

## Inter prediction

### Tests 2.1-2.4: Affine DMVR (JVET-AC0144)

In Test 2.1, the non-translation parameters of affine model for affine merge coded blocks are refined. Firstly, the top-left CPMV is fixed as base MV of the affine model and the non-translation parameters are searched by minimizing the cost of bilateral matching. Then CPMVs are recalculated with the refined non-translation parameter values. Secondly, the new top-right CPMV is fixed as the base MV and the non-translation parameters are searched again to find new best values, and CPMVs are recalculated again after the search. At last, the new left-bottom CPMV is fixed as the base MV and the same search process is applied on the non-translation parameters once again to get the final refined model. For each search point in the search process, motion compensation is applied to the whole block and the SATD between two predictors of the CU are used as the cost. To reduce the complexity, different number of search rounds and search directions are set for different stages with cost based early termination.

In Test 2.2, the affine DMVR process is altered such that bilateral matching cost is calculated per sub-block. Then, the subblock bilateral matching costs and refined subblock MVs are used to determine the overall best refined control-point motion vectors for the affine merge block. The proposed method can be summarized as the following:

- Perform integer bilateral matching for subblocks. Accumulate the subblock bilateral matching cost to determine the best integer MV offset.

- Perform half-pel bilateral matching search using the best integer MV offset as initial offset and output the best MV offset that minimizes the bilateral matching cost for the same set of the subblocks of step 1.

- Perform linear regression using the refined subblock MVs from step 1 as input and output a set of control-point motion vectors.

- Compare the bilateral matching cost of the output of the steps 2 and 3 to select the CPMV set with the smallest cost.

In Test 2.3, CPMV refinement for affine DMVR is performed, wherein a different MV offset is applied to different control-point motion vectors. The method is applied to merge candidate of affine merge and affine MMVD modes and consists of the following steps:

- Step 1: For each control-point, perform bilateral matching for a block that centered by the control-points to derive the refined CPMVs as illustrated in Figure 4. Loop over combinations of initial and refined CPMVs to derive the best set of CPMVs that minimize the bilateral matching cost of the current block.

- Step 2: Iteratively further refine the CPMVs to minimize the bilateral matching cost of the current block. In each iteration, one CPMV is refined while the others are fixed.

Figure 4. Bilateral matching search for CPMVs.

In the test, adaptive DMVR method of regular inter is applied to affine merge mode, and affine DMVR is applied to affine MMVD.

In Test 2.3a, only the first step is applied, in Test 2.3b all the steps are applied.

Test 2.4a is a combination of Test 2.1 and Test 2.2.

Test 2.4b is a combination of Test 2.1, Test 2.2, and the step 1 of Test 2.3, without extensions to affine MMVD and adaptive DMVR modes.

Test 2.4c is a combination of Test 2.1, Test 2.2, and all steps of Test 2.3 and with extensions to affine MMVD and adaptive DMVR modes.

### Test 2.5: Pixel based affine motion compensation (JVET-AC0158)

In the test, the minimum affine subblock size is changed from 4x4 to 1x1 for both luma and chroma components, 1x1 subblock size allows pixel based affine MC. When affine subblock width or height is smaller than 4, PROF is disabled.

In the tests, OBMC flag is signalled only for affine AMVP coded block without MHP, and if MHP is used then OBMC flag is disabled without signalling. For affine merge mode, OBMC flag is inherited from neighbour affine block. For regular inter AMVP the OBMC flag is enabled without signalling, and OBMC is disabled for SCC classes by existed SPS flag.

In Test 2.5a, minimum affine subblock size is set to 1x1 for luma component when OBMC is not applied, minimum subblock size is always set to 1x1 for chroma components.

In Test 2.5b the minimum subblock size is set to 1x1 for all affine blocks regardless of OBMC condition and for both luma and chroma components.

### Test 2.6: ARMC merge candidate list reordering for AMVP-merge mode (JVET-AC0150)

In Test 2.6a, ARMC is applied to reorder merge candidates of AMVP-merge mode. A merge candidate with the smallest TM cost is selected after ARMC reordering to pair with the first AMVP candidate and the MVP index is not signalled. The method is enabled on a slice level based on the existed LDC check. Compared to ECM-7.0, the number of template matching based MV refinement operations is reduced from 2 to 1 and the number of rate-distortion cost check operations is also reduced from 2 to 1.

In Test 2.6b, a selection of bilateral matching cost or template matching cost merge candidate list reordering is applied on a block level and flag is signalled to indicate the choice. It is applied only to random access configuration since the candidates of low-delay B pictures do not meet the requirement of bilateral matching cost reordering. The merge candidate list is sorted with BM or TM cost. The least cost merge candidate is paired with the AMVP candidate as the candidate pair of AMVP-merge mode.

### Test 2.7: Enhanced temporal motion information derivation (JVET-AC0185)

In this test, two aspects are included for TMVP derivation:

- two collocated pictures are utilized

- the motion shift to locate TMVP is adaptively determined from multiple locations according to template costs

Two reference frames with the least POC distance relative to the current pictures are selected to be the collocated pictures and two motion shift candidate lists are constructed for each collocated picture. The motion shifts with the minimum template matching cost are used to derive SbTMVP or TMVP candidates. At most 4 SbTMVP candidates are included in the sub-block-based merge list. The SbTMVP candidate with the least template matching cost derived from the first collocated picture is placed in the first entry without reordering, while other SbTMVP candidates are sorted together with affine candidates.

In Test 2.7a, the prediction direction of each subblock template is determined based on the center subblock. As illustrated in Figure 5(a), if the center subblock is uni-predicted, then all the subblock templates are uni-predicted, and vice versa. If the MV of corresponding adjacent subblock at the determined reference list is not available for a subblock template, zero MV is used for that subblock template.

In Test 2.7b, the prediction direction and the MV of each subblock template is determined based on the corresponding adjacent subblock. As shown in Figure 5(b), when a subblock in template area has bi-prediction MV, the bi-prediction is applied to obtain the corresponding subblock reference template and if a subblock has uni-prediction MV, uni-prediction is applied to get the subblock reference template.

- EE2-2.7a                        (b) EE2-2.7b

Figure 5. Subblock templates generation of SbTMVP.

## Screen content coding

### Test 3.1: Direct block vector mode for chroma prediction (JVET-AC0071)

A direct BV mode is introduced for chroma, where chroma BV in dual tree is derived from one of the 5 collocated luma positions shown in Figure 6. The BV of the location coded in IBC or IntraTMP is used for chroma prediction.

Figure 6. Collocated luma positions.

If the luma block is coded with a RRIBC, a flip-aware BV adjustment is performed.

In Test 3.1a, the direct BV mode is signalled, then the direct BV chroma prediction is performed from one of the 5 collocated luma positions; while in Test 3.1b if chroma block is coded in DM mode and the center collocated luma location has a BV (IBC or IntraTMP), then the direct BV chroma prediction is performed with that BV.

### Test 3.2: Block vector difference sign prediction for IBC blocks (JVET-AC0072)

In test 3.2a, similar to MVD, BVD sign prediction is introduced, where possible BVD sign combinations shown in Figure 7 are sorted according to template matching cost, a context coded index corresponding to the true BVD sign is signalled.

Figure 7. BVD sign prediction.

Test 3.2b is a combination of Test 3.2a and Test 3.1a.

Test 3.2c is a combination of Test 3.2a and Test 3.1b.

### Test 3.3: Block Vector Difference Prediction for IBC blocks (JVET-AC0104)

In the test, BVD sign prediction with BVD suffix prediction is introduced. Similar to the existed MVD BVD sign prediction used. The suffix predictor is derived by template matching search performed around the BVP + signalled BVD prefix + non-predicted BVD suffix as shown in Figure 8, then the signalled suffix bins are derived comparing the true suffix bins and the predictor.

Figure 8. BVD template matching based suffix prediction.

In test 3.3a, 4 BVD suffix bins are predicted.

In test 3.3b, 6 BVD suffix bins are predicted.

In test 3.3c, 8 BVD suffix bins are predicted.

In test 3.3d, 10 BVD suffix bins are predicted.

Test 3.3e is a combination of Test 3.3a and Test 3.1a.

Test 3.3f is a combination of Test 3.3a and Test 3.1b.

### Test 3.4: BVP candidates clustering and BVD sign derivation for reconstruction-reordered IBC mode (JVET-AC0060)

In the test, two aspects related to IBC AMVP list construction are introduced:

- If both BV candidate components are non-zero, the clustering is performed for IBC AMVP list derivation.

- If one of BV candidate components is zero or block is coded in RRIBC, two flags are introduced to indicate such case and the non-zero component. Instead of IBC AMVP list, two new BVP candidates are derived, and the sign of the non-zero BV component is derived at decoder side.

The clustering as shown in Figure 9 with L2 distance is applied if there are more than 2 valid BV candidates and up to 6 candidates are clustered, the clustering radius is defined as

Radius=log2((cbWidth∙cbHeight)≫MIN_PU_SIZE)

The clustering method is applied in the candidate list order, and the candidates assigned to a group are removed from the list for the subsequent clusters. In each group, the BVP with a lowest TM cost is selected as the representative candidate of that group. Finally, the representative candidates of the two first groups are chosen as the candidates for the IBC AMVP list.

Figure 9. AMVP IBC candidate clustering based on the L2 distance and the TM cost.

If one of BV components is zero, a flag is signalled to indicate this case with a directional flag indicating horizontal or vertical component is non-zero. Instead of IBC AMVP list, two new BVP candidate are derived which are adjusted to the boundaries of the valid IBC search region in the direction indicated by signalled direction. The AMVP BVP0 is set to the nearest valid location to the current block (-cbWidth or -cbHeight), so the non-zero BVD is always negative, pointing to the left for a BV with a zero vertical component or to the above for a BV with a zero horizontal component. Likewise, the AMVP BVP1 is set to the farthest position from the current block in the valid reference region, that is the left boundary or the top boundary of the IBC search region. Consequently, if the BVP1 is selected, the BVD always is positive, pointing to the right for BV with a zero vertical component or to the bottom for BV with a zero horizontal component.

The optimal IBC AMVP index is signalled, which allows deriving the sign of the non-zero BVD component at the decoder side. The absolute magnitude of non-zero BVD component is further signalled. In RRIBC, the direction of the flipping mode is derived from the signalled directional flag.

### Test 3.5: Combination of Test 3.1, Test 3.2, Test 3.3a, and Test 3.4 (JVET-AC0113)

The following combinations are tested:

- Test3.5a is a combination of Test 3.1a, Test 3.3a and Test 3.4.

- Test3.5b is a combination of Test 3.1b, Test 3.3a and Test 3.4.

### Test 3.6: Combined IBC and intra prediction (JVET-AC0112)

The following combinations of IBC and intra prediction methods are introduced:

- IBC and intra prediction (2 methods)

- Geometry partitioning for IBC and intra prediction combination

- IBC with local illumination compensation

IBC and intra prediction

In test 3.6a, IBC merge mode is combined with intra prediction as a new coding mode.

For DC and planar intra modes, the weighting combination is performed between the two predictors as follows:

P=wibc*Pibc+(1≪shift-wibc)*Pintra+(1≪(shift-1))≫shift,

where (wibc, shift) are set equal to (1, 2) if both the above and left CUs are intra coded, (2, 2) if one of the above or left CUs are intra coded, (3, 2) if both the above and left CUs are IBC coded.

For angular intra modes, the final prediction is obtained by adaptively switching the prediction samples of the intra mode and the IBC. Let the current CU has size w x h and the intra mode is one of horizontal modes (2 <= angular mode index < 34) or vertical modes (34 <= angular mode index <= 66), the left 3/4w*h part (horizontal modes) or top w*3/4h part (vertical modes) of the final prediction is set to intra prediction signal if both the top and left neighboring CUs are intra coded; and the left 1/2w*h part (horizontal modes) or the top w*1/2h part (vertical modes) of the final prediction is set to intra prediction signal if only one of the top and left CUs are intra coded; and the left 1/4w*h part (horizontal modes) or the top w*1/4h part (vertical modes) of the final prediction is set to intra prediction signal if both the up and left CUs are IBC or inter coded. In the above, besides the intra prediction portion, the other part of the final prediction is set to the IBC prediction samples.

In Test 3.6b, IBC and intra prediction is combined as

P=wibc*Pibc+(1≪shift-wibc)*Pintra+(1≪(shift-1))≫shift,

where (wibc, shift) are set equal to (13, 4) and (1, 1) for IBC merge and IBC AMVP modes. An intra prediction mode candidate list of size 2 is used to generate the intra prediction signal. An IPM index is signalled to indicate which IPM is used.

In Test 3.6e, a direct combination of Test 3.6a and 3.6b is introduced, one flag is introduced in SPS level to indicate whether the blending method from Test 3.6a or Test 3.6b is utilized for IBC merge mode. At encoder side, the flag is determined by the hash block hit percent which is already available at ECM encoder.

Geometry partitioning for IBC and intra prediction combination.

Geometry partitioning for IBC and intra prediction combination is introduced, where the prediction signals of the two sub-partitions are generated by either using IBC or intra prediction. An intra prediction mode candidate list of size 3 is constructed using the same method as GPM with inter and intra prediction. There are 48 geometry partitioning modes in total divided into 2 sets. A set flag is signalled to indicate whether the first or the second geometry partitioning mode set is selected, followed by the geometry partitioning mode index.

The usage of either intra or IBC prediction per sub-partition is indicated, with further signalled intra prediction mode index or merge index.

IBC with local illumination compensation

LIC is introduced for IBC, which is applied to IBC AMVP and IBC merge modes. For IBC AMVP mode, a LIC flag is signalled, while in IBC merge mode, the LIC flag is inferred from the merge candidate. LIC parameters are derived in the same way as it is done for inter prediction.

In Test 3.6d, Test 3.6b with GPM for IBC and intra, and LIC for IBC methods are included.

In Test 3.6f, Test 3.6e with GPM for IBC and intra, and LIC for IBC methods are included.

In Test 3.6g, Test 3.6b with GPM for IBC and intra are included.

### Test 3.7: Chroma IBC method as in VTM-5.0 (JVET-AC0080)

In VTM-5.0, IBC mode is applied for chroma component in dual tree partition. The chroma IBC mode does not need to signal BV, but directly derives the block vector from its co-located luma position. For each 2x2 chroma sub-block, a BV is derived as followed and used for motion compensation:

bvC[ 0 ] = ( bvL[ 0 ] >> ( 3 + SubWidthC ) ) * 32

bvC[ 1 ] = ( bvL[ 1 ] >> ( 3 + SubHeightC ) ) * 32

where bvC is the chroma block vector of 2x2 sub-block in 1/32 fractional-sample accuracy, bvL is the block vector of the co-located luma samples in 1/16 fractional-sample accuracy. The variables SubWidthC and SubHeightC depend on the chroma format sampling structure. In YCbCr 4:2:0 sampling, the values of SubWidthC and SubHeightC are equal to 2.

If one of the sub-blocks cannot find a valid block vector, the chroma IBC mode will be disabled by the encoder. A flag is signalled to indicate whether the chroma IBC mode is used or not. Besides, the following modifications are made to harmonized with existing ECM methods:

1) Luma blocks in IntraTMP mode can be used to derive the chroma BV.

2) The flip-aware BV adjustment approach is applied to refine the chroma block vector, if the co-located luma position is inside a RRIBC coded block.

### Test 3.8: Combination of Test 3.1 and Test 3.7 (JVET-AC0125)

In Test 3.8a, a BV is derived for each 2x2 sub-block according to the method in Test 3.7. If no valid BV can be found, the default BV which is derived from the pre-defined luma position in Test 3.1 will be used.

In Test 3.8b, both the BV derivation and refinement methods of Test 3.1 are used, but this mode is signalled as IBC mode rather than intra mode.

## Transform

### Test 4.1: Modifications of MTS and LFNST for IntraTMP coded block (JVET-AC0115)

In Test 4.1a, DIMD is used on the prediction samples to derive an intra prediction mode, then the MTS transform set or the LFNST transform set are determined by the derived intra mode for IntraTMP.

In Test 4.1b, the derived intra prediction mode by DIMD on the prediction samples is added to intra MPM list.

In Test 4.1c, the combination test of Test 4.1a and Test 4.1c is performed.

### Test 4.2: Non-separable primary transform for intra coding (JVET-AC0130)

In the tests, DCT-II plus LFNST transform combination is replaced by non-separable primary transform (NSPT) for certain block sizes.

All NSPT kernels consist of 35 sets and 3 candidates (similar to the current LFNST) with the following shapes:

- NSPT4x4: 16x16

- NSPT4x8/NSPT8x4: 32x20

- NSPT8x8: 64x32

- NSPT4x16/NSPT16x4: 64x24

- NSPT8x16/NSPT16x8: 128x40

Therefore, 12, 32, 40 and 88 coefficients are zeroed-out using NSPT4x8/NSPT8x4, NSPT8x8, NSPT4x16/NSPT16x4 and NSPT8x16/NSPT16x8 respectively.

In Test 4.1a, DCT-II plus LFNST transform combination is replaced with NSPT for the block shapes 4x4, 4x8, 8x4, and 8x8.

In Test 4.1b, DCT-II plus LFNST transform combination is replaced with NSPT for the block shapes 4x4, 4x8, 8x4, 8x8, 4x16, 16x4, 8x16, and 16x8.

## In-loop filtering

### Test 5.1: Using prediction samples or residual samples for adaptive loop filter (JVET-AC0162)

In the tests, the prediction samples (Test 5.1a) or the residual samples (Test 5.1b) are used as additional inputs to the ALF as shown in Figure 10.

Figure 10. Prediction or residual inputs to ALF.

In Test 5.1a, i=3131cipi+i=3232cipFilterdi term is added to ALF, where pi is the clipped difference between the collocated prediction sample and the current sample and pFilterdi is the clipped difference between the collocated prediction sample filtered by the existed fixed-filter and the current sample.

In Test 5.1b, i=3131ciri+i=3232cirFilterdi term is added to ALF, where ri is the clipped neighboring residual sample value and rFilteredi is the clipped residual sample filtered by the existed fixed-filter.

In an APS, a flag is signalled to indicate whether only prediction/residual input or both prediction/residual input with filtering by the fixed filter are used for the ALF.

In the tests, this flag is always set as true at encoder for AI test, which means only prediction/residual input is used for the ALF, and this flag is always set as false at encoder for RA/LDB test, which means both prediction/residual input with filtering by the fixed filter are used for the ALF.

### Test 5.2: ALF with diversified extended taps (JVET-AC0183)

The diversified extended taps added to ALF are shown in Figure 11, where spatial taps (i.e., tap #0 ~ #19),  reconstruction-before-DBF based taps (i.e., tap #26, #27, #38) and fixed-filter-output based taps (i.e., tap #20 ~ #25, #36, #37) are kept the same as in ECM-7.0, and several extended taps (i.e., tap #28 ~ #35, #39, # 40) are introduced into luma online-trained filters.

The diversified extended taps can take either reference samples or samples filtered by a pre-defined filter as the input source. The encoder selects the best input source and signal one flag in each APS to indicate which source is applied.

Figure 11. ALF with diversified extended taps.

The diversified extended taps are disabled in AI configuration.

### Test 5.3: Combination Tests of 5.1 and 5.2 (JVET-AC0184)

In Test 5.3a, the prediction samples (Test 5.1a) are jointly used with diversified extended taps (Test 5.2)

In Test 5.3b, the residual samples (Test 5.2b) are jointly utilized with diversified extended taps (Test 5.2).

# Test results

See the accompanied Excel table.