| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0044-v1 |
| --- | --- |

| Title: | [AHG4] Occupancy-only PSNR calculations for V3C V-PCC coding evaluation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sebastian Schwarz, Miska M. Hannuksela Nokia Technologies Munich, Germany | Email: | {sebastian.schwarz, miska.hannuksela}@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes a modified PSNR calculation for V3C V-PCC content for simplified evaluation of video coding efficiency.

# Introduction

In [1] we proposed to study coding tools targeted at V3C content in JVET. We discussed a simplified evaluation framework, based on provided V3C videos as illustrated in Fig. 1. We stressed the need for simplified evaluation metrics, providing a reliable correlation between distortions on the 2D videos against distortion on the 3D reconstruction. In the contribution at hand, we present our initial findings for such a metric for V3C V-PCC content.

Figure 1. Proposal of V3C video coding evaluation framework [1].

# Occupancy-only PSNR for V3C V-PCC encoding evaluation

In the V-PCC standard, a dynamic 3D point cloud model is segmented into coherent groups of 3D points, so called “patches” based on point surface normals in relation to a minimum of six projection planes. These patches are then packed into 2D video frames. Typically, there are three types of videos associated with a single point cloud sequence: the geometry video, the occupancy video, and at least one attribute video. An example of this process is shown in Fig. 2 below.

Figure 2: Example of the V-PCC patch generation.

Since the patches have irregular shapes, there are empty areas in the packed video frame, not associated with any point cloud information. To improve coding efficiency, these areas are padded in the geometry and attribute videos. The occupancy video carries the information which pixel coordinates carry point cloud information for 3D reconstruction.

The occupancy video is typically compressed using lossless video coding settings; thus, we propose to calculate distortions for geometry and texture videos for occupied pixels only, to determine the reconstruction quality. To determine if such occupancy-only metrics correlate well with the established V-PCC metrics [2], we ran tests for the test content and test points presented in [1]. Tab. 1 below summarises our findings, showing high Pearson correlation coefficients (PCC) and Spearman’s rank correlation coefficients (SPCC) between the proposed metrics and their established V-PCC counterparts.

Table 1: Occupancy-only PSNR correlations to V-PCC metrics (against original point clouds)

|  | D1-PSNR (point-point) |  | D2-PSNR (point-plane) |  | D1 Y-PSNR |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SPCC | PCC | SPCC | PCC | SPCC |
| Geometry Y-PSNR | 0,82 | 0,86 | 0,87 | 0,89 | 0,75 | 0,74 |
| Texture Y-PSNR | 0,84 | 0,88 | 0,86 | 0,87 | 0,96 | 0,96 |

Pearson correlation coefficient (PCC) and Spearman’s rank correlation coefficient (SPCC) are statistical measures of linear correlation between two sets of data. The Pearson correlation coefficient is a normalised measure of the covariance. The Spearman’s rank correlation coefficient is equivalent to the Pearson correlation but calculated over ranked data. Pearson's correlation assesses linear relationships, Spearman's correlation assesses monotonic relationships (whether linear or not) [3].

Correlation is a statistical measure that tells us about the association between the two variables. It describes how one variable behaves if there is some change in the other variable. Thus, the high correlation coefficients shown in Tab. 1 point at strong predictability of established V-PCC quality metrics from the proposed occupancy-only PSNR calculations.

## Details on occupancy-only PSNR calculations

The results of Tab. 1 were obtained as follows.

Geometry Y-PSNR: MSE calculated for all occupied pixels (occupancy map value at pixel position x, y = 1), between uncompressed geometry video luma channel and compressed geometry video luma channel. PSNR calculated with a maximum value of 255 (8 Bit video).

Texture Y-PSNR: MSE calculated for all occupied pixels (occupancy map value at pixel position x, y = 1), between uncompressed texture video luma channel and compressed texture video luma channel. PSNR calculated with a maximum value of 255 (8 Bit video).

D1-, D2-, and D1 Y-PSNR: Distortion between original and reconstructed point clouds following [2].

All the above metrics were derived for the test content and test points presented in [1].

Pearson and Spearman’s rank correlation coefficients were calculated per-frame, over a total of 2570 correlation points (1 sequence with 64 frames, 3 sequences with 150 frames, 5 test points each).

In a second study, D1-, and D1 Y-PSNR distortion were calculated between point clouds reconstructed from uncompressed V3C video and reconstructed point clouds from compressed V3C video. The result for this correlation is presented in Tab. 2.

Table 2: Occupancy-only PSNR correlations to V-PCC metrics (against uncompressed reconstructed point clouds)

|  | D1-PSNR (point-point) |  | D1 Y-PSNR |  |
| --- | --- | --- | --- | --- |
|  | PCC | SPCC | PCC | SPCC |
| Geometry Y-PSNR | 0,85 | 0,89 | 0,81 | 0,80 |
| Texture Y-PSNR | 0,85 | 0,87 | 0,99 | 0,96 |

# Conclusion

In this contribution, we introduced occupancy-only PSNR for V3C V-PCC encoding evaluation. Our initial results show a strong correlation to the established V-PCC metrics.

We propose to introduce occupancy-only PSNR calculations as an initial metric for further studies on common testing conditions for V3C V-PCC content in JVET.

# References

[1] JVET-AC0041, On visual volumetric video-based coding (V3C) testing conditions,
https://jvet-experts.org/doc_end_user/current_document.php?id=12226

[2] Common testing conditions for V-PCC, https://dms.mpeg.expert/doc_end_user/current_document.php?id=76918&id_meeting=184

[3] Comparison of Pearson and Spearman correlation coefficients [Link]

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).