| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0045-v1 |
| --- | --- |

| Title: | EE2-1.7: Adaptive Reference Region DIMD |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zheming Fan Yukinobu Yasugi Tomohiro Ikai | Tel: Email: | fan.zheming@sharp.co.jp yasugi.yukinobu@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution reports simulation EE2-Test-1.7 results of Adaptive Reference Region DIMD for intra prediction. In Test-1.7, three DIMD modes are designed, i.e.DIMD_TL (top and left), DIMD_L (left) and DIMD_T (top). where a new syntax element is signaled to select neighboring reconstructed pixels (reference region) used to derive a DIMD intra prediction mode. The experimental results reportedly show that the average Luma BDrate gain of the EE2-Test-1.7 is XX % and XX %, under AI and RA respectively.

Introduction

This contribution reports the simulation results of EE2-Test-1.7 [1]. In EE2-Test-1.7, a new intra prediction method named Adaptive Reference Region DIMD [2] is studied. In this method, three DIMD modes (DIMD_T, DIMD_L, and DIMD_TL) are made to select different neighborhoods (reference region). In the DIMD_T, top-left, top, and top-right neighborhoods are used as reference region. In DIMD_L, top-left, left, and left-bottom neighborhoods are used as reference region. Moreover, the number of lines in the reference area in DIMD_T and DIMD_L modes are set to 4. The DIMD_TL is just the original DIMD.

Experiment Result

The results of Test-1.7 in AI and RA experiment are show in table1 and table 2, respectively.

Table.1 The result of Test-1.7 in AI experiment

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #NAME? | #NAME? | #NAME? | % | % |
| Class A2 | #NAME? | #NAME? | #NAME? | % | % |
| Class B | #NAME? | #NAME? | #NAME? | % | % |
| Class C | -0.10% | 0.09% | 0.07% | 116% | 109% |
| Class E | -0.11% | 0.04% | 0.02% | 115% | 103% |
| Overall | #NAME? | #NAME? | #NAME? | % | % |
| Class D | -0.06% | -0.07% | 0.13% | 113% | 103% |
| Class F | #NAME? | #NAME? | #NAME? | % | % |
| Class TGM | #NAME? | #NAME? | #NAME? | #DIV/0! | #DIV/0! |

Table.2 The result of Test-1.7 in RA experiment

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #NAME? | #NAME? | #NAME? | % | % |
| Class A2 | #NAME? | #NAME? | #NAME? | % | % |
| Class B | #NAME? | #NAME? | #NAME? | % | % |
| Class C | -0.01% | -0.07% | -0.02% | 108% | 104% |
| Class E |  |  |  |  |  |
| Overall | #NAME? | #NAME? | #NAME? | % | % |
| Class D | -0.08% | -0.33% | -0.21% | 107% | 107% |
| Class F | #NAME? | #NAME? | #NAME? | % | % |
| Class TGM | #NAME? | #NAME? | #NAME? | #DIV/0! | #DIV/0! |

Conclusion

The EE2-Test-1.7 proves that the Adaptive Reference Region DIMD shows XX% XX% XX% Bd-rate gain on Y Cb Cr in AI experiment, XX% XX% XX% Bd-rate gain on Y Cb Cr in RA experiment, respectively.

Acknowledgements

These research results were obtained from the commissioned research (No.05101) by National Institute of Information and Communications Technology (NICT), Japan.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

- Reference

[1] V. Seregin, J. Chen etc, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2), ” JVET-AB2024, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 28th Meeting: Mainz, DE, 20–28 October 2022.

[2] Z. Fan, Y. Yasugi, T. Ikai, “Non-EE2: Adaptive reference region DIMD,” JVET-AB0065, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 28th Meeting: Mainz, DE, 20–28 October 2022.