| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0046-v1 |
| --- | --- |

| Title: | EE2-1.11c: Combination of EE2-1.7, EE2-1.8, EE2-1.9, and EE2-1.10 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zheming Fan Yukinobu Yasugi Tomohiro Ikai | Tel: Email: | fan.zheming@sharp.co.jp yasugi.yukinobu@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution reports simulation results of EE2-Test-1.11c. In Test-1.11c, a combination experiment is studied, in which EE2-TEST-1.7, EE2-TEST-1.8, EE2-TEST-1.9, and EE2-TEST-1.0 are applied at the same time. The experimental results reportedly show that the average Luma BDrate gain of the EE2-Test-1.11c is XX % and XX %, under AI and RA respectively.

Introduction

This contribution reports simulation results of EE2-Test-1.11c [1]. In EE2-Test-1.11c, four EE2 tests are performed together including EE2-TEST-1.7, EE2-TEST-1.8, EE2-TEST-1.9, and EE2-TEST-1.0.

In EE2-TEST-1.7, a new intra prediction method named Adaptive Reference Region DIMD [2] is studied. In this method, three DIMD modes (DIMD_T, DIMD_L, and DIMD_TL) are made to select different neighborhoods (reference region). In DIMD_T, top-left, top, and top-right neighborhoods are used as reference region. In DIMD_L, top-left, left, and left-bottom neighborhoods are used as reference region. Moreover, the number of lines in the reference area in DIMD_T and DIMD_L modes are set to 4. The DIMD_TL is just the original DIMD.

In EE2-TEST-1.8, Location-dependent Decoder-side Intra Mode Derivation method [3] is studied. In this method, sample-based weights are used to blend the DIMD predictors. The usage of sample-based blending, and the specific weights to use for a given predictor, are inferred during the DIMD derivation process. In particular, when deriving a DIMD mode, it is proposed to also determine whether the derivation of such mode was mostly influenced by the template region above or on the left of the current block, and higher weights should be used for samples of above or left template region which is closer to the derived DIMD mode.

In EE2-TEST-1.9, Template-based Intra Mode Derivation with directional blending method [4] is studied. In this method, directional blending is used to fuse predictors determined using Template-based Intra Mode Derivation (TIMD). TIMD intra mode searches are separately performed on template of above reference region and left reference region. And the final prediction mode is obtained by fusing these two intra modes.

In EE2-TEST-1.10, method of optimizing the use of available decoded reference samples [5] is studied. In this method, the use of the available decoded reference samples around the current CU are optimized during the intra prediction step. This method includes three parts that are: 1. Partial MPM reordering depending on the available decoded reference samples. 2. Extension of the DIMD region depending on the available decoded reference samples. 3. A few additional candidate wide-angle modes tested during the TIMD derivation step depending on the available decoded reference samples.

Experiment Result

The results of Test-1.11c in AI and RA experiment are show in table1 and table 2, respectively.

Table.1 The result of Test-1.7 in AI experiment

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #NAME? | #NAME? | #NAME? | % | % |
| Class A2 | #NAME? | #NAME? | #NAME? | % | % |
| Class B | #NAME? | #NAME? | #NAME? | % | % |
| Class C | -0.31% | -0.14% | -0.13% | 127% | 112% |
| Class E | #NAME? | #NAME? | #NAME? | % | % |
| Overall | #NAME? | #NAME? | #NAME? | % | % |
| Class D | -0.27% | -0.08% | -0.20% | 126% | 117% |
| Class F | #NAME? | #NAME? | #NAME? | % | % |
| Class TGM | #NAME? | #NAME? | #NAME? | #DIV/0! | #DIV/0! |

Table.2 The result of Test-1.7 in RA experiment

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #NAME? | #NAME? | #NAME? | % | % |
| Class A2 | #NAME? | #NAME? | #NAME? | % | % |
| Class B | #NAME? | #NAME? | #NAME? | % | % |
| Class C | #NAME? | #NAME? | #NAME? | % | % |
| Class E |  |  |  |  |  |
| Overall | #NAME? | #NAME? | #NAME? | % | % |
| Class D | #NAME? | #NAME? | #NAME? | % | % |
| Class F | #NAME? | #NAME? | #NAME? | % | % |
| Class TGM | #NAME? | #NAME? | #NAME? | #DIV/0! | #DIV/0! |

Conclusion

The EE2-Test-1.11c proves that the combination of above four methods can obtain XX% XX% XX% Bd-rate gain on Y Cb Cr in AI condition, XX% XX% XX% Bd-rate gain on Y Cb Cr in RA condition, respectively.

Acknowledgements

These research results were obtained from the commissioned research (No.05101) by National Institute of Information and Communications Technology (NICT), Japan.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

- Reference

[1] V. Seregin, J. Chen, etc, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2)”, JVET-AB2024, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 28th Meeting: Mainz, DE, 20–28 October 2022.

[2] Z. Fan, Y. Yasugi, and T. Ikai, “Non-EE2: Adaptive reference region DIMD”, JVET-AB0065, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 28th Meeting: Mainz, DE, 20–28 October 2022.

[3] S. Blasi and J. Lainema, “AHG12 - Location-dependent Decoder-side Intra Mode Derivation”, JVET-AB0116, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 28th Meeting: Mainz, DE, 20–28 October 2022.

[4] S. Blasi, J. Lainema, and etc, “AHG12 - Template-based Intra Mode Derivation with directional blending”, JVET-AB0117, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 28th Meeting: Mainz, DE, 20–28 October 2022.

[5] T. Dumas, K. Reuzé, and Y. Chen, “Non-EE2: optimizing the use of available decoded reference samples”, JVET-AB0142, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 28th Meeting: Mainz, DE, 20–28 October 2022.