| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0046-v3 |
| --- | --- |

| Title: | EE2-1.11c: Combination of EE2-1.7, EE2-1.8, EE2-1.9, and EE2-1.10 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zheming Fan Yukinobu Yasugi Tomohiro Ikai | Tel: Email: | fan.zheming@sharp.co.jp yasugi.yukinobu@sharp.co.jp ikai.tomohiro@sharp.co.jp |
|  | Saverio Blasi Jani Lainema |  | saverio.blasi@nokia.com jani.lainema@nokia.com |
|  | Thierry Dumas Kevin Reuzé |  | thierry.dumas@interdigital.com kevin.reuze@interdigital.com |
| Source: | Sharp, Nokia, and InterDigital |  |  |

_____________________________

- Abstract

This contribution reports simulation results of EE2-Test-1.11c. The experimental results reportedly show that the average Luma Bd-rate gain of the EE2-Test-1.11c is 0.31% and 0.16%, under AI and RA respectively.

Introduction

In test EE2-1.11c [1], combination of tests Test 1.7 [2], Test 1.8 [3], Test 1.9 [4] and Test 1.10 [5] is tested. These tests propose modifications to Decoder Side Intra mode Derivation (DIMD) and Template-based Intra Mode Derivation (TIMD).

Experiment Result

The results of Test-1.11c in AI and RA experiment are show in table1 and table 2, respectively.

Table.1 The result of Test-1.7 in AI experiment

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.36% | -0.38% | -0.31% | 128% | 107% |
| Class A2 | -0.28% | -0.15% | -0.14% | 125% | 105% |
| Class B | -0.28% | -0.10% | -0.19% | 131% | 106% |
| Class C | -0.31% | -0.14% | -0.13% | 127% | 112% |
| Class E | -0.35% | -0.29% | -0.33% | 131% | 109% |
| Overall | -0.31% | -0.20% | -0.21% | 129% | 108% |
| Class D | -0.27% | -0.08% | -0.20% | 126% | 117% |
| Class F | -0.22% | -0.01% | -0.09% | 117% | 113% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table.2 The result of Test-1.7 in RA experiment

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.22% | -0.21% | -0.25% | 106% | 100% |
| Class A2 | -0.09% | -0.08% | -0.12% | 104% | 99% |
| Class B | -0.15% | -0.27% | -0.36% | 107% | 100% |
| Class C | -0.16% | -0.19% | -0.22% | 115% | 113% |
| Class E |  |  |  |  |  |
| Overall | -0.16% | -0.20% | -0.25% | 108% | 103% |
| Class D | -0.16% | 0.02% | 0.07% | 113% | 115% |
| Class F | -0.13% | -0.12% | 0.00% | 106% | 100% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Conclusion

The EE2-Test-1.11c proves that the combination of above four methods can obtain 0.31% 0.20% 0.21% Bd-rate gain on Y Cb Cr in AI condition, 0.16% 0.20% 0.25% Bd-rate gain on Y Cb Cr in RA condition, respectively.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Interdigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

- Reference

[1] V. Seregin, J. Chen, etc, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” JVET-AB2024, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 28th Meeting: Mainz, DE, 20–28 October 2022.

[2] Z. Fan, Y. Yasugi, and T. Ikai, “Non-EE2: Adaptive reference region DIMD,” JVET-AB0065, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 28th Meeting: Mainz, DE, 20–28 October 2022.

[3] S. Blasi and J. Lainema, “AHG12 - Location-dependent Decoder-side Intra Mode Derivation,” JVET-AB0116, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 28th Meeting: Mainz, DE, 20–28 October 2022.

[4] S. Blasi, J. Lainema, and etc, “AHG12 - Template-based Intra Mode Derivation with directional blending,” JVET-AB0117, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 28th Meeting: Mainz, DE, 20–28 October 2022.

[5] T. Dumas, K. Reuzé, and Y. Chen, “Non-EE2: optimizing the use of available decoded reference samples,” JVET-AB0142, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 28th Meeting: Mainz, DE, 20–28 October 2022.