| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0048-v4 |
| --- | --- |

| Title: | Non-EE: Modification of TIMD |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yukinobu Yasugi Tomohiro Ikai | Tel: Email: | yasugi.yukinobu@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution proposes a modification of template-based intra derivation (TIMD) mode on top of ECM-7.0 software. This modification allows explicit selection of template from three regions; TL, T and L. Experimental results reportedly show BD-rate -0.09% and XX% for AI and RA conditions for test a (no block restrictions), respectively.

Introduction

Template-based intra mode derivation (TIMD) on ECM-7.0 implicitly selects a template region to be used from three regions, T(Top)+L(Left), T, and L, depending on the coordinate of the current block. For example, if the block is at the left edge of the frame (x==0), only the template region (T) adjacent to the upper side is used. If the block is at the top of the frame (y==0), the template region (L) adjacent to the left side is used. Otherwise, both the top and left template regions (TL) are used.

Proposed Method

This contribution proposes to introduce a mechanism to explicitly select one of the three types of template regions, even for CUs that are not located at the top or left boundary of the frame.

Changes

- Introduce the following syntax element timd_ref_mode to select a template region.

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| … |  |
| if( CuPredMode[ x0 ][ y0 ] == MODE_INTRA ) { |  |
| … |  |
| timd_flag[ x0 ][ y0 ] | ae(v) |
| if( timd_flag[ x0 ][ y0 ] != 0 ) |  |
| timd_ref_mode[ x0 ][ y0 ] | ae(v) |
| … |  |
| } |  |
| } |  |

timd_ref_mode[ x0 ][ y0 ] specifies the mode of template region. The array indices x0, y0 specify the location (x0, y0) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture. The value of timd_ref_mode shall be in the range of 0 to 2, inclusive. When the value of timd_ref_mode[ x0 ][ y0 ] is equal to 0, the upper and left template regions are used if available. When the value of timd_ref_mode[ x0 ][ y0 ] is equal to 1 or 2, the upper or left template region is used, respectively. When timd_ref_mode[ x0 ][ y0 ] is not present, it is inferred to be equal 0.

Note:

- When the timd_ref_mode is equal to 0, it is the same as the original TIMD mode in ECM-7.0.

Experimental Results

We implemented the proposed method on top of ECM-7.0 and conducted experiments in the following three settings.

- No block size limitation for using the proposed method

- Using the proposed method only for blocks with an area of 32 luma samples or more

- Using the proposed method only for with the size of 8x8  or more

Experimental results are as follows.

Test a) No block size limitations

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.01% | -0.04% | 117% | 100% |
| Class A2 | -0.05% | 0.03% | 0.07% | 117% | 101% |
| Class B | -0.10% | 0.05% | -0.01% | 120% | 100% |
| Class C | -0.12% | 0.02% | 0.05% | 120% | 101% |
| Class E | -0.12% | 0.13% | -0.03% | 120% | 100% |
| Overall | -0.09% | 0.04% | 0.01% | 119% | 100% |
| Class D | -0.12% | -0.16% | 0.23% | 120% | 103% |
| Class F | -0.07% | 0.13% | 0.04% | 111% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.21% | -0.07% | 104% | 98% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.03% | -0.06% | -0.31% | 104% | 100% |
| Class C | -0.02% | -0.02% | 0.04% | 104% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.05% | -0.06% | -0.19% | 104% | 100% |
| Class F | -0.05% | 0.01% | -0.06% | 103% | 99% |

Test b) Blocks with an area of 32 luma samples or more

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | 0.02% | 0.04% | 116% | 99% |
| Class A2 | -0.06% | 0.00% | 0.10% | 117% | 100% |
| Class B | -0.09% | -0.05% | -0.05% | 120% | 100% |
| Class C | -0.12% | 0.13% | 0.05% | 119% | 100% |
| Class E | -0.10% | 0.08% | 0.06% | 120% | 100% |
| Overall | -0.09% | 0.03% | 0.03% | 118% | 100% |
| Class D | -0.10% | -0.29% | 0.10% | 119% | 102% |
| Class F | -0.09% | 0.11% | 0.19% | 111% | 101% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.21% | 0.04% | 104% | 99% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.04% | -0.13% | -0.33% | 105% | 100% |
| Class C | -0.02% | -0.17% | 0.00% | 105% | 101% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.04% | -0.33% | 0.04% | 105% | 101% |
| Class F | -0.09% | 0.05% | -0.13% | 104% | 100% |

Test c) Blocks with the size of 8x8 or more

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.04% | -0.02% | 111% | 99% |
| Class A2 | -0.04% | 0.03% | 0.03% | 111% | 101% |
| Class B | -0.08% | 0.03% | -0.11% | 112% | 100% |
| Class C | -0.07% | 0.01% | 0.00% | 110% | 100% |
| Class E | -0.10% | -0.04% | 0.10% | 111% | 99% |
| Overall | -0.07% | 0.00% | -0.01% | 111% | 100% |
| Class D | -0.04% | -0.22% | -0.10% | 109% | 101% |
| Class F | -0.04% | 0.14% | 0.10% | 106% | 101% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.17% | -0.01% | 103% | 99% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.04% | -0.02% | -0.19% | 103% | 100% |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | 0.17% | 0.11% | 103% | 101% |
| Class F | -0.06% | -0.06% | -0.16% | 103% | 100% |

Conclusion

In this contribution, a modification of Template-based Intra Mode Derivation mode is proposed. A coding efficiency more than 0.1% in some classes and 0.09% in average has been achieved with selecting template regions. We suggest investigating the method in EE2.

Acknowledgements

These research results were obtained from the commissioned research (No.05101) by National Institute of Information and Communications Technology (NICT), Japan.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

- References

- Y. Wang, L. Zhang, K. Zhang, Z. Deng, and N. Zhang, “EE-related: Template-based intra mode derivation using MPMs”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC29, JVET-V0098, April 2021.