| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0053 |
| --- | --- |

| Title: | AHG12: Simplified linear model solver |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jani Lainema Pekka Astola Alireza Aminlou Ramin G. Youvalari | Email: | first.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

Some ECM tools, such as CCCM (Convolutional Cross-Component Model) and GLM (Gradient Linear Model), need to solve sets of linear equations when calculating filter coefficients. ECM-7 currently solves those using LDL decomposition. This contribution proposes to replace that with a more straight-forward approach based on Gaussian elimination. It is asserted the proposed approach is beneficial in multiple ways:

- Using row-based matrix operations (enabling SIMD/parallel implementation)

- Smaller amount of arithmetic operations

- Smaller amount of numbers multiplied together (leading to reduced dynamic range)

- Smaller amount of back-substitutions (leading to reduced latency)

In addition to the asserted implementation benefits, the CTC simulations reportedly indicate some tendency towards improved coding efficiency:

AI { 0.00%, -0.15%, 0.03%, 99%, 100%}, RA { -0.01%, -0.10%, -0.02%, 99%, 99%}

LB { 0.00%, -0.38%, -0.42%, 98%, 100%}, LP { 0.00%, -0.07%, -0.34%, 98%, 100%}

# Introduction and analysis

Some ECM tools, such as CCCM (Convolutional Cross-Component Model) and GLM (Gradient Linear Model), are solving sets of linear equations when calculating filter coefficients. ECM-7 finds the solutions using LDL decomposition. However, it is noted that a more straight-forward Gaussian elimination based approach proposed here has several benefits over the ECM-7 implementation.

Firstly, the proposed approach is using only row-based matrix operations making it easy to optimize the implementation using e.g. SIMD operations. The ECM-7 approach is having loops in both row and column direction making such optimizations more difficult.

Secondly, the number of arithmetic operations is reduced compared to the ECM-7 approach. The combined number of operations to solve both Cb and Cr 7-tap filters in one TU is asserted as follows:

- ECM-7:     231 mults, 7 division parameter estimates

- Proposed:  210 mults, 7 division parameter estimates

Thirdly, the amount of numbers multiplied together is reduced from three to two when switching from ECM-7 approach to the proposed one. This is asserted beneficial for keeping the dynamic range of the intermediate values low.

Fourthly, the LDL approach of ECM-7 requires two consecutive back-substitution operations as the intermediate filter coefficients need to be first solved with respect to the first triangular matrix L followed by solving the final filter coefficient with respect to the transpose of L. In the proposed approach only a single back-substitution process is needed reducing the total latency of the solver.

# Implementation

Elimination and back-substitution can be implemented for example as given below. The same function can be used to solve coefficients for one or both chrominance channels by setting numFilters parameter to 1 or 2, respectively. The xGetDivScaleRoundShift function returns scale, round and shift parameters approximating a division operation with the input diag parameter. Calculation of those parameters is already implemented in ECM-7 in its “xDivide” function.

for( int i = 0; i < numEq; i++ )

{

TCccmCoeff *src = C[i];

TCccmCoeff diag = src[i] < 1 ? 1 : src[i];

xGetDivScaleRoundShift(diag, dscale, dround, dshift);

for( int j = i+1; j < numEq+numFilters; j++ )

{

src[j] = (src[j] * dscale + dround) >> dshift;

}

for( int j = i + 1; j < numEq; j++ )

{

TCccmCoeff *dst  = C[j];

TCccmCoeff scale = dst[i];

// On row j all elements with k < i+1 are now zero

for( int k = i + 1; k < numEq+numFilters; k++ )

{

dst[k] -= FIXED_MULT(scale, src[k]);

}

}

}

// Solve with backsubstitution

if ( numFilters == 2 )

{

backsubstitution(C, x0, numEq, colChr0);

backsubstitution(C, x1, numEq, colChr1);

}

else

{

backsubstitution(C, x0, numEq, colChr0);

}

Where:

void backsubstitution(TE2 C, Ty x, int numEq, int col)

{

x[numEq-1] = C[numEq-1][col];

for( int i = numEq-2; i >= 0; i-- )

{

x[i] = C[i][col];

for( int j = i+1; j < numEq; j++ )

{

x[i] -= FIXED_MULT(C[i][j], x[j]);

}

}

}

# Experimental results

Tables below summarize the coding efficiency and runtime impact of the proposed method in ECM 7.0 using the JVET common test conditions. Full results are provided in the attached spreadsheet.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02 % | -0.36 % | 0.50 % | 99 % | 98 % |
| Class A2 | 0.01 % | -0.16 % | -0.11 % | 99 % | 99 % |
| Class B | -0.01 % | -0.06 % | 0.06 % | 99 % | 100 % |
| Class C | 0.00 % | 0.01 % | -0.08 % | 99 % | 101 % |
| Class E | -0.02 % | -0.30 % | -0.20 % | 98 % | 100 % |
| Overall | 0.00 % | -0.15 % | 0.03 % | 99 % | 100 % |
| Class D | 0.02 % | -0.18 % | 0.14 % | 99 % | 100 % |
| Class F | -0.04 % | 0.03 % | -0.09 % | 99 % | 102 % |
| ClassTGM | -0.01 % | -0.02 % | 0.00 % | 99 % | 101 % |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04 % | -0.56 % | 0.27 % | 99 % | 98 % |
| Class A2 | -0.01 % | 0.06 % | -0.17 % | 99 % | 98 % |
| Class B | 0.00 % | -0.11 % | -0.13 % | 99 % | 98 % |
| Class C | 0.02 % | 0.14 % | 0.02 % | 99 % | 97 % |
| Class E |  |  |  |  |  |
| Overall | -0.01 % | -0.10 % | -0.02 % | 99 % | 98 % |
| Class D | 0.02 % | -0.19 % | 0.16 % | 100 % | 98 % |
| Class F | -0.02 % | -0.05 % | -0.05 % | 98 % | 97 % |
| ClassTGM | 0.07 % | -0.07 % | 0.11 % | 98 % | 102 % |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | 0.04 % | -0.62 % | -0.68 % | 98 % | 99 % |
| Class C | -0.01 % | 0.19 % | 0.07 % | 99 % | 100 % |
| Class E | -0.04 % | -0.73 % | -0.66 % | 97 % | 100 % |
| Overall | 0.00 % | -0.38 % | -0.42 % | 98 % | 100 % |
| Class D | -0.01 % | -0.28 % | 0.66 % | 100 % | 100 % |
| Class F | 0.05 % | -0.16 % | -0.55 % | 98 % | 99 % |
| ClassTGM |  |  |  |  |  |
|  | Low delay P Main10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.04 % | -0.10 % | -0.53 % | 98 % | 100 % |
| Class C | 0.00 % | -0.32 % | -0.29 % | 99 % | 100 % |
| Class E | 0.08 % | 0.29 % | -0.10 % | 96 % | 100 % |
| Overall | 0.00 % | -0.07 % | -0.34 % | 98 % | 100 % |
| Class D | 0.01 % | -0.69 % | 0.96 % | 100 % | 100 % |
| Class F | -0.11 % | 0.33 % | -0.52 % | 97 % | 101 % |
| ClassTGM |  |  |  |  |  |

# Conclusion

The proposed approach is asserted to have several benefits and no disadvantages over the LDL based ECM-7 linear model solver. It is proposed to substitute the ECM-7 solver with the proposed solution.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).