| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0055-v2 |
| --- | --- |

| Title: | EE1-1.11: Content-adaptive post-filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Maria Santamaria Ruiying Yang Francesco Cricri Jani Lainema Honglei Zhang Ramin G. Youvalari Miska M. Hannuksela Nokia Technologies Tampere, Finland | Email: | maria.santamaria_gomez@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution reports the results of tests EE1-1.11, related to the content-adaptive CNN post-filter presented in JVET-AB0048. The signalling involves the NN post-filter characteristics and activation SEI messages in JVET-Z0244. The inference was done using float and int16 precision and SADL library. EE1-1.11 is evaluated against VTM 11.0 NNVC 3.0 in the RA configuration. It is reported that the overall coding gains (for int16 inference) are -4.78% (Y), -18.31%  (Cb), -16.67% (Cr).

# Introduction

JVET-AB0048 [1] studies a post-filter and the NN post-filter characteristics and activation (per picture) SEI messages, described in JVET-Z0244 [2] . In this EE, the same post-filter is evaluated, but on top of VTM 11.0 NNVC 3.0.

# Content-adaptive post-filter

The content-adaptive post-filter in JVET-AB0048 is obtained by overfitting multiplier parameters that are applied after the bias is added to the convolution result:

σW*x+b∙m,
where W is the kernel, * is the convolution operator, x is the input, b is the bias, m is the multiplier and σ is the activation function.

The NN architecture is based on JVET-W0131 [3], illustrated in Figure 1, and it includes the multiplier layers. Additionally, the boundary strength filtering data is no longer used as part of the input, instead zeros are provided.

Figure 1. Network architecture [3].

The input to the NN is a CTU, 8 neighbouring samples at each of its sides and the slice QP step. The luma samples are interleaved into four blocks of size 72x72. The output is the filtered CTU with size 64x64x6, where the luma samples are interleaved as the input luma samples were.

For each frame, a single model is chosen out of four candidate filters, based on the SSD. The filter that minimises the SSD (also including the SSD of the picture without post-filter) is selected and signalled via an NN post-filter activation SEI message.

The source code for training and inference is available at [4].

## Offline finetuning

Since the NN in JVET-W0131 was trained as an in-loop filter, here it is adapted to serve as post-filter. This process is done offline on BVI-DVC [5] and DIV2K [6] datasets and with no boundary strength filtering data (zeros are fed instead). The resulting models are referred to as base models. The validation dataset consisted of a subset of the mandatory JVET CTC sequences for RA.

Table I shows the network information during the finetuning stage.

Table I. Network information for NN-based video coding tool testing in finetuning stage

| Mandatory | GPU Type | GPU NVIDIA A100-SXM-80GB |
| --- | --- | --- |
|  | Framework: | TensorFlow 2.4.1 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | See training time |
|  | Batch size: | 64 |
|  | Loss function: | Weighted-MSE |
|  | Training time: | model0 & model2-> 70 hours model1 -> 11 hours model3 -> 109 hours |
|  | Training data information: | BVI-DVC & DIV2K |
|  | Training configurations for generating compressed training data (if different to VTM CTC): |  |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size |  |
|  | Learning rate: | 1,00E-04 |
|  | Optimizer: | ADAM |
|  | Preprocessing: | Convert 144x144 YUV420 signal to 6 72x72 blocks. Normalize to 0~1 |
|  | Mini-batch selection process: |  |
|  | Other information: |  |

## Overfitting

One of the base models is overfitted per {sequence, QP} pair. The base model to overfit is the one that provides the highest PSNR gain on the first RA segment. Only the multipliers are overfitted and the full video sequence is used as overfitting data. As a result of this process, a weight update is computed and coded using NNC v2 [7, 8] with uniform quantisation, iterative QP assignment, QP density 2, CABAC unary length 10 and CABAC optimisation.

In each simulation, five post-filter characteristics SEI messages are signalled: four of them contain the information about the base filters and the fifth message consists of the overfitted model including the NNC bitstream.

Table I shows the NN information during the overfitting stage.

Table II. Network information for NN-based video coding tool testing in overfitting stage

| Mandatory | GPU Type | GPU NVIDIA A100-SXM-80GB |
| --- | --- | --- |
|  | Framework: | TensorFlow 2.4.1 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 200 |
|  | Batch size: | 64 |
|  | Loss function: | Weighted-MSE |
|  | Training time: | A1 & A2: 13 hours B: 6 hours C: 2 hours D: 45 mins F: 4.5 hours |
|  | Training data information: | JVET CTC RA mandatory sequences |
|  | Training configurations for generating compressed training data (if different to VTM CTC): |  |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size |  |
|  | Learning rate: | 1,00E-03 |
|  | Optimizer: | ADAM |
|  | Preprocessing: | Convert 144x144 YUV420 signal to 6 72x72 blocks. Normalize to 0~1 |
|  | Mini-batch selection process: |  |
|  | Other information: |  |

## NN Quantisation

The quantisation used is post-training quantisation and it is guided by the training data. The new NN quantisation includes some bug fixes, e.g. make sure the internal quantisers are not negative, and calibration of the base models quantisation using BVI-DVC dataset.

## Inference

Table III. Network information for NN-based video coding tool testing in inference stage

| Mandatory | HW environment: |  |
| --- | --- | --- |
|  | GPU Type | CPU only |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 109068 |
|  | Total Number of Parameters (All Models) | 436272 |
|  | Parameter Precision (Bits) | 16 |
|  | Memory Parameter (MB) | 0,832122803 |
|  | Multiply Accumulate (kMAC/pixel) | 34 (block) |
| Optional |  |  |
|  | Total Conv. Layers | 35 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size |  |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling | Use padding samples if necessary |
|  | Other information: |  |

# Simulation results

The proposed approach was implemented on top of VTM 11.0 NNVC 3.0 and evaluated on the JVET CTC for NNVC [9] on the RA configuration.

The computing environment is a cluster with CentOS Linux (Core), Intel Xeon Gold 6154 CPUs and GCC 9.3.1.

The bitrate of the test includes the overhead of the NNPFC and NNPFA messages.

Table IV. Random access main 10 - float inference

|  | BD-rate Over NNVC3.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | bit DIFF |
| Class A1 | -6,43 % | -13,64 % | -20,44 % | -8,41 % | -16,54 % | -21,29 % | 130 % | 8846 % | 0 % |
| Class A2 | -4,22 % | -18,33 % | -13,61 % | -3,45 % | -18,09 % | -11,97 % | 127 % | 8066 % | 0 % |
| Class B | -5,15 % | -20,55 % | -17,01 % | -3,27 % | -20,83 % | -16,80 % | 132 % | 10405 % | 0 % |
| Class C | -3,60 % | -18,93 % | -15,99 % | -2,42 % | -17,65 % | -15,92 % | 123 % | 10929 % | 1 % |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -4,80 % | -18,29 % | -16,74 % | -4,11 % | -18,58 % | -16,50 % | 128 % | 9699 % | 1 % |
| Class D | -3,61 % | -18,29 % | -17,36 % | 0,24 % | -16,61 % | -15,69 % | 125 % | 11245 % | 3 % |
| Class F | -1,56 % | -15,52 % | -12,05 % | -1,35 % | -15,69 % | -13,06 % | 158 % | 14724 % | 1 % |
| Class H |  |  |  |  |  |  |  |  |  |

Table V. Random access main 10 - int16 inference

|  | BD-rate Over NNVC3.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | bit DIFF |
| Class A1 | -6,37 % | -13,77 % | -20,35 % | -8,39 % | -16,56 % | -21,25 % | 133 % | 10501 % | 0 % |
| Class A2 | -4,21 % | -18,37 % | -13,57 % | -3,46 % | -18,08 % | -11,95 % | 129 % | 9598 % | 0 % |
| Class B | -5,12 % | -20,51 % | -16,87 % | -3,26 % | -20,80 % | -16,75 % | 131 % | 10861 % | 0 % |
| Class C | -3,59 % | -18,90 % | -15,97 % | -2,41 % | -17,63 % | -15,90 % | 120 % | 10240 % | 1 % |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -4,78 % | -18,31 % | -16,67 % | -4,10 % | -18,56 % | -16,46 % | 128 % | 10361 % | 1 % |
| Class D | -3,59 % | -18,29 % | -17,34 % | 0,27 % | -16,57 % | -15,65 % | 122 % | 10565 % | 3 % |
| Class F | -1,42 % | -13,10 % | -10,60 % | -1,24 % | -14,55 % | -12,41 % | 152 % | 7161 % | 1 % |
| Class H |  |  |  |  |  |  |  |  |  |

# Recommendation

It is recommended to adopt this content-adaptive post-filter into the common software base for NNVC (NCS) provided the cross-check is successful.

# References

- M. Santamaria, R. Yang, F. Cricri, J. Lainema, H. Zhang, R. G. Youvalari, M. M. Hannuksela. EE1-1.1: Content-adaptive post-filter with SADL inference and signalling of NN post-filter characteristics and activation SEI messages. JVET-AB0048. October 2022.

- M. M. Hannuksela, M. Santamaria, F. Cricri, E. B. Aksu, H. R. Tavakoli, T. Chujoh, Y. Yasugi, T. Ikai, S. McCarthy, A. Arora, T. Shao, P. Yin, T. Lu, F. Pu, W. Husak. AHG9: NN post-filter SEI. JVET-Z0244. April 2022.

- H. Wang, J. Chen, K. Reuze, A. M. Kotra and M. Karczewicz. EE1-related: Neural-Network-based in-loop filter with constrained computational complexity. JVET-W0131. July 2021.

- https://vcgit.hhi.fraunhofer.de/jvet-ab-ee1/VVCSoftware_VTM/-/tree/EE1-1.11/

- D. Ma, F. Zhang and D. Bull, "BVI-DVC: A Training Database for Deep Video Compression," in IEEE Transactions on Multimedia, doi: 10.1109/TMM.2021.3108943.

- E. Agustsson and R. Timofte, "NTIRE 2017 Challenge on Single Image Super-Resolution: Dataset and Study," 2017 IEEE Conference on Computer Vision and Pattern Recognition Workshops (CVPRW), 2017, pp. 1122-1131, doi: 10.1109/CVPRW.2017.150.

- ISO/IEC FDIS 15938-17. Information technology – Multimedia content description interface – Part 17: Compression of neural networks for multimedia content description and analysis.

- Test Model of Incremental Compression of Neural Networks for Multimedia Content Description and Analysis (INCTM), N0179. February 2022.

- E. Alshina, R.-L. Liao, S. Liu, A. Segall. Common Test Conditions and evaluation procedures for neural network-based video coding technology. JVET-AA2016. July 2022.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).