| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0056-v2 |
| --- | --- |

| Title: | EE1-3.1 CompressAI models integration using SADL |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Franck Galpin Frédéric Lefebvre Fabien Racapé | Tel: Email: | firstname.lastname@interdigital.com |
| Source: | InterDigital Inc. |  |  |

_____________________________

# Abstract

JVET-AB0125 presented a very low-complexity, full integer operations, auto-encoder, suitable for hardware implementation. The proposed changes of the entropy coder and the implementation of an RDOQ method, on top of a basic fully-factorized prior-based approach, shows a bdrate change of -19% on the Kodak dataset, compared to the original. Further tests are conducted to test the impact of the model integerization. Reported results show comparable gains with an average bdrate of -18.6% compared to the anchor for int16 models.

# Introduction

In the AhG 11, a reference software has been introduced to test neural network-based technologies on top of traditional video codec. The NN-based technologies explored in this group and in Exploration Experiments currently include in-loop filters, intra prediction and inter prediction. JVET-AA00063 shows results when the whole intra frame coding process has been replaced by an end-to-end trained auto-encoder.

To continue evaluating such approaches, additional layers were implemented in SADL to handle the basic fully factorized prior auto-encoder. Further study on the impact of the integerization of the models were conducted.

Figure 1 Overview of the model architecture. N=128 and M=192.

# Static quantization

The original float models were directly quantized using 16-bit integers after training and the models converted to use only integer arithmetic. The generic code of the auto-encoder was slightly modified to support both float and int16 models.

Both simple encoding and RDOQ encoding using the conditional CDFs were used during the tests.

# Quantization aware training

During the static quantization stage, it was noticed that the integerized decoder model for lambda=0.08 (highest rate) performs significantly worse than the float version, unlike for the other lambdas.

We thus retrained a decoder for this specific point only using Quantization Aware Training (QAT):

- All weights were quantized on 15 bits + 1 bit for the sign

- All intermediate variables were quantized on 15 bits + 1 bits (sign) after the first deconvolution layer.

The figure above shows that performing QAT for the highest lambda enables the int16 model to be in par with the float version of the model.

# Results

Note: we report the bdrate of the average performances over the test set, as usually reported in the learned image compression community, please refer to the excel sheets which also contain the average of the bdrates.

## Float models anchors

The tables below report the performances of the SADL models using ReLU/ReLU, which stands for the use of ReLU activation at both encoder/decoder, over the Kodak dataset:

- The SADL model (ReLU/ReLU) and a simple CDF per channel of the latent underperforms the original floating point fully-factorized prior model (with GDN/iGDN, using pytorch, with a loss of about 8.85%.

- Starting from the above SADL model, the bitrate reductions obtained using conditional CDFs lead to a BD-rate gain of -11.45% (i.e. about -3% on top of the original model)

- Adding the RDOQ provides additional 8 to 9% BD-rate gains.

| Comparisons using float models |  |  |
| --- | --- | --- |
| impact of gdn to relu change |  |  |
| anchor | test | bdrate |
| float gdn, pytorch | float Relu, SADL simple CDF | 8.85% |
| Comparisons using float models |  |  |
| impact of conditional CDFs and RDOQ over simple cdf |  |  |
| anchor | test | bdrate |
| float Relu, SADL simple CDF | float Relu, SADL cond. CDF | -11.45% |
|  | float Relu + RDOQ, SADL cond. CDF | -19.01% |

## Comparisons of float models to int16 models

### Direct static quantization

Using a static quantization for all models leads to about 1% loss when using the integerized models, as reported in the table below.

| Comparisons between float and int16 |  |  |
| --- | --- | --- |
| all static quantized |  |  |
| anchor | test | bdrate |
| float Relu, SADL cond. CDF | int Relu, SADL cond. CDF | 1.03% |
| float Relu + RDOQ, SADL cond. CDF | int Relu + RDOQ, SADL | 1.09% |

### Quantization aware training

In order to correct the above model, only the decoder of model for lambda 0.08 has been retrained using QAT (all other models are using static quantization).

The reported loss is reduced to about 0.5%.

| Comparisons between float and int16 |  |  |
| --- | --- | --- |
| note: lambda 0.08 has been quantized using QAT |  |  |
| anchor | test | bdrate |
| float Relu, SADL cond. CDF | int Relu, SADL cond. CDF | 0.44% |
| float Relu + RDOQ, SADL cond. CDF | int Relu + RDOQ, SADL | 0.55% |

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).