| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0060-v2 |
| --- | --- |

| Title: | EE2-3.4: BVP candidates clustering and BVD sign derivation for Reconstruction-Reordered IBC mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | D. Ruiz Coll, Jung-Kyung Lee, V. Warudkar | Tel: Email: | duizcoll@ofinno.com, jlee@ofinno.com, vwarudkar@ofinno.com |
| Source: | Ofinno, LLC |  |  |

_____________________________

# Abstract

This document reports results for EE2-3.4 on the modifications of the IBC AMVP list construction based on clustering the block vector predictors according to the L2 distance between the candidates and selecting one candidate per cluster based on the TM cost. Likewise related to the IBC AMVP mode, if the Reconstruction-Reordered IBC (RRIBC) mode is selected, two new BVP candidates are determined, and the sign of the non-null BVD component is inferred at the decoder side. This method is extended to any BVs having one null component, regardless of whether it uses the RRIBC mode.

On top of ECM-7.0, the test results for the proposed method for class F and class TGM are reported as follows:

- AI: -0.28%, -0.19%, and -0.21%, with 98% EncT, 97% DecT.

- RA: -0.22%, -0.19%, and -0.07%, with 100% EncT, 100% DecT.

- LDB: -0.20%, -0.35%, and -0.27%, with 101% EncT, 99% DecT.

# Introduction

In JVET-AA0070 [1], a Reconstruction-Reordered IBC (RRIBC) tool was proposed for screen content coding, and it was adopted in ECM-7.0. In RRIBC, to exploit the symmetry that frequently occurs in computer-generated content, the original block is flipped in a horizontal or vertical direction before searching the Block Vector (BV) for the current block. The search for the best matching reference block is constrained to the same row or column as the current block, requiring the BV to have one null component.

Furthermore, the IBC-TM merge mode applies a candidate's pruning method, followed by the merge list reordering (ARMC-TM), and the refinement of candidates using the Template Matching (TM) cost. In the IBC-TM AMVP mode, the three merge candidates are TM refined and sorted based on the TM cost, and the first two candidates are selected for the motion estimation process.

In JVET-AB0173 [2], modifications to the IBC AMVP list construction were proposed to improve the IBC coding efficiency for screen content coding.

# Proposal

In this contribution, the IBC AMVP list construction is modified based on the clustering of the block vector predictor (BVP) candidates according to the distance between them, and the sign prediction of the BVD if the BV has one null component.

For the blocks whose BV has both non-null components, a clustering of the BVP candidates before selecting the two AMVP candidates is applied. The clustering is used if the number of valid BVP candidates exceeds two, and up to six BVP candidates are clustered based on the L2 Euclidean distance between them. The radius (R) determines a group of vectors as a logarithmic function of the width (cbWidth) and height (cbHeight) of the current block as follows:

R=log2((cbWidth∙cbHeight)≫MIN_PU_SIZE)  		(1)

The clustering method is applied in the candidate list order, and the candidates assigned to a group are removed from the list for the subsequent clusters. In each group, the BVP with a lowest TM cost is selected as the representative candidate of that group. The representative candidates of the two first groups are chosen for the motion estimation process as in the regular IBC AMVP list.

Figure 1. AMVP IBC candidate clustering based on the L2 distance and the TM cost.

On the contrary, BVs with one null component, including the RRIBC blocks, are signaled to the decoder by a bvOneNullComp flag. Instead of invoking the AMVP IBC list construction, two new BVP candidates are determined, which are adjusted to the boundaries of the valid IBC search region according to the horizontal or vertical direction indicated by a bvNullCompDir flag.

The AMVP BVP0 is set to the nearest valid location to the current block (-cbWidth or -cbHeight), so the BVD, if it is not null, is always negative, pointing to the left for a BV with a null vertical component or to the above for a BV with a null horizontal component. Likewise, the AMVP BVP1 is set to the farthest position from the current block in the valid reference region, that is the left boundary or the top boundary of the IBC search region. Consequently, if the BVP1 is selected, the BVD always is positive, pointing to the right for BV with a null vertical component or to the bottom for BV with a null horizontal component.

The optimal IBC AMVP index is signaled, which allows deriving the sign of the non-null BVD component at the decoder side. Consequently, the absolute value of the non-null component of the BVD is signaled to the decoder, improving the coding efficiency. The RRIBC mode is signaled using the existing syntax flag, and the direction of the flipping mode is derived from the bvNullCompDir flag.

# Experimental Results

The proposed method is tested on top of ECM-7.0 software [3] under JVET ECM common test conditions [4] in AI/RA/LDB configurations for the EE2 test. The simulation results are shown in Tables I to III:

Table I. Experimental result of the proposed method under AI configuration

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.22% | -0.09% | -0.07% | 98% | 98% |
| Class TGM | -0.34% | -0.28% | -0.35% | 98% | 96% |
| Overall | -0.28% | -0.19% | -0.21% | 98% | 97% |

Table II. Experimental result of the proposed method under RA configuration

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.12% | -0.05% | 0.06% | 100% | 100% |
| Class TGM | -0.31% | -0.33% | -0.21% | 100% | 100% |
| Overall | -0.22% | -0.19% | -0.07% | 100% | 100% |

Table III. Experimental result of the proposed method under LDB configuration

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.06% | -0.52% | -0.35% | 101% | 100% |
| Class TGM | -0.34% | -0.19% | -0.19% | 101% | 99% |
| Overall | -0.20% | -0.35% | -0.27% | 101% | 99% |

- Conclusions

This contribution proposes a modification in the IBC AMVP list construction. The merge candidates are clustered according to their L2 distance, and one candidate in each cluster is selected based on the TM cost. Blocks encoded using the Reconstruction-Reordered IBC (RRIBC) mode, two new BVP candidates are determined, and the sign of the non-null BVD component is inferred at the decoder side. This method is extended to any BVs having one null component, regardless of whether it uses the RRIBC mode. On top of ECM-7.0, the test results report some coding gain with a reduction in Enc./Dec. run-time. It is recommended to be adopted into the next ECM release.

- References

- Zhipin Deng, Kai Zhang, Li Zhang, “EE2-3.2: Reconstruction-Reordered IBC for screen content coding”, JVET-AA0070, July 2022.

- D. Coll, V. Warudkar, J.-K. Lee, “AHG12: BVP candidates clustering and BVD sign derivation for Reconstruction-Reordered IBC mode”, JVET-AB0173, October 2022.

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-7.0

- M. Karczewicz and Y. Ye, "Common Test Conditions and evaluation procedures for enhanced compression tool testing," Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-Y2017, 25th meeting, by teleconference, January 2022.

# Patent rights declaration(s)

Ofinno, LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).