| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0061-v2 |
| --- | --- |

| Title: | AHG9: Comments on Neural-network Post-filter Characteristics SEI Message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

Some modifications are proposed to the Neural-network post-filter characteristics SEI Message. Following is proposed:

- Proposal 1: It is proposed to move nnpfc_purpose syntax element to signal it earlier in the NNPFC SEI message.

- Proposal 2: Two asserted bug-fixes are proposed related to variables in NNPFC SEI message.

  - Bug-fix A: It is asserted that the outermost for loop in equation 84 which defines the process StoreOutputTensors( ), should use the variable numOutputPics instead of numInputPics.

  - Bug-fix B: It is asserted that when nnpfc_purpose has a value other than value 5, the variable numOutputPics is not derived and it should be derived.

- Proposal 3: It is proposed to either define a new syntax element for specifying the bit depth of chroma sample values in the input integer tensor or to define the semantics of an existing syntax element to apply to chroma samples in addition to the luma samples.

In v2, specification text is provided for additional aspects that were agreed.

# Introduction

Additional SEI messages for VSEI [1] specifies the syntax and semantics for Neural network post-filter characteristics SEI message and for neural-network post-filter activation SEI message. Neural network post-filter characteristics SEI message specifies a neural network that may be used as a post-processing filter. The use of specified post-processing filters for specific pictures is indicated with neural-network post-filter activation SEI messages.

Some modifications are proposed to the neural-network post-filter activation SEI message.

# Proposals

Some modifications are proposed to the Neural-network post-filter characteristics SEI. Message. Following is proposed:

- Proposal 1: It is proposed to move nnpfc_purpose syntax element to signal it earlier in the NNPFC SEI message.

- Proposal 2: Asserted bug-fixes are proposed related to variables in NNPFC SEI message.

  - Bug-fix A: It is asserted that the outermost for loop in equation 84 which defines the process StoreOutputTensors( ), should use the variable numOutputPics instead of numInputPics.

  - Bug-fix B: It is asserted that when nnpfc_purpose has a value other than value 5, the variable numOutputPics is not derived and it should be derived.

- Proposal 3: It is proposed to either define a new syntax element for specifying the bit depth of chroma sample values in the input integer tensor or to define the semantics of an existing syntax element to apply to chroma samples in addition to the luma samples.

The proposed changes are shown in red color compared to [1].

## Proposal 1

It is proposed to move nnpfc_purpose syntax element to signal it earlier in the NNPFC SEI message. It is asserted that parsing nnpfc_purpose syntax element as early as possible in the SEI message will be useful in determining whether the particular NNPFC SEI message has the purpose which is of interest to the receiver and whether it needs to parse other data in the message, which it may do only in that case.

As a result, it is also proposed to signal nnpfc_purpose unconditionally. It is asserted that knowing the purpose of a NNPFC message directly instead of indirectly by keeping track of nnpfc_id allows a receiver to unambiguously know the purpose information quickly.

As an example, a first NNPFC SEI message (which has nnpfc_formatting_and_purpose_flag is equal to 1 and which includes nnpfc_purpose syntax element), in decoding order, that has a particular nnpfc_id value within the current CLVS may have been lost and then the second NNPFC SEI message may have been signalled with the same nnpfc_id values and with nnpfc_formatting_and_purpose_flag equal to 0. In this case before discovering what may be the second NNPFC SEI message’s purpose (which would remain unknown due to conditional signalling), additional parsing would be needed. Currently this would need parsing of nnpfc_mode_idc and if that is equal to 1, parsing of any byte-alignment nnpfc_reserved_zero_bit bits, nnpfc_tag_uri, nnpfc_uri, and then nnpfc_formatting_and_purpose_flag syntax elements. It is asserted that moving the nnpfc_purpose flag earlier and signaling it unconditionally will help in this situation.

In this case it is also proposed to rename the flag nnpfc_formatting_and_purpose_flag as nnpfc_formatting_flag.

8.28 Neural-network post-filter characteristics SEI message

8.28.1 Neural-network post-filter characteristics SEI message syntax

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | ue(v) |
| nnpfc_id | ue(v) |
| nnpfc_mode_idc | ue(v) |
| if( nnpfc_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_reserved_zero_bit_a | u(1) |
| nnpfc_tag_uri | st(v) |
| nnpfc_uri | st(v) |
| } |  |
| nnpfc_formatting_and_purpose_flag | u(1) |
| if( nnpfc_formatting_and_purpose_flag ) { |  |
| nnpfc_purpose | ue(v) |
| /* input and output formatting */ |  |
| … | u(1) |
| } |  |

## When an NNPFC SEI message is not the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS, the value of nnpfc_purpose syntax element in it shall be the same as the value of the nnpfc_purpose syntax element in an SEI message that is the first NNPFC SEI message, in decoding order, that has that particular nnpfc_id value within the current CLVS.

## Proposal 2

Two asserted bug-fixes are proposed related to variables in NNPFC SEI message.

Bug-fix A:

It is asserted that the outermost for loop in equation 84 which defines the process StoreOutputTensors( ), should use the variable numOutputPics instead of the variable numInputPics, as the process applies to the number of output pictures.

nnpfc_out_order_idc indicates the output order of samples resulting from the post-processing filter.

The value of nnpfc_out_order_idc shall be in the range of 0 to 3, inclusive, in bitstreams conforming to this edition of this document. Values of 4 to 255, inclusive, for nnpfc_out_order_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_out_order_idc in the range of 4 to 255, inclusive. Values of nnpfc_out_order_idc greater than 255 shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use.

When nnpfc_purpose is equal to 2 or 4, nnpfc_out_order_idc shall not be equal to 3.

Table 22 contains an informative description of nnpfc_out_order_idc values.

Table 22 – Description of nnpfc_out_order_idc values

| nnpfc_out_ order_idc | Description |
| --- | --- |
| 0 | Only the luma matrix is present in the output tensor, thus the number of channels is 1. |
| 1 | Only the chroma matrices are present in the output tensor, thus the number of channels is 2. |
| 2 | The luma and chroma matrices are present in the output tensor, thus the number of channels is 3. |
| 3 | Four luma matrices and two chroma matrices are present in the output tensor, thus the number of channels is 6. This nnpfc_out_order_idc can only be used when the chroma format is 4:2:0. |
| 4..255 | Reserved |

The process StoreOutputTensors( ), for deriving sample values in the filtered output sample arrays FilteredYPic, FilteredCbPic, and FilteredCrPic from the output tensor outputTensor for a given vertical sample coordinate cTop and a horizontal sample coordinate cLeft specifying the top-left sample location for the patch of samples included in the input tensor, is specified as follows:

for( i = 0; i < numInputPicsnumOutputPics; i++ ) {
	if( nnpfc_out_order_idc  = =  0 )
		for( yP = 0; yP < outPatchHeight; yP++)
			for( xP = 0; xP < outPatchWidth; xP++ ) {
				yY = cTop * outPatchHeight / inpPatchHeight + yP
				xY = cLeft * outPatchWidth / inpPatchWidth + xP
				if ( yY < nnpfc_pic_height_in_luma_samples  &&  xY < nnpfc_pic_width_in_luma_samples )
					if( !nnpfc_component_last_flag )
						FilteredYPic[ i ][ xY ][yY ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
					else
						FilteredYPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]			}
	else if( nnpfc_out_order_idc  = =  1 )		(83)
		for( yP = 0; yP < outPatchCHeight; yP++)
			for( xP = 0; xP < outPatchCWidth; xP++ ) {
				xSrc = cLeft * horCScaling + xP
				ySrc = cTop * verCScaling + yP
				if ( ySrc < nnpfc_pic_height_in_luma_samples / outSubHeightC  &&
						xSrc < nnpfc_pic_width_in_luma_samples / outSubWidthC )
					if( !nnpfc_component_last_flag ) {
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 1 ][ yP ][ xP ]
					} else {
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 1 ]
					}
			}
	else if( nnpfc_out_order_idc  = =  2 )
		for( yP = 0; yP < outPatchHeight; yP++)
			for( xP = 0; xP < outPatchWidth; xP++ ) {
				yY = cTop * outPatchHeight / inpPatchHeight + yP
				xY = cLeft * outPatchWidth / inpPatchWidth + xP
				yC = yY / outSubHeightC 
				xC = xY / outSubWidthC 
				yPc = ( yP / outSubHeightC ) * outSubHeightC
				xPc = ( xP / outSubWidthC ) * outSubWidthC
				if ( yY < nnpfc_pic_height_in_luma_samples && xY < nnpfc_pic_width_in_luma_samples)
					if( !nnpfc_component_last_flag ) {
						FilteredYPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
						FilteredCbPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ 1 ][ yPc ][ xPc ]
						FilteredCrPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ 2 ][ yPc ][ xPc ]
					} else {
						FilteredYPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
						FilteredCbPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ yPc ][ xPc ][ 1 ]
						FilteredCrPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ yPc ][ xPc ][ 2 ]
					}
			}
	else if( nnpfc_out_order_idc  = =  3 )
		for( yP = 0; yP < outPatchHeight; yP++ )
			for( xP = 0; xP < outPatchWidth; xP++ ) {
				ySrc = cTop / 2 * outPatchHeight / inpPatchHeight + yP
				xSrc = cLeft / 2 * outPatchWidth / inpPatchWidth + xP
				if ( ySrc < nnpfc_pic_height_in_luma_samples / 2  &&
						xSrc < nnpfc_pic_width_in_luma_samples / 2 )
					if( !nnpfc_component_last_flag ) {
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
						FilteredYPic[ i ][ xSrc * 2 + 1 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ 1 ][ yP ][ xP ]
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ 2 ][ yP ][ xP ]
						FilteredYPic[ i ][ xSrc * 2 + 1][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ 3 ][ yP ][ xP ]
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 4 ][ yP ][ xP ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 5 ][ yP ][ xP ]
					} else {
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
						FilteredYPic[ i ][ xSrc * 2 + 1 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 1 ]
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 2 ]
						FilteredYPic[ i ][ xSrc * 2 + 1][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 3 ]
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 4 ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 5 ]
					}
			}
}

Bug-fix B:

It is asserted that when nnpfc_purpose has a value other than value 5, the variable numOutputPics is not derived. It is asserted that the variable numOutputPics should be used in the equation 84 which defines the process StoreOutputTensors( ). Thus, it is proposed to initialize the variable numOutputPics when nnpfc_purpose is not equal to 5.

nnpfc_interpolated_pics[ i ] specifies the number of interpolated pictures generated by the post-processing filter between the i-th and the ( i + 1 )-th picture used as input for the post-processing filter.

The variables numInputPics, specifying the number of pictures used as input for the post-processing filter, and numOutputPics, specifying the total number of pictures resulting from the post-processing filter, are derived as follows:

if( nnpfc_purpose  = =  5 ) {
	numInputPics = nnpfc_num_input_pics_minus2 + 2
	for( i = 0, numOutputPics = 0; i  <=  numInputPics − 2; i++ ) 	(76)
		numOutputPics  +=  nnpfc_interpolated_pics[ i ]
} else {
	numInputPics = 1
	numOutputPics = 1
	}

## Proposal 3

It is asserted that the NNPFC SEI messages takes as input separate bit depth for luma and chroma (BitDepthY and BitDepthC) and defines two separate functions InpY(x) and InpC(x) for luma and chroma samples. However, equation 80 calculates InpC(x) for chroma utilizing inpTensorBitDepth variable which is calculated based on the syntax element nnpfc_inp_tensor_bitdepth_minus8. However, the semantics of nnpfc_inp_tensor_bitdepth_minus8 are that it plus 8 specifies the bit depth of luma sample values in the input integer tensor. Two alternative options are proposed to handle this:

- Option 1: It is proposed to define an additional syntax element nnpfc_inp_tensor_bitdepth_chroma_minus8 for specifying the bit depth of chroma sample values in the input integer tensor. Or

- Option 2: It is proposed to define the semantics of nnpfc_inp_tensor_bitdepth_minus8 such that it plus 8 specifies the bit depth of luma and chroma sample values in the input integer tensor.

Option 1:

It is proposed to define an additional syntax element nnpfc_inp_tensor_bitdepth_chroma_minus8 for specifying the bit depth of chroma sample values in the input integer tensor. It is asserted that since the NNPFC SEI message uses separate bit depth values BitDepthY and BitDepthC for the luma sample array and chroma sample array of the cropped decoded output pictures and since the calculations in equation 79 and 80 define separate functions InpY( ) and InpC( ), it will be more flexible to define a separate syntax element nnpfc_inp_tensor_bitdepth_chroma_minus8 for specifying separately bit depth of chroma sample values in the input integer tensor which may be different than the bit depth of luma sample values in the input integer tensor.

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_id | ue(v) |
| nnpfc_mode_idc | ue(v) |
| … |  |
| nnpfc_formatting_and_purpose_flag | u(1) |
| if( nnpfc_formatting_and_purpose_flag ) { |  |
| nnpfc_purpose | ue(v) |
| … | u(1) |
| nnpfc_inp_format_idc | ue(v) |
| if( nnpfc_inp_format_idc = = 1 ) { |  |
| nnpfc_inp_tensor_bitdepth_luma_minus8 | ue(v) |
| nnpfc_inp_tensor_bitdepth_chroma_minus8 | ue(v) |
| } |  |
| nnpfc_inp_order_idc | ue(v) |
| … |  |
| } |  |
| } |  |
| … |  |
| } |  |
| } |  |

The neural-network post-filter characteristics (NNPFC) SEI message specifies a neural network that may be used as a post-processing filter. The use of specified post-processing filters for specific pictures is indicated with neural-network post-filter activation SEI messages.

Use of this SEI message requires the definition of the following variables:

–	Cropped decoded output picture width and height in units of luma samples, denoted herein by CroppedWidth and CroppedHeight, respectively.

–	Luma sample array CroppedYPic[ idx ] and chroma sample arrays CroppedCbPic[ idx ] and CroppedCrPic[ idx ], when present, of the cropped decoded output pictures with idx in the range of 0 to numInputPics − 1, inclusive, that are used as input for the post-processing filter.

–	Bit depth BitDepthY for the luma sample array of the cropped decoded output pictures.

–	Bit depth BitDepthC for the chroma sample arrays, if any, of the cropped decoded output pictures.

–	A chroma format indicator, denoted herein by ChromaFormatIdc, as described in subclause ‎7.3.

–	When nnpfc_auxiliary_inp_idc is equal to 1, a filtering strength control value StrengthControlVal that shall be a real number in the range of 0 to 1, inclusive.

The variables SubWidthC and SubHeightC are derived from ChromaFormatIdc as specified by Table 2.

NOTE 1 – More than one NNPFC SEI message can be present for the same picture. When more than one NNPFC SEI message with different values of nnpfc_id is present or activated for the same picture, they can have the same or different values of nnpfc_purpose and nnpfc_mode_idc.

…

nnpfc_inp_format_idc indicates the method of converting a sample value of the cropped decoded output picture to an input value to the post-processing filter. When nnpfc_inp_format_idc is equal to 0, the input values to the post-processing filter are real numbers and the functions InpY( ) and InpC( ) are specified as follows:

InpY( x ) = x ÷ ( ( 1  <<  BitDepthYY ) − 1 )		(77)

InpC( x )= x ÷ ( ( 1  <<  BitDepth CC ) − 1 )		(78)

When nnpfc_inp_format_idc is equal to 1, the input values to the post-processing filter are unsigned integer numbers and the functions InpY( ) and InpC( ) are specified as follows:

shiftY = BitDepthY − inpTensorBitDepth
if( inpTensorBitDepth  >=  BitDepthY)
	InpY( x ) = x  <<  ( inpTensorBitDepth − BitDepthY )	(79)
else
	InpY( x ) = Clip3(0, ( 1  <<  inpTensorBitDepth ) − 1, ( x + ( 1  <<  ( shiftY − 1 ) ) )  >>  shiftY )

shiftC = BitDepthC − inpTensorBitDepthChroma
if( inpTensorBitDepthChroma  >=  BitDepthC )
	InpC( x ) = x  <<  ( inpTensorBitDepthChroma − BitDepthC ) 	(80)
else
	InpC( x ) = Clip3(0, ( 1  <<  inpTensorBitDepthChroma ) − 1, ( x + ( 1  <<  ( shiftC − 1 ) ) )  >>  shiftC )

The variable inpTensorBitDepth is derived from the syntax element nnpfc_inp_tensor_bitdepth_luma_minus8 as specified below. The variable inpTensorBitDepthChroma is derived from the syntax element nnpfc_inp_tensor_bitdepth_chroma_minus8 as specified below.

Values of nnpfc_inp_format_idc greater than 1 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages that contain reserved values of nnpfc_inp_format_idc.

nnpfc_inp_tensor_bitdepth_luma_minus8 plus 8 specifies the bit depth of luma sample values in the input integer tensor. The value of inpTensorBitDepth is derived as follows:

inpTensorBitDepth = nnpfc_inp_tensor_bitdepth_luma_minus8 + 8	(81)

It is a requirement of bitstream conformance that the value of nnpfc_inp_tensor_bitdepth_luma_minus8 shall be in the range of 0 to 24, inclusive.

nnpfc_inp_tensor_bitdepth_chroma_minus8 plus 8 specifies the bit depth of chroma sample values in the input integer tensor. The value of inpTensorBitDepthChroma is derived as follows:

inpTensorBitDepthChroma = nnpfc_inp_tensor_bitdepth_chroma_minus8 + 8	(81)

It is a requirement of bitstream conformance that the value of nnpfc_inp_tensor_bitdepth_chroma_minus8 shall be in the range of 0 to 24, inclusive.

For output tensors:

Changes compared to [1] are shown highlighted.

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_id | ue(v) |
| nnpfc_mode_idc | ue(v) |
| … |  |
| nnpfc_formatting_and_purpose_flag | u(1) |
| if( nnpfc_formatting_and_purpose_flag ) { |  |
| … | ue(v) |
| nnpfc_out_format_idc | ue(v) |
| if( nnpfc_out_format_idc = = 1 ) { |  |
| nnpfc_out_tensor_bitdepth_luma_minus8 | ue(v) |
| nnpfc_out_tensor_bitdepth_chroma_minus8 | ue(v) |
| } |  |
| nnpfc_out_order_idc | ue(v) |
| nnpfc_constant_patch_size_flag | u(1) |
| … |  |
| } |  |
| } |  |
| … |  |
| } |  |
| } |  |

nnpfc_out_format_idc equal to 0 indicates that the sample values output by the post-processing filter are real numbers where the value range of 0 to 1, inclusive, maps linearly to the unsigned integer value range of 0 to ( 1  <<  bitDepth ) – 1, inclusive, for any desired bit depth bitDepth for subsequent post-processing or displaying.

nnpfc_out_format_flag equal to 1 indicates that the luma sample values output by the post-processing filter are unsigned integer numbers in the range of 0 to ( 1  <<  ( nnpfc_out_tensor_bitdepth_luma_minus8 + 8 ) ) − 1, inclusive and chroma sample values output by the post-processing filter are unsigned integer numbers in the range of 0 to ( 1  <<  ( nnpfc_out_tensor_bitdepth_chroma_minus8 + 8 ) ) − 1, inclusive.

Values of nnpfc_out_format_idc greater than 1 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages that contain reserved values of nnpfc_out_format_idc.

nnpfc_out_tensor_bitdepth_luma_minus8 plus 8 specifies the bit depth of luma sample values in the output integer tensor. The value of nnpfc_out_tensor_bitdepth_luma_minus8 shall be in the range of 0 to 24, inclusive.

nnpfc_out_tensor_bitdepth_chroma_minus8 plus 8 specifies the bit depth of chroma sample values in the output integer tensor. The value of nnpfc_out_tensor_bitdepth_chroma_minus8 shall be in the range of 0 to 24, inclusive.

Option 2:

It is proposed to define the semantics of nnpfc_inp_tensor_bitdepth_minus8 such that it plus 8 specifies the bit depth of luma and chroma sample values in the input integer tensor.

The proposed changes are as follows:

nnpfc_inp_tensor_bitdepth_minus8 plus 8 specifies the bit depth of luma and chroma sample values in the input integer tensor. The value of inpTensorBitDepth is derived as follows:

inpTensorBitDepth = nnpfc_inp_tensor_bitdepth_minus8 + 8	(81)

It is a requirement of bitstream conformance that the value of nnpfc_inp_tensor_bitdepth_minus8 shall be in the range of 0 to 24, inclusive.

# Conclusion

This document proposes modifications to neural network post processing characteristics SEI. It is proposed to adopt the proposals in the next version of “Additional SEI messages for VSEI” text.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] MDS21947_WG05_N00158/ JVET-AB2006, Additional SEI messages for VSEI, Online, October 2022

Acknowledgement
These research results were obtained from the commissioned research (No.05101) by National Institute of Information and Communications Technology (NICT), Japan.