| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0062-v3 |
| --- | --- |

| Title: | AHG9: On NNPFC SEI Message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande Ahmed Sidiya | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

Some modifications are proposed to the Neural-network post-filter characteristics (NNPFC) SEI Message. Following is proposed:

- Proposal A: It is proposed to modify a constraint when nnpfc_constant_patch_size_flag is equal to 1. It is asserted that without the proposed modification, the actual input size to the post-processing filter may not be a multiple of a specific value as may be required by some typical neural network post filters.

- Proposal B: It is proposed to define a valid value range for the two ue(v) coded syntax elements nnpfc_num_input_pics_minus2 and nnpfc_interpolated_pics[ i ]. Currently there is no range specified for these two syntax elements.

# Introduction

Additional SEI messages for VSEI [1] specifies the syntax and semantics for Neural network post-filter characteristics SEI message and for neural-network post-filter activation SEI message. Neural network post-filter characteristics SEI message specifies a neural network that may be used as a post-processing filter. The use of specified post-processing filters for specific pictures is indicated with neural-network post-filter activation SEI messages.

Some modifications are proposed to the NNPFC SEI message.

# Proposals

Some modifications are proposed to the Neural-network post-filter characteristics (NNPFC) SEI Message. Following is proposed:

- Proposal A: It is proposed to modify a constraint when nnpfc_constant_patch_size_flag is equal to 1.

- Proposal B: It is proposed to define a valid value range for the two ue(v) coded syntax elements nnpfc_num_input_pics_minus2 and nnpfc_interpolated_pics[ i ].

## Proposal A

It is proposed to modify a constraint when nnpfc_constant_patch_size_flag is equal to 1. It is asserted that without the proposed modification, the actual input size to the post-processing filter may not be a multiple of a specific value as may be the required by some typical neural network post filters.

It is asserted that when nnpfc_constant_patch_size_flag is equal to 0, i.e. when the post-processing filter accepts any patch size that is a positive integer multiple of the patch size indicated by nnpfc_patch_width_minus1 and nnpfc_patch_height_minus1, and the overlapping horizontal and vertical sample counts of adjacent input tensors of the post-processing filter needs to be considered in relation to the variables inpPatchWidth and inpPatchHeight which represent the patch size width and the patch size height, respectively.

It is asserted that without the proposed modification related to this, the actual input size to the post-processing filter as defined by DeriveInputTensors() – which is specified by the for loops which are defined for the range of yP values from yP = −overlapSize to yP < inpPatchHeight + overlapSize and the range of xP values from −overlapSize to xP < inpPatchWidth + overlapSize may not be a multiple of a specific value as may be the required by typical neural network post filters.

The proposed changes are shown in red color compared to [1].

nnpfc_constant_patch_size_flag equal to 1 indicates that the post-processing filter accepts exactly the patch size indicated by nnpfc_patch_width_minus1 and nnpfc_patch_height_minus1 as input. nnpfc_constant_patch_size_flag equal to 0 indicates that the post-processing filter accepts any patch size that is a positive integer multiple of the patch size indicated by nnpfc_patch_width_minus1, and nnpfc_patch_height_minus1 and nnpfc_overlap as input.

nnpfc_patch_width_minus1 + 1, when nnpfc_constant_patch_size_flag equal to 1, indicates the horizontal sample counts of the patch size required for the input to the post-processing filter. The value of nnpfc_patch_width_minus1 shall be in the range of 0 to Min( 32766, CroppedWidth − 1 ), inclusive.

nnpfc_patch_height_minus1 + 1, when nnpfc_constant_patch_size_flag equal to 1, indicates the vertical sample counts of the patch size required for the input to the post-processing filter. The value of nnpfc_patch_height_minus1 shall be in the range of 0 to Min( 32766, CroppedHeight − 1 ), inclusive.

Let the variables inpPatchWidth and inpPatchHeight be the patch size width and the patch size height, respectively.

If nnpfc_constant_patch_size_flag is equal to 0, the following applies:

–	The values of inpPatchWidth and inpPatchHeight are either provided by external means not specified in this document or set by the post-processor itself.

–	The value of inpPatchWidth + 2*overlapSize shall be a positive integer multiple of nnpfc_patch_width_minus1 + 1 and inpPatchWidth shall be less than or equal to CroppedWidth. The value of inpPatchHeight  + 2*overlapSize shall be a positive integer multiple of nnpfc_patch_height_minus1 + 1 and inpPatchHeight  shall be less than or equal to CroppedHeight.

Otherwise (nnpfc_constant_patch_size_flag is equal to 1), the value of inpPatchWidth is set equal to nnpfc_patch_width_minus1 + 1 and the value of inpPatchHeight is set equal to nnpfc_patch_height_minus1 + 1.

nnpfc_overlap indicates the overlapping horizontal and vertical sample counts of adjacent input tensors of the post-processing filter. The value of nnpfc_overlap shall be in the range of 0 to 16383, inclusive.

The variables outPatchWidth, outPatchHeight, horCScaling, verCScaling, outPatchCWidth, outPatchCHeight, and overlapSize are derived as follows:

outPatchWidth = ( nnpfc_pic_width_in_luma_samples * inpPatchWidth ) / CroppedWidth	(84)

outPatchHeight = ( nnpfc_pic_height_in_luma_samples * inpPatchHeight ) / CroppedHeight	(85)

horCScaling = SubWidthC / outSubWidthC		(86)

verCScaling = SubHeightC / outSubHeightC		(87)

outPatchCWidth = outPatchWidth * horCScaling		(88)

outPatchCHeight = outPatchHeight * verCScaling		(89)

overlapSize = nnpfc_overlap		(90)

It is a requirement of bitstream conformance that outPatchWidth * CroppedWidth shall be equal to nnpfc_pic_width_in_luma_samples * inpPatchWidth and outPatchHeight * CroppedHeight shall be equal to nnpfc_pic_height_in_luma_samples * inpPatchHeight.

Example:

As an example, if the neural network post-filter accepts any patch size which is a multiple of 8 (both horizontally and vertically), then nnpfc_constant_patch_size_flag is set equal to 0, nnpfc_patch_width_minus1 is set equal to 7 and nnpfc_patch_height_minus1 is set equal to 7. Then with the current constraints, inpPatchWidth and inpPatchHeight are both required to be multiple of 8. However, in this case depending upon the overlapSize (i.e. nnpfc_overlap) value, the for loops in DeriveInputTensors can result in the actual input to the neural network post-filter to not be a multiple of 8 (horizontally and vertically), as the for loops in it are as follows:

for( yP = −overlapSize; yP < inpPatchHeight + overlapSize; yP++)
			for( xP = −overlapSize; xP < inpPatchWidth + overlapSize; xP++ ) {
				
In this case the proposed modified constraint will result in the actual input to the neural network post-filter to be a multiple of 8 (horizontally and vertically).

## Proposal B

It is proposed to define a valid value range for the two ue(v) coded syntax elements nnpfc_num_input_pics_minus2 and nnpfc_interpolated_pics[ i ]. Currently there is no range specified for these two syntax elements.

The proposed changes are shown in red color compared to [1].

nnpfc_num_input_pics_minus2 plus 2 specifies the number of decoded output pictures used as input for the post-processing filter. The value of nnpfc_num_input_pics_minus2 shall be in the range of 0 to 14, inclusive, in bitstreams conforming to this edition of this document.

nnpfc_interpolated_pics[ i ] specifies the number of interpolated pictures generated by the post-processing filter between the i-th and the ( i + 1 )-th picture used as input for the post-processing filter. The value of nnpfc_interpolated_pics[ i ] shall be in the range of 0 to 64, inclusive, in bitstreams conforming to this edition of this document.

# Conclusion

This document proposes modifications to neural network post processing characteristics SEI. It is proposed to adopt the proposal in the next version of “Additional SEI messages for VSEI” text.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] MDS21947_WG05_N00158/ JVET-AB2006, Additional SEI messages for VSEI, Online, October 2022

Acknowledgement
These research results were obtained from the commissioned research (No.05101) by National Institute of Information and Communications Technology (NICT), Japan.