| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, teleconference, 11–20 January 2023 | Document: JVET-AC0063-v2 |
| --- | --- |

| Title: | EE1-1.3: On chroma order adjustment in NNLF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhenyu Dai Yue Yu Haoping Yu Dong Wang | Tel: Email: | daizhenyu@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution reports the EE1-1.3 results of JVET-AB0158[1]. A chroma order adjustment method was proposed based on JVET-AA0088[2] to further improve the coding performance of Filter Set #0. In JVET-AA0088, the chroma information including U and V components are parts of the inputs/outputs to the neural network. JVET-AB0158 proposed to allow the input/output order switch between the U and V components at the frame level. The switch is signaled at frame level. On top of the NNVC3.0 anchor with the activation of specific Filter Set, it is reported that the proposed method can further improve the coding efficiency, {Y, U, V, EncT, DecT}, shown as follows:

EE1-1.3 (on top of NNVC3.0 Filter Set #0):

RA: Y -0.01%, U -1.50%, V -0.62%, EncT 106%, DecT 102%;

LDB: Y -0.13%, U -2.45%, V -1.28%, EncT 107%, DecT 104%.

EE1-1.3 (on top of NNVC3.0 anchor):

RA: Y -8.92%, U -19.69%, V -19.67%, EncT 186%, DecT 70126%;

LDB: Y -8.34%, U -21.05%, V -21.39%, EncT 191%, DecT 57544%.

Appendix information: EE1-1.3 (on top of NNVC3.0 Filter Set #1):

RA: Y -0.01%, U -0.97%, V -0.67%, EncT 99%, DecT 95%;

LDB: Y -0.01%, U -3.05%, V -2.48%, EncT 99%, DecT 94%.

Appendix information: EE1-1.3 (on top of NNVC3.0 anchor):

RA: Y -9.49%, U -21.43%, V -20.91%, EncT 216%, DecT 46317%;

LDB: Y -8.51%, U -18.24%, V -16.25%, EncT 225%, DecT 35182%.

Introduction

In JVET-AA0088[2], a neural network-based in-loop filter with constrained memory size and low complexity was presented and was adopted as one of the two NNLF sets (Filter Set #0) in the NNVC reference software.

The network structure of JVET-AA0088 is shown in Fig. 1. Along with the reconstructed image (rec_yuv), additional side information is also fed into the network, such as the prediction image (pred_yuv), slice QP, base QP and slice type.

Fig. 1: Network architecture of JVET-AA0088

Proposed method

In the current neural network-based in-loop filter design, the input tensors are constructed by concatenating the video colour components and corresponding information in a fixed order defined in the training stage. Similarly, the output video colour components are extracted from the output tensor in a fixed order. In JVET-AA0088, the input order is {rec_y, rec_u, rec_v, pred_y, pred_u, pred_v, slice QP, base QP, slice type}, while the output order is {output_y, output_u, output_v}. The same fixed orders for input and output are followed in application of the NNLF (i.e. in inference).

EE1-1.3 proposed that, instead of following a fixed order of input/output information, a frame level chroma order adjustment can be made to switch the input/output order of U and V components during the NNLF inference stage for inter frames. When chroma order adjustment is true, the input order is modified to {rec_y, rec_v, rec_u, pred_y, pred_v, pred_u, slice QP, base QP, slice type}, and the output order is modified to {output_y, output_v, output_u}.

A frame-level flag is signaled to indicate whether the chroma order adjustment method is used.

Encoder optimization

EE1-1.3 employed an encoder optimization scheme to only filter 1 out of every 8 CTUs to decide which chroma order is used for a frame. As a result, this encoding optimization significantly reduces encoding complexity. As shown in Figure 2, only the green CTUs are filtered and used in calculating the cost to decide which chroma order will be used. After deciding a chroma order , all CTUs will be filtered based upon the selected chroma order.

|  |  |  | CTU |
| --- | --- | --- | --- |

Figure 2. Proposed encoder optimization

Experiment results

EE1-1.3 is implemented on top of the NNVC3.0 with the macro NnlfOption=1, which indicates the activation of the Filter Set #0 from JVET-AA0088. The simulations were performed by following the NNVC common test conditions (CTC)[3]. Since the chroma order adjustment method is only used for B-Slice, only Random Access and Low Delay B configurations were tested.

Table 1. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over NNVC3.0 (NnlfOption=1) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | -3.19% | -0.71% | 107% | 102% |
| Class A2 | -0.02% | -1.14% | -0.38% | 107% | 102% |
| Class B | -0.02% | -1.23% | -0.57% | 106% | 103% |
| Class C | -0.04% | -0.84% | -0.80% | 105% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -1.50% | -0.62% | 106% | 102% |
| Class D | -0.03% | -0.11% | -1.74% | 106% | 103% |
| Class F | -0.05% | -0.54% | -0.98% | 109% | 110% |

Table 2. Performance of the proposed method (LDB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over NNVC3.0 (NnlfOption=1) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.09% | -2.38% | -0.78% | 106% | 104% |
| Class C | -0.19% | -2.53% | -1.90% | 105% | 103% |
| Class E | -0.10% | -1.58% | -1.04% | 112% | 106% |
| Overall | -0.13% | -2.45% | -1.28% | 107% | 104% |
| Class D | -0.04% | -2.12% | -4.30% | 105% | 103% |
| Class F | -0.20% | -1.37% | -3.12% | 109% | 110% |

Table 3. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over NNVC3.0 (anchor) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -9.19% | -15.84% | -19.46% | 199% | 77153% |
| Class A2 | -9.52% | -20.37% | -15.04% | 194% | 74674% |
| Class B | -8.60% | -20.03% | -21.20% | 192% | 74523% |
| Class C | -8.66% | -21.62% | -21.38% | 164% | 57717% |
| Class E |  |  |  |  |  |
| Overall | -8.92% | -19.69% | -19.67% | 186% | 70126% |
| Class D | -9.77% | -20.09% | -22.44% | 166% | 53219% |
| Class F | -3.84% | -12.62% | -11.93% | 248% | 36065% |

Table 4. Performance of the proposed method (LDB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over NNVC3.0 (anchor) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -7.87% | -20.15% | -20.93% | 178% | 63800% |
| Class C | -8.43% | -22.17% | -21.96% | 148% | 49398% |
| Class E | -8.99% | -16.97% | -17.52% | 298% | 59384% |
| Overall | -8.34% | -21.05% | -21.39% | 191% | 57544% |
| Class D | -9.65% | -20.55% | -24.01% | 152% | 47108% |
| Class F | -4.38% | -11.85% | -11.65% | 241% | 29193% |

Appendix information

In JVET-AA0111[4], a deep learning-based in-loop filtering method was presented and was adopted as one of the two NNLF sets (Filter Set #1) in the NNVC reference software.

The network structure of JVET-AA0111 is shown in Fig. 3. The input of the network comprises reconstruction, prediction, boundary strengths, and QP (split is additionally taken as input in intra slice models).

Fig. 3: Network architecture of JVET-AA0111

The chroma order adjustment method can also be implemented on top of Filter Set #1 by the similar principle, this appendix shows the corresponding performance.

The simulations were performed by following the NNVC common test condition (CTC)[3]. Since the proposed method is only used for B-Slice, only Random Access and Low Delay B configurations were tested.

Table 5. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over NNVC3.0 (NnlfOption=2) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -1.53% | -0.86% | 99% | 95% |
| Class A2 | 0.00% | -0.80% | -0.19% | 99% | 94% |
| Class B | 0.00% | -1.02% | -0.62% | 100% | 95% |
| Class C | -0.01% | -0.63% | -0.94% | 100% | 96% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.97% | -0.67% | 99% | 95% |
| Class D | 0.03% | -0.70% | -0.70% | 99% | 95% |
| Class F | 0.02% | -0.36% | -1.00% | 99% | 97% |

Table 6. Performance of the proposed method (LDB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over NNVC3.0 (NnlfOption=2) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -3.30% | -1.27% | 100% | 95% |
| Class C | -0.05% | -2.73% | -3.99% | 100% | 94% |
| Class E | 0.08% | -1.72% | -2.24% | 99% | 94% |
| Overall | -0.01% | -3.05% | -2.48% | 99% | 94% |
| Class D | 0.00% | -1.90% | -1.92% | 100% | 95% |
| Class F | 0.05% | -1.70% | -3.01% | 99% | 99% |

Table 7. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over NNVC3.0 (anchor) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -8.90% | -17.05% | -19.41% | 232% | 49837% |
| Class A2 | -10.24% | -20.82% | -17.58% | 224% | 48516% |
| Class B | -9.05% | -23.75% | -22.13% | 224% | 48031% |
| Class C | -9.92% | -22.26% | -23.01% | 190% | 40463% |
| Class E |  |  |  |  |  |
| Overall | -9.49% | -21.43% | -20.91% | 216% | 46317% |
| Class D | -11.45% | -24.14% | -24.46% | 185% | 38710% |
| Class F | -5.00% | -12.07% | -11.48% | 301% | 22402% |

Table 8. Performance of the proposed method (LDB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over NNVC3.0 (anchor) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -7.85% | -18.72% | -14.81% | 205% | 36666% |
| Class C | -9.19% | -17.65% | -18.04% | 173% | 35279% |
| Class E | -8.72% | -18.55% | -19.04% | 373% | 32720% |
| Overall | -8.51% | -18.24% | -16.25% | 225% | 35182% |
| Class D | -10.82% | -20.99% | -20.19% | 172% | 33075% |
| Class F | -4.90% | -10.88% | -9.13% | 295% | 18152% |

Conclusions

This contribution proposes a chroma order adjustment method in the NNVC reference software to further improve the coding performance. This method can provide a good chroma coding improvement both on top of Filter Set #0 and Filter Set #1 without increasing decoder-side complexity. It is suggested to adopt this method into both Filter Set #0 and Filter Set #1 in the NNVC reference software.

References

- [AHG11] On chroma order adjustment in NNLF, JVET-AB0158, Z. Dai, Y. Yu, H. Yu, D. Wang

- EE1-1.5: neural network based in-loop filter with a single model , JVET-AA0088, L. Wang, S. Lin, X. Xu, S. Liu (Tencent), F. Galpin (InterDigital)

- Common Test Conditions and evaluation procedures for neural network-based video coding technology, JVET-AA2016, E. Alshina, R.-L. Liao, S. Liu, A. Segall

- EE1-1.6: Deep In-Loop Filter With Fixed Point Implementation, JVET-AA0111, Y. Li, K. Zhang, J. Li, L. Zhang (Bytedance), H. Wang, M. Coban, A.M. Kotra, M. Karczewicz (Qualcomm), F. Galpin (InterDigital), K. Andersson, J. Ström, D. Liu, R. Sjöberg (Ericsson)

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).