| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0065-v1 |
| --- | --- |

| Title: | Non-EE1: On flipping of input and output of model in NNVC filter set 0 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhihuang Xie Yue Yu Haoping Yu Dong Wang | Tel: Email: | xiezhihunag@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

In this contribution, flipping operations applied to the inputs and output of the NNVC filter set 0[1] neural network are proposed to further improve the performance. The input of filter set 0 includes reconstructed samples, prediction samples, BaseQP, SliceQP and SliceType. When a flipping operation is applied, only the reconstructed samples and prediction samples are flipped and fed into the network with the other inputs to perform the inference. Then the output of network is flipped back to restore the order of the filtered samples. On top of NNVC-3.0 filter set 0, the performance of the proposed method is reported as below:

Compared with NNVC-3.0 filter set 0:

RA:   Y -0.18%,  U -0.37%,  V -0.44%,  EncT 113%,  DecT 100%;

LDB:  Y -0.57%,  U -1.46%,  V -2.03%,  EncT 114%,  DecT 102%.

Introduction

In NNVC-3.0, neural-network based filter set 0 is treated as a loop filter and applied before ALF. The architecture of filter set 0 is shown in Fig. 1. As shown in the architecture diagram, the inputs of the network are labelled rec_yuv (reconstructed yuv), pred_yuv (prediction yuv), BaseQP, SliceQP and Slice type, respectively. The output of the network is labelled output_yuv.

Fig. 1 Architecture of the network in filter set 0

Proposed method

Flipping operation

In the proposed method, the rec_yuv and pred_yuv inputs of the network may be flipped horizontally or vertically before injection to the network. When flipping is performed on the inputs, the output of the network is flipped back to restore the correct order of the filtered samples. The horizontal and vertical flipping operations are shown in Fig. 2 and Fig. 3 respectively.

Fig. 2 Horizontal flip

Fig. 3 Vertical flip

An encoder optimization similar to that used in [2], is also used to reduce the encoding time in this contribution.

The proposed method is only allowed in B-slices. A slice-level flag is signalled to indicate the usage of this flipping operation, and if it is applied, one additional slice-level flag is signalled to indicate the flipping type. Then all CTUs in the slice apply the same flipping operation before and after network inference.

Inference

The inference information is identical to NNVC-3.0 filter set 0.

Table. 1 Inference Information

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 1.9M |
|  | Total Number of Parameters (All Models) | 1.9M |
|  | Parameter Precision (Bits) | 16 |
|  | Memory Parameter (MB) | 3.8MB/model, 1 model in all |
|  | Multiply Accumulate (kMAC/pixel) | 485K (w/o block extension) 615K (w/ block extension) |
|  | Calculation Method | block basis |
| Optional |  |  |
|  | Total Conv. Layers | 101 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 144x144 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

Experimental results

The proposed method is implemented on top of NNVC-3.0 filter set 0 and all the tests are conducted under the common test condition[3]. Only filter set 0 is enabled, which means configuration NnlfOption is set to 1.

Compared with NNVC-3.0 filter set 0 (NnlfOption = 1)

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.03% | -0.23% | 114% | 99% |
| Class A2 | -0.24% | -0.69% | -0.39% | 114% | 99% |
| Class B | -0.15% | -0.38% | -0.42% | 114% | 99% |
| Class C | -0.23% | -0.37% | -0.65% | 111% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.18% | -0.37% | -0.44% | 113% | 100% |
| Class D | -0.24% | -0.40% | -0.56% | 112% | 102% |
| Class F | -0.14% | -0.44% | -0.43% | 118% | 106% |

|  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.38% | -1.43% | -1.95% | 113% | 101% |
| Class C | -0.63% | -1.50% | -2.13% | 111% | 101% |
| Class E | -0.82% | -1.50% | -1.14% | 122% | 107% |
| Overall | -0.57% | -1.46% | -2.03% | 114% | 102% |
| Class D | -0.47% | -1.59% | -2.04% | 111% | 101% |
| Class F | -0.40% | -1.59% | -1.45% | 118% | 110% |

Compared with NNVC-3.0 anchor (NnlfOption = 0)

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -9.36% | -13.17% | -19.09% | 200% | 76663% |
| Class A2 | -9.72% | -20.02% | -15.05% | 187% | 70783% |
| Class B | -8.73% | -19.35% | -21.10% | 189% | 74116% |
| Class C | -8.82% | -21.24% | -21.26% | 152% | 52962% |
| Class E |  |  |  |  |  |
| Overall | -9.08% | -18.75% | -19.53% | 180% | 67598% |
| Class D | -9.95% | -20.32% | -21.57% | 155% | 52449% |
| Class F | -3.92% | -12.54% | -11.45% | 229% | 30126% |

|  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -8.13% | -19.32% | -21.86% | 170% | 52549% |
| Class C | -8.84% | -21.29% | -22.14% | 146% | 42874% |
| Class E | -9.65% | -16.96% | -17.57% | 266% | 47621% |
| Overall | -8.75% | -20.20% | -21.99% | 181% | 47909% |
| Class D | -10.04% | -20.02% | -22.17% | 141% | 39125% |
| Class F | -4.56% | -12.07% | -9.94% | 211% | 24445% |

Conclusion

In this contribution, horizontal and vertical flipping operations are proposed to be applied to both input and output of the network to improve the performance. This method brings a good trade-off between coding gain and complexity. It is suggested to include this contribution into next round of EE for further investigation.

Reference

[1] L. Wang, S. Lin, X. Xu, S. Liu, F. Galpin. EE1-1.5: Neural network based in-loop filter with a single model. JVET-AA0088, 27th Meeting, by teleconference, 13–22 July 2022

[2] Z. Xie, Y. Yu, H. Yu, D. Wang. EE1-1.3: On BaseQP adjustment in CNNLF. JVET-AA0122, 27th Meeting, by teleconference, 13-22 July 2022

[3] E. Alshina, R.-L. Liao, S. Liu, A. Segall. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-AB2016, 28th Meeting, Mainz DE, 21–28 Oct 2022

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).