| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0066-v2 |
| --- | --- |

| Title: | EE1-related: Improvement on EE1-1.7 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhihuang Xie Yue Yu Haoping Yu Dong Wang | Tel: Email: | xiezhihuang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

In EE1-1.7, two aspects were proposed to improve the coding performance of NNVC-3.0 filter set #1. First, it is proposed to feed collocated blocks from reference frames into the CNN-based in-loop filtering. Second, the input samples of CNN can be flipped and the output samples of CNN can be flipped back. This contribution proposes a slice-level model switching method to further improve the performance of filter set #1. Compared with EE1-1.7.1, the performance is reported as below:

RA:   Y  0.00%,  U 0.00%,  V 0.00%,  EncT 100%,  DecT 100%;

LDB:  Y  -0.93%,  U 0.01%,  V 0.06%,  EncT 103%,  DecT 100%.

Introduction

In JVET-AB0073[1], two aspects were proposed to improve the performance of NNVC-3.0 filter set #1. The first aspect is that collocated blocks from pictures in reference picture list 0 and 1 are taken as additional inputs to a multiple frame-based CNN model. This method is only allowed for B slices belonging to temporal layers with tid > 2. In the random access configuration of the common test condition[2], this is the highest 3 temporal layers. Fig.1 shows the architecture of this multiple frame-based CNN model. The second aspect is that horizontal flipping can be applied to input samples of CNN. The reconstruction and all the auxiliary inputs are flipped horizontally then fed into the CNN. The output of the CNN is then flipped again to get the final filtered samples.

Fig.1 Architecture of multiple frame-based CNN model in EE1-1.7

Proposed method

Because the first aspect, multiple frame-based CNN model, is only enabled for temporal layers with tid > 2, this model is not used in B slices for LDB configuration.

To get more potential performance and higher usage of this model, it is proposed that the original inter-luma model and the multiple frame-based model may be switched at the slice level for temporal layer slices with tid <= 2. For higher temporal layer slices, which is identical to EE1-1.7.1, multiple frame-based model remains used in B slices by default. A slice-level flag is signaled for lower temporal layer B slices to indicate which model is used. If the flag is true, all CTUs in the slice use the multiple frame-based model. Otherwise, all CTUs in the slice use the default inter-luma model.

In addition, if the picture POC from RPL0 is equal to the picture POC from RPL1 and there are at least two reference pictures in RPL1, then the second reference picture is taken as Col_1.

To speed up the encoder, only half of the total number of CTUs are filtered during the encoder-decision process. For example, if the current slice has 12 CTUs, then only 6 of them are filtered by multiple frame-based model and inter-luma model to calculate the cost of each other, respectively. Finally, the model with lower cost is selected as the final model to be used for all CTUs in the current slice.

Other aspects are unchanged from EE1-1.7.1.

Inference

The inference information is identical to EE1-1.7.1.

Table 1. Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 1.56M/model, 5 models in total |
|  | Parameter Precision (Bits) | 16 (I) |
|  | Memory Parameter (MB) | 3.1MB/model, 5 models in total |
|  | Multiply Accumulate (kMAC/pixel) | 664K/pixel (assuming block-level input) |
| Optional |  |  |
|  | Total Conv. Layers | 25+16 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 128×128, 256×256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage |  |
|  | Other information: |  |

Experimental results

The proposed method is implemented on top of EE1-1.7.1 and all the tests are conducted under the common test condition[2]. Only filter set 1 is enabled, which means configuration NnlfOption is set to 2.

Compared with EE1-1.7.1

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |

|  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.71% | 0.21% | -0.01% | 103% | 98% |
| Class C | -0.77% | -0.24% | 0.16% | 102% | 98% |
| Class E | -1.54% | -0.42% | 0.12% | 104% | 109% |
| Overall | -0.93% | 0.01% | 0.06% | 103% | 100% |
| Class D | -0.60% | 0.23% | 0.53% | 103% | 96% |
| Class F | -0.62% | 0.92% | 0.55% | 105% | 100% |

Compared with NNVC-3.0 filter set 1 (NnlfOption = 2)

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.47% | 0.11% | 0.09% | 82% | 79% |
| Class A2 | -0.55% | -0.04% | 0.08% | 85% | 89% |
| Class B | -0.57% | -0.01% | 0.02% | 89% | 87% |
| Class C | -1.05% | 0.07% | 0.02% | 95% | 92% |
| Class E |  |  |  |  |  |
| Overall | -0.67% | 0.03% | 0.05% | 88% | 87% |
| Class D | -1.59% | 0.08% | 0.07% | 93% | 91% |
| Class F | -0.39% | 0.11% | 0.02% | 95% | 84% |

|  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.87% | 0.33% | 0.37% | 103% | 96% |
| Class C | -1.04% | 0.41% | 0.38% | 103% | 96% |
| Class E | -1.77% | -0.17% | 0.05% | 105% | 107% |
| Overall | -1.15% | 0.37% | 0.37% | 103% | 98% |
| Class D | -0.91% | 0.51% | 0.82% | 104% | 95% |
| Class F | -0.76% | 0.99% | 0.97% | 105% | 98% |

Compared with NNVC-3.0 anchor (NnlfOption = 0)

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -9.31% | -15.83% | -18.64% | 168% | 38646% |
| Class A2 | -10.73% | -20.28% | -17.36% | 162% | 37549% |
| Class B | -9.57% | -23.02% | -21.65% | 169% | 38890% |
| Class C | -10.85% | -21.70% | -22.28% | 151% | 33723% |
| Class E |  |  |  |  |  |
| Overall | -10.09% | -20.68% | -20.36% | 162% | 37131% |
| Class D | -12.87% | -23.57% | -23.88% | 146% | 34231% |
| Class F | -5.37% | -11.70% | -10.56% | 233% | 17731% |

|  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -8.62% | -15.69% | -13.48% | 186% | 32022% |
| Class C | -10.08% | -14.88% | -14.20% | 162% | 29318% |
| Class E | -10.41% | -17.33% | -17.24% | 316% | 28738% |
| Overall | -9.56% | -15.33% | -13.80% | 203% | 30264% |
| Class D | -11.64% | -19.05% | -17.73% | 157% | 27193% |
| Class F | -5.64% | -8.49% | -5.48% | 255% | 14682% |

Conclusion

In this contribution, a slice-level model switching method related to EE1-1.7 is proposed to further improve the coding efficiency of filter set #1. On top of EE1-1.7.1, the proposed method can provide a good performance for LDB. Considering the good trade-off, it is suggested to adopt this into NNVC filter set 1.

Reference

[1]	Y. Li, K. Zhang, L. Zhang, EE1-1.4: Deep In-Loop Filter with Additional Input Information, JVET-AB0073, Mainz DE, Oct 2022.

[2] E. Alshina, R. -L. Liao, S. Liu, A. Segall, Common test conditions and evaluation procedures for neural network-based video coding technology, JVET-AB2016, Mainz DE, Oct 2022.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).