| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 Jan 2023 | Document: JVET-AC0067-v1 |
| --- | --- |

| Title: | Non-EE2: Modifications on template-based multiple reference line intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Luhang Xu Yue Yu Haoping Yu Dong Wang | Tel: Email: | xuluhang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

# Abstract

This contribution proposes three modifications on template-based multiple reference line intra prediction. The first modification is to use neighboring 9 position PUs’ intra modes to fill the intra candidate list. The second modification is to use partitioning angles to fill the intra candidate list when neighboring PUs are coded by SGPM or GPM. The third modification is to extend angular precision from 65 to 129 in TMRL mode. The proposed modifications are implemented on top of ECM-7.0 software [1], and reportedly provide Y, U, V-BD rate reduction with encoder and decoder runtime as follows:

AI: -0.08%/-0.06%/-0.13%  EncT:99% DecT:100%

RA:

# Introduction

The TMRL [2] mode adopted into ECM-7.0 includes the following aspects:

- Extended reference line candidate list and intra-prediction-mode candidate list.

The extended reference line candidate list used in this method is {1, 3, 5, 7, 12}. The restriction on the top CTU row is unchanged. The size of the intra-prediction-mode candidate list used is 10. The construction of the intra-prediction-mode candidate list is similar to MPM. The differences are:

- PLANAR mode is excluded from the intra-prediction-mode candidate list.

- DC mode is added after the 5 neighboring PUs’ modes and DIMD modes if it has not been included.

- The angular modes with delta angles from ±1 to ±4 (compared to the existing angular modes in the intra-prediction-mode candidate list) are added.

- Construction of the TMRL candidate list.

There are 5 x10 = 50 combinations of the extended reference line and the allowed intra-prediction modes for a block. Since the extended reference line starts from reference line 1, the area covered by reference line 0 is used for template matching. The SAD costs over the template area (see Figure 1) are calculated between the predictions (generated by 50 combinations) and the reconstructions. The 20 combinations with the least SAD cost are selected in an ascending order to form the TMRL candidate list.

Figure 1. Illustration of the template area

- Signaling of TMRL

Instead of coding the reference line and the intra mode directly, an index to the TMRL candidate list is coded to indicate which combination of the reference line and prediction mode is used for coding the current block. In the TMRL mode, a truncated Golomb-Rice coding with a divisor 4 is employed to code selected combinations from the combination list. The binarization process and the codewords are shown in Table 1.

Table 1 the binarization process of the TMRL index

| Index | Bin string (prefix) | Bin string (suffix) |
| --- | --- | --- |
| 0 | 0 | 00 |
| 1 | 0 | 01 |
| 2 | 0 | 10 |
| 3 | 0 | 11 |
| 4 | 10 | 00 |
| … | … |  |
| 18 | 1111 | 10 |
| 19 | 1111 | 11 |

# Proposed method

Three modifications are proposed to the process of TMRL and MPM as follows,

- Use neighboring 9 position PUs’ intra modes to fill the intra candidate list.

- Use partitioning angles to fill the intra candidate list when neighboring PUs are coded by SGPM or GPM.

- Extend angular precision from 65 to 129 in TMRL prediction.

Figure-1. PUs at neighboring 9 positions

# Results

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.03% | -0.13% | 99% | 100% |
| Class A2 | -0.07% | -0.02% | -0.01% | 99% | 100% |
| Class B | -0.10% | -0.06% | -0.15% | 99% | 100% |
| Class C | -0.07% | -0.10% | -0.18% | 100% | 100% |
| Class E | -0.13% | -0.06% | -0.13% | 98% | 100% |
| Overall | -0.08% | -0.06% | -0.13% | 99% | 100% |
| Class D | -0.01% | -0.24% | -0.14% | 99% | 99% |
| Class F | -0.03% | 0.11% | 0.04% | 99% | 100% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

# References

- ECM-7.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-7.0.

- L. Xu, et. al., “EE2-1.12: Combination of EE2-1.10 and EE2-1.11,” JVET-AB0157, October 2022.

# Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).