| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0068-v1 |
| --- | --- |

| Title: | Non-EE2: multi-candidate IntraTMP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fan Wang Lai Zhang Yue Yu Haoping Yu Dong Wang | Email: | wangfan6@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution proposes a multi-candidate IntraTMP method. In the proposed method, a candidate list is constructed with the candidate BVs ranked in ascending order of their template matching costs, and the index of selected candidate is signaled in the bit-stream. On top of ECM-7.0, the performance of multi-candidate IntraTMP is reported as follow:

AI {-0.19% Y, -0.22% U, -0.22% V, 98% EncT, 101% DecT}

RA {xx% Y, xx% U, xx% V, xx% EncT, xx% DecT}

This contribution further proposes to enlarge the search range for smaller blocks. On top of ECM-7.0, the performance of multi-candidate IntraTMP with minimum search range 128 is reported as follow:

AI {-0.32% Y, -0.31% U, -0.40% V, 103% EncT, 106% DecT}

RA {xx% Y, xx% U, xx% V, xx% EncT, xx% DecT}

Introduction of IntraTMP in ECM-7.0

IntraTMP is an intra prediction tool in ECM [1]. If the block level IntraTMP flag is true, the decoder searches for the most similar template to the current template in a pre-defined reconstructed area of the current frame and uses the corresponding block as a prediction block. No other signaling is needed in block level. IntraTMP is enabled for both camera captured content and screen captured content in current CTC.

The search range of IntraTMP can reach the following 4 regions: (1) within the current CTU, (2) top-left CTU, (3) above CTU, (4) left CTU. Within these regions, the search range width and height are set proportional to the width and height of current block, respectively. That is:

SearchRangeWidth = a * BlkWidth

SearchRangeHight = a * BlkHeight

Where ‘a’ is set as 5 in ECM-7.0.

To reduce the complexity of the searching process, a sparse search is first performed in the search range with sub-sampling by a factor of 2. After finding the best match, a refinement search is performed around the best match with a reduced search range.

Proposed method

Multi-candidate IntraTMP

IntraTMP selects only one BV which has the smallest template matching cost. In camera captured content, the decoder can hardly find a perfect match. There are usually several blocks that are similar to current block and their template matching cost are comparable. The BV with the smallest template matching cost may not be the best predictor.

This contribution proposes to use multiple candidates for IntraTMP. A candidate list is constructed and the candidate BVs are ranked in ascending order of their template matching costs. An index is signaled in the bit-stream to indicate which candidate BV is actually used for current block. This method uses template matching to select a shortlist of promising candidates from a huge amount of possible BVs and allows the encoder, who can refer to the original current block, to make the final decision.

The syntax change is as follow:

…

intra_tmp_flag

if(intra_tmp_flag) {

intra_tmp_idx

}

…

Where intra_tmp_flag equals to 1 means that current block uses IntraTMP and intra_tmp_idx further indicates which BV in the candidate BV list will be used to identify the prediction block.

To build the candidate list, a sparse search and a refinement search are used. In the sparse search, the sub-sampling factor is 3 instead of 2 in ECM7.0 and the top 30 BVs are maintained. In the refinement search, each 3x3 block around each of the 30 BVs is checked and the top 15 BVs are selected to form the candidate list.

Enlarge the search range for smaller blocks.

A second aspect of the proposed method modifies the IntraTMP search range as follows:

SearchRangeWidth = max(a * BlkWidth, minSearchRange)

SearchRangeHight = max(a * BlkHeight, minSearchRange)

Where minSearchRange is set as 128.

Test results

The proposed methods were implemented on top of the ECM-7.0 software and tested based on the common test condition [2]. The test results are summarized below.

Test 1: proposed method 2.1

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.13% | -0.07% | 99% | 101% |
| Class A2 | -0.22% | -0.25% | -0.30% | 98% | 102% |
| Class B | -0.20% | -0.25% | -0.33% | 99% | 101% |
| Class C | -0.07% | -0.08% | -0.06% | 98% | 99% |
| Class E | -0.44% | -0.43% | -0.33% | 98% | 102% |
| Overall | -0.19% | -0.22% | -0.22% | 98% | 101% |
| Class D | -0.03% | -0.20% | -0.07% | 99% | 99% |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.22% | 0.08% | 100% | 100% |
| Class A2 |  |  |  |  |  |
| Class B | -0.09% | -0.14% | -0.51% | 100% | 100% |
| Class C | 0.00% | -0.04% | 0.22% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.03% | 0.09% | 0.11% | 100% | 100% |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |

Test 2: proposed method 2.1+2.2

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.07% | -0.03% | 101% | 101% |
| Class A2 | -0.32% | -0.36% | -0.40% | 102% | 106% |
| Class B | -0.43% | -0.39% | -0.52% | 104% | 110% |
| Class C | -0.11% | -0.06% | -0.26% | 104% | 106% |
| Class E | -0.68% | -0.69% | -0.77% | 105% | 107% |
| Overall | -0.32% | -0.31% | -0.40% | 103% | 106% |
| Class D | -0.07% | -0.27% | -0.12% | 102% | 104% |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.21% | -0.04% | 100% | 100% |
| Class A2 |  |  |  |  |  |
| Class B | -0.24% | -0.34% | -0.37% | 100% | 100% |
| Class C | -0.01% | 0.04% | -0.07% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.07% | 0.07% | 0.01% | 100% | 99% |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |

Conclusions

This contribution proposes a multi-candidate IntraTMP method. The proposed multi-candidate IntraTMP method brings -0.19% coding gain in AI and xx coding gain in RA with negligible EncT and DecT change. This contribution further proposes to enlarge the search range for smaller blocks. The coding gain of multi-candidate IntraTMP with the search range modification reaches -0.32% in AI and xx in RA with minor EncT and DecT increase. It is suggested to investigate the proposed method in EE2.

References

- ECM7.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-7.0

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, July, 2022.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).