| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0069-v1 |
| --- | --- |

| Title: | Non-EE2: Intra Template-Matching Prediction Fusion |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lai Zhang Fan Wang Yue Yu Haoping Yu Dong Wang | Tel: Email: | zhanglai@oppo.com wangfan6@oppo.com yue.yu@oppo.com v-yuhaoping@oppo.com wangdong7@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution proposes an Intra Template-Matching Prediction fusion method that blends multiple matched blocks derived from intra template matching process. The proposed method is implemented on top of ECM-7.0 software, it shows coding performance gains as follows:

Test a (IntraTMP fusion with SAD-based weights):

AI { -0.15% Y, -0.16% U, -0.18% V, 100% EncT, 100% DecT }

RA {xx% Y, xx% U, xx% V, xx% EncT, xx% DecT }

Test b (IntraTMP fusion with fixed weights):

AI { xx % Y, xx % U, xx % V, xx % EncT, xx % DecT }

RA {xx% Y, xx% U, xx% V, xx% EncT, xx% DecT }

Test c (IntraTMP fusion with SAD-based weights and enlarged search range):

AI {-0.28% Y, -0.33% U, -0.32% V, 104% EncT, 106% DecT }

RA {xx% Y, xx% U, xx% V, xx% EncT, xx% DecT }

Introduction

In ECM-7.0 [1], an optimized Intra Template-Matching Prediction (IntraTMP) mode was adopted. By subsampling the search range, the IntraTMP mode has lower complexity and may be enabled for both screen content and camera captured content with control over the complexity-gain trade-off.

The search range of IntraTMP is proportional to the block width and height. That is:

SearchRangeWidth = a * BlkWidth

SearchRangeHeight = a * BlkHeight

where ‘a’ is set 5 in ECM-7.0.

The search regions are sub-sampled by a factor of 2. After finding the best match, a refinement process is performed. The refinement is done via a second template matching search around the best match with a reduced range.

Proposed Method

This contribution proposes a method that derives multiple IntraTMP matched blocks and then fuses them to produce a better overall predictor. The proposed method is asserted to be beneficial for camera captured content. The proposed IntraTMP fusion algorithm includes the following aspects:

- Generate multiple matched blocks during the IntraTMP search.

During the subsampled IntraTMP search process, a candidate list is initially generated with 30 matched blocks having the smallest template SAD. For each matched block, a full pixel refinement search is performed within a small region. The sampling factor of subsampled search process is 3 and the refinement region is 3x3 around each of the 30 matched blocks. Then the best 3 candidate matched blocks by template SAD across all refinement regions are selected.

- Select candidate matched blocks used for fusion.

For each of the best 3 candidate matched blocks, a threshold is used to judge whether it should be used for fusion.

Threshold = SAD1 << 1

where SAD1 is the smallest template SAD of the three candidate matched blocks. Candidate matched blocks with SAD <= Threshold are used for fusion and therefore, the number of candidate matched blocks is determined.

- Calculate weights for each matched block used for fusion.

Once blocks to be fused are decided, they are fused with weights. Two methods of determining these fusion weights are tested in this contribution.

In the first method, fusion weights are calculated by their SAD. The weights are calculated as follows:

sum= i=1nSADi

wi= sum-SADin-1*sum

To reduce the implementation cost, the division operations are replaced by an integer look-up table (LUT).

In the second method, fixed weights are used to further reduce the complexity. The weights are set as {w1,w2}={12,12} or {w1,w2,w3}={2264,2164,2164}

- The final fused predictor is determined by

pfusion=i=1nwipi

where pi is the ith matched block, and n is the number of blocks selected for fusion. In a special case when only one matched block remains after step b), the final predictor is calculated as

pfusion=w1pTMP+w2pintra

where pTMP is the single matched block and pintra is the intra predictor derived by planar mode. In this special case, the weights are set as w1=7/8 and w2=1/8.

A CU level flag is added to signal whether an IntraTMP CU is predicted by the proposed fusion method or original method.

The proposed method with enlarged search range is also tested. The modified search range can be described as follows:

SearchRangeWidth = max(a * BlkWidth, minSearchRange)

SearchRangeHeight = max(a * BlkHeight, minSearchRange)

where the minSearchRange is set as 128 in the following test.

Simulation Results

The proposed method is implemented on top of ECM-7.0 [2]. Several tests are set as follows. These test results are obtained on top of ECM-7.0 under the common test conditions [3]:

Test a: sampling factor = 3, refinement region = 3x3, SAD-based weights

Test b: sampling factor = 3, refinement region = 3x3, fixed weights

Test c: sampling factor = 3, refinement region = 3x3, SAD-based weights, minSearchRange = 128

Table 1. proposed test-a on top of ECM-7.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.07% | -0.07% | 99% | 100% |
| Class A2 | -0.19% | -0.19% | -0.18% | 100% | 101% |
| Class B | -0.18% | -0.17% | -0.24% | 100% | 101% |
| Class C | -0.05% | -0.14% | -0.09% | 101% | 100% |
| Class E | -0.34% | -0.38% | -0.30% | 99% | 100% |
| Overall | -0.15% | -0.16% | -0.18% | 100% | 100% |
| Class D | 0.00% | -0.35% | -0.02% | 98% | 97% |
| Class F | -0.05% | 0.15% | 0.07% | 101% | 100% |
| Class TGM | -0.19% | -0.04% | -0.11% | 99% | 102% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | 0.01% | -0.18% | -0.04% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.02% | -0.46% | -0.31% | 99% | 100% |
| Class F | -0.10% | -0.03% | 0.00% | 98% | 100% |
| Class TGM | -0.07% | -0.13% | -0.01% | 98% | 100% |

Table 2. proposed test-b on top of ECM-7.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Table 3. proposed test-c on top of ECM-7.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.03% | -0.01% | 103% | 100% |
| Class A2 | -0.30% | -0.39% | -0.35% | 102% | 105% |
| Class B | -0.40% | -0.42% | -0.50% | 105% | 109% |
| Class C | -0.09% | -0.11% | -0.14% | 103% | 106% |
| Class E | -0.57% | -0.69% | -0.56% | 105% | 106% |
| Overall | -0.28% | -0.33% | -0.32% | 104% | 106% |
| Class D | -0.04% | -0.26% | -0.21% | 103% | 103% |
| Class F | -0.25% | -0.06% | -0.29% | 103% | 105% |
| Class TGM | -0.53% | -0.35% | -0.39% | 102% | 110% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | 0.00% | 0.11% | 0.09% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.02% | -0.03% | 0.04% | 100% | 100% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Conclusions

The proposed fusion method of IntraTMP mode can improve the coding efficiency of ECM with minor complexity increase. It is recommended to further study this method in an exploration experiment.

Reference

- M. Coban, et. al., “Algorithm description of Enhanced Compression Model 7 (ECM 7)”, JVET-AB2025, October, 2022.

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-7.0

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, July, 2022.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).