| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0070-v2 |
| --- | --- |

| Title: | Non-EE2: combination of JVET-AC0068 and JVET-AC0069 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fan Wang Lai Zhang Yue Yu Haoping Yu Dong Wang | Email: | wangfan6@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

In JVET-AC0068, a multi-candidate IntraTMP method is proposed. In JVET-AC0069, an IntraTMP fusion method is proposed. This contribution proposes a combination method of multi-candidate IntraTMP and IntraTMP fusion. On top of ECM7.0, the simulation results are reported as follow:

AI {-0.41% Y, -0.45% U, -0.47% V, 104% EncT, 107% DecT}

RA {xx% Y, xx% U, xx% V, xx% EncT, xx% DecT}

Introduction

In JVET-AC0068 [1], a multi-candidate IntraTMP method is proposed. In the proposed method, a candidate list is constructed with the candidate BVs ranked in ascending order of their template matching costs, and the index of selected candidate is signaled in the bit-stream.

In JVET-AC0069 [2], an IntraTMP fusion method is proposed. The proposed method derives multiple IntraTMP matched blocks and then fuses them to produce a better overall prediction.

Proposed method

The multi-candidate IntraTMP method introduces a block level syntax element, intra_tmp_idx, which indicates the index of the selected candidate. The IntraTMP fusion method introduces a block level syntax element, intra_tmp_fusion_flag, which indicates whether the intraTMP fusion method is used for current block. In the proposed combination method, the intra_tmp_fusion_flag is signalled before intra_tmp_idx. If intra_tmp_fusion_flag is false, the intra_tmp_idx flag is signalled then.

The syntax change is as follows:

…

intra_tmp_flag

if(intra_tmp_flag) {

intra_tmp _fusion_flag

if(!intra_tmp_fusion_flag) {

intra_tmp_idx

}

}

…

The searching process is reused for multi-candidate IntraTMP and IntraTMP fusion. The enlarged search range for smaller blocks is used and the minSearchRange is set as 128.

Test results

The proposed methods were implemented on top of the ECM-7.0[3] software and tested based on the common test condition [4]. The test results are summarized below.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.11% | -0.07% | 104% | 102% |
| Class A2 | -0.43% | -0.58% | -0.52% | 103% | 108% |
| Class B | -0.55% | -0.57% | -0.71% | 106% | 112% |
| Class C | -0.15% | -0.10% | -0.05% | 102% | 105% |
| Class E | -0.87% | -0.94% | -0.96% | 106% | 109% |
| Overall | -0.41% | -0.45% | -0.47% | 104% | 107% |
| Class D | -0.10% | -0.26% | -0.15% | 103% | 104% |
| Class F | -0.36% | -0.22% | -0.46% | 102% | 108% |
| Class TGM |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.28% | 0.03% | 102% | 101% |
| Class A2 |  |  |  |  |  |
| Class B | -0.29% | -0.25% | -0.64% | 101% | 100% |
| Class C | -0.04% | -0.03% | -0.06% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.07% | -0.11% | -0.13% | 101% | 100% |
| Class F | -0.30% | -0.42% | -0.44% | 100% | 100% |
| Class TGM |  |  |  |  |  |

Conclusions

This contribution proposes a combination method of multi-candidate IntraTMP and IntraTMP fusion. On top of ECM7.0, the coding gain is -0.41% in AI and xx in RA. It is suggested to investigate the combination method together with each single method in EE2.

References

- F. Wang, L. Zhang, Y. Yu, H. Yu, D. Wang, “Non-EE2: multi-candidate IntraTMP”, JVET-AC0068, Jan, 2023

- L. Zhang, F. Wang, Y. Yu, H. Yu, D. Wang, “Non-EE2: Intra Template-Matching Prediction Fusion”, JVET-AC0069, Jan, 2023

- ECM7.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-7.0

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, July, 2022.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).