| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0071-v1 |
| --- | --- |

| Title: | EE2-3.1: Direct block vector mode for chroma prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junyan Huo, Xue Hao, Yanzhuo Ma, Fuzheng Yang, Ming Li, Lai Zhang, Jing Ren | Email: | jyhuo@mail.xidian.edu.cn xhao@stu.xidian.edu.cn yzma@mail.xidian.edu.cn fzhyang@mail.xidian.edu.cn myron.li@oppo.com zhanglai@oppo.com renjing1@oppo.com |
| Source: | Xidian University, Guangdong OPPO Mobile Telecommunications Corp., Ltd |  |  |

_____________________________

- Abstract

This contribution reports the results of EE2 Test3.1 on a chroma intra prediction method for screen content, direct block vector (DBV). It is proposed to use BV of corresponding luma blocks to derive the prediction for chroma components when dual tree is activated in intra slice. It is reported that the coding performance of ECM 7.0 is as below:

- Test 3.1a: DBV of 5 blocks and signalling

For AI configuration:

F: -0.43%, -0.35%, and -0.60%, with 99 % EncT, 100% DecT.

TGM: -2.23%, -2.31%, and -2.93%, with 99 % EncT, 100% DecT.

For RA configuration:

F: -0.31%, -0.40%, and -0.45 %, with 98 % EncT, 100% DecT.

TGM: -0.66%, -0.71%, and -0.76%, with 98% EncT, 100% DecT.

- Test 3.1b: DBV of center block and no signalling

For AI configuration:

F: -0.38%, -0.25%, and -0.56%, with 98 % EncT, 100% DecT.

TGM: -2.22%, -2.24%, and -2.79%, with 98 % EncT, 99% DecT.

For RA configuration:

F: -0.28%, -0.34%, and -0.54 %, with 98 % EncT, 99% DecT.

TGM: -0.65%, -0.62%, and -0.69%, with 98% EncT, 99% DecT.

Introduction

In ECM-7.0[1], the intra prediction modes for chroma components include 6 cross component linear model (LM) modes, convolution cross component model (CCCM) mode, gradient linear model (GLM) mode, DIMD mode, direct mode (DM) , and four default intra prediction modes.

In the signalling of chroma intra modes, an intra_chroma_pred_mode is signaled to indicate the specific coding mode, as shown in Table 1.

Table 1 The binarization process for intra_chroma_pred_mode in ECM7.0

| intra_chroma_pred_mode | bin string | chroma intra mode |
| --- | --- | --- |
| 0 | 1100 | list[0] |
| 1 | 1101 | list[1] |
| 2 | 1110 | list[2] |
| 3 | 1111 | list[3] |
| 4 | 10 | DIMD chroma |
| 5 | 0 | DM |

ECM7.0 includes IBC mode to construct the prediction according to a block vector. IntraTMP mode is also enabled to derive BV according to the template.

Proposed Methods

This contribution proposes a method, namely direct block vector (DBV), to improve the coding efficiency for chroma components when dual tree is activated in intra slice. This method specifically includes two implementations.

Test 3.1a: DBV of 5 blocks and signalling

1. A bin is signalled to indicate whether the CU is coded as DBV mode or not, as listed in Table 2.

Table 2 The binarization process for intra_chroma_pred_mode in the proposed method

| intra_chroma_pred_mode | bin string | chroma intra mode |
| --- | --- | --- |
| 0 | 11100 | list[0] |
| 1 | 11101 | list[1] |
| 2 | 11110 | list[2] |
| 3 | 11111 | list[3] |
| 4 | 110 | DIMD chroma |
| 5 | 10 | DM |
| 6 | 0 | DBV |

2. When chroma dual tree is activated in intra slice, for a chroma CU coded as DBV mode, if one of the luma blocks in five locations (in Fig. 1) is coded with IBC mode or IntraTmp mode, its block vector bvL is used to derive chroma block vector bvC. The bv scaling process is determined according to template matching. If the luma block is coded with a RRIBC, a flip-aware BV adjustion is conducted for bvL, same as [2].

3. Then, as depicted in Fig. 2, by using the position of the current chroma block (xCb, yCb) and its bvC, the corresponding offset position (xCb+ bvC[0] , yCb + bvC[1]) is determined, and a block copy prediction is performed.

Figure 1 the five locations in reconstructed luma samples

Figure 2 the prediction process of DBV method

Test 3.1b: DBV of center block and no signalling

In this test, no additional bin is signalled. It works as follows:

1. When chroma dual tree is activated in intra slice, for a chroma CU coded as DM mode, if the center block in Fig. 1 is coded with IBC mode or IntraTmp mode, its block vector bvL is used to derive chroma block vector bvC. The bv scaling process is determined according to template matching. If the luma block is coded with a RRIBC, a flip-aware BV adjustion is conducted for bvL, same as [2].

2. Then, as depicted in Fig. 2, by using the position of the current chroma block (xCb, yCb) and its bvC, the corresponding offset position (xCb+ bvC[0] , yCb + bvC[1]) is determined, and a block copy prediction is performed.

Experimental results

The proposed methods are implemented based on ECM-7.0 under common test conditions[3]. Table 3 and Table 4 summarize the results with all intra and random access configurations.

Table 3. Simulation results of the Test 3.1a

|  |  |  | All Intra Main 10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.43% | -0.35% | -0.60% | 99% | 100% |
| Class TGM | -2.23% | -2.31% | -2.93% | 99% | 100% |

|  |  |  | Random Access Main 10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.31% | -0.40% | -0.45% | 98% | 100% |
| Class TGM | -0.66% | -0.71 % | -0.76 % | 98% | 100% |

Table 4. Simulation results of the Test 3.1b

|  |  |  | All Intra Main 10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.38% | -0.25% | -0.56% | 98% | 100% |
| Class TGM | -2.22% | -2.24% | -2.79% | 98% | 99% |

|  |  |  | Random Access Main 10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.28% | -0.34% | -0.54% | 98% | 99% |
| Class TGM | -0.65% | -0.62 % | -0.69 % | 98% | 99% |

Conclusions

This contribution reports test results of the EE2 3.1 which uses block vector of luma for chroma block copy prediction. The experimental results reportedly show that the proposed method achieves coding performance for screen content under AI and RA cases with slight complexity reduction. Therefore, it is recommended to include the proposed method into the next version of ECM.

- 5 References

- ECM-7.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

- Zhipin Deng, Kai Zhang, Li Zhang, “EE2-3.2: Reconstruction-Reordered IBC for screen content coding,” JVET-AA0070, July 2022.

- M. Karczewicz, et. al., “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, February 2022.

- 6 Patent rights declaration(s)

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).