| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0072-v1 |
| --- | --- |

| Title: | EE2-3.2: Block vector difference sign prediction or IBC blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junyan Huo, Zhenyao Zhang, Xue Hao, Yanzhuo Ma, Fuzheng Yang, Ming Li, Lai Zhang, Jing Ren | Email: | jyhuo@mail.xidian.edu.cn zhyzhang@stu.xidian.edu.cn xhao@stu.xidian.edu.cn yzma@mail.xidian.edu.cn fzhyang@mail.xidian.edu.cn myron.li@oppo.com zhanglai@oppo.com renjing1@oppo.com |
| Source: | Xidian University, Guangdong OPPO Mobile Telecommunications Corp., Ltd |  |  |

_____________________________

- Abstract

This contribution reports the results of EE2 Test3.2 on the block vector difference sign prediction (BVDSP) and its combination with Test 3.1. It is proposed to improve coding efficiency, where possible BVD signs combinations are sorted according to template matching cost and index of true BVD signs is coded with context model. It is reported that the coding performance to ECM 7.0 is as below:

- Test 3.2a: (BVDSP)

For AI configuration:		F:-0.20%, -0.06%, and -0.04%, with 99 % EncT, 101 % DecT.

TGM:-0.40%, -0.39%, and -0.35%, with 99 % EncT, 101 % DecT.

For RA configuration:	F: -0.09 %, -0.04%, and -0.17 %, with 100 % EncT, 99 % DecT.

TGM: -0.18 %, -0.19 %, and -0.08 %, with 100 % EncT, 99 % DecT.

- Test 3.2b: (Test 3.2a +Test3.1a)

For AI configuration: 	F: -0.58%, -0.54%, and -0.63%, with 99 % EncT, 99 % DecT.

TGM: -2.65%, -2.71%, and -3.32%, with 99 % EncT, 100 % DecT.

For RA configuration: 	F: -0.37%, -0.46%, and -0.54 %, with 99 % EncT, 99 % DecT.

TGM: -0.92%, -0.95%, and -1.03%, with 100 % EncT, 100 % DecT.

- Test 3.2c: (Test 3.2a +Test3.1b)

For AI configuration: 	F: -0.57%, -0.39%, and -0.79%, with 98 % EncT, 100 % DecT.

TGM: -2.64%, -2.63%, and -3.25%, with 98 % EncT, 101 % DecT.

For RA configuration: 	F: -0.36%, -0.45%, and -0.61 %, with 98 % EncT, 100 % DecT.

TGM: -0.91%, -0.92%, and -1.02%, with 98 % EncT, 100 % DecT.

Introduction

In ECM7.0[1], the BVD signs are directly coded in EP mode.

The MVD sign prediction indices representing the correct MVD signs are encoded with the context model. At decoder side, the MVD signs are derived as follows [2]:

- Parse the absolute values of MVD components

- Parse context-coded MVD sign prediction index

- Build MV candidates by creating combinations between possible signs and the absolute MVD value and add them to the MV predictor

- Derive MVD sign prediction cost for each derived MV based on template matching cost and sort

- Use MVD sign prediction index to pick the true MVD sign

- Add the true MVD to the MV predictor for final MV

The transition from EP coding to context-based coding provides gains.

Proposed Methods

Test3.2a Block Vector Difference Sign Prediction (BVDSP) for IBC blocks

Block vector difference sign prediction (BVDSP) can be applied for IBC blocks when the block vector difference contains non-zero component. Possible BVD sign combinations are sorted according to template matching cost and index corresponding to the true BVD sign is derived and coded with context model. At decoder side, the BVD signs are derived as following major steps:

- Step1: 4 candidate BVs are derived by BVP and combinations between possible signs and absolute BVD

( BVP[0] + absBVD[0], BVP[1] + absBVD[1] )

( BVP[0] + absBVD[0], BVP[1] - absBVD[1] )

( BVP[0] - absBVD[0], BVP[1] + absBVD[1] )

( BVP[0] - absBVD[0], BVP[1] - absBVD[1] )

- Step2: Derive prediction costs of 4 candidate BVs based on template matching and sort them         according to the costs

- Step3: Use BVDSP index to pick the true BVD sign

- Step4: Add the true BVD to the BVP to get the final BV

- The template matching cost is measured by the SAD between the neighbouring samples of the current CU and their corresponding reference samples, as illustrated in Figure 1.

Figure 1. Template matching cost derivation in BVDSP

- The Step3 of deriving the true BVD signs from BVDSP index is same as [2]. This test codes the index with a single ctxTable different from [2] in the initialization process. In [2], two ctxTables are employed which select the ctxIdx based on the magnitude of the BVD.

- In general case 4 BVD candidates are evaluated (+/– bvd_abs.x, +/– bvd_abs.y). If one of BVD components (x or y) is zero, only 2 BVD candidates are possible. Case when all BVD component are zero is trivial and no sign derivation is needed.

Test3.2b : Test3.2a + Test3.1a

Test 3.1a:

Test3.1 proposes a method, namely direct block vector (DBV), to improve the coding efficiency for chroma components when dual tree is activated in intra slice. Test3.1a is one implementation of DBV.

1. A bin is signalled to indicate whether the CU is coded as DBV mode or not, as listed in Table 1.

Table 1 The binarization process for intra_chroma_pred_mode in the proposed method

| intra_chroma_pred_mode | bin string | chroma intra mode |
| --- | --- | --- |
| 0 | 11100 | list[0] |
| 1 | 11101 | list[1] |
| 2 | 11110 | list[2] |
| 3 | 11111 | list[3] |
| 4 | 110 | DIMD chroma |
| 5 | 10 | DM |
| 6 | 0 | DBV |

2. When chroma dual tree is activated in intra slice, for a chroma CU coded as DBV mode, if one of the luma blocks in five locations (in Fig. 2) is coded with IBC mode or IntraTmp mode, its block vector bvL is used to derive chroma block vector bvC. The bv scaling process is determined according to template matching. If the luma block is coded with a RRIBC, a flip-aware BV adjustion is conducted for bvL, same as [3].

3. Then, by using the position of the current chroma block (xCb, yCb) and its bvC, the corresponding offset position (xCb+ bvC[0] , yCb + bvC[1]) is determined, and a block copy prediction is performed.

Figure 2 the five locations in reconstructed luma samples

Test3.2c : Test3.2a + Test3.1b

Test 3.1b:

Test3.1b proposes another implementation of direct block vector (DBV).

In this test, no additional bin is signalled. It works as follows:

1. When chroma dual tree is activated in intra slice, for a chroma CU coded as DM mode, if the center block in Fig. 2 is coded with IBC mode or IntraTmp mode, its block vector bvL is used to derive chroma block vector bvC. The bv scaling process is determined according to template matching. If the luma block is coded with a RRIBC, a flip-aware BV adjustion is conducted for bvL, same as [3].

2. Then, by using the position of the current chroma block (xCb, yCb) and its bvC, the corresponding offset position (xCb+ bvC[0] , yCb + bvC[1]) is determined, and a block copy prediction is performed.

Experimental results

The proposed method was implemented based on ECM-7.0 under common test conditions [4]. Table 2, Table 3 and Table 4 summarize the results with all intra and random access configurations.

Table 2. Simulation results of Test3.2a

|  |  |  | All Intra Main 10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.20% | -0.06% | -0.04% | 99% | 101% |
| Class TGM | -0.40% | -0.39% | -0.35% | 99% | 101% |

|  |  |  | Random Access Main 10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.09% | -0.04% | -0.17% | 100% | 99% |
| Class TGM | -0.18% | -0.19% | -0.08% | 100% | 99% |

Table 3. Simulation results of Test3.2b

|  |  |  | All Intra Main 10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.58% | -0.54% | -0.63% | 99% | 99% |
| Class TGM | -2.65% | -2.71% | -3.32% | 99% | 100% |

|  |  |  | Random Access Main 10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.37% | -0.46% | -0.54% | 99% | 99% |
| Class TGM | -0.92% | -0.95% | -1.03% | 100% | 100% |

Table 4. Simulation results of Test3.2c

|  |  |  | All Intra Main 10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.57% | -0.39% | -0.79% | 98% | 100% |
| Class TGM | -2.64% | -2.63% | -3.25% | 98% | 101% |

|  |  |  | Random Access Main 10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.36% | -0.45% | -0.61% | 98% | 100% |
| Class TGM | -0.91% | -0.92% | -1.02% | 98% | 100% |

Conclusions

This contribution reports test results of the EE2 3.2. Instead of direct signaling of BVD signs in EP mode, BVDSP mode is proposed where possible BVD signs combinations are sorted according to template matching cost and index of true BVD signs is coded with context model. The experimental results reportedly show that the proposed method achieves coding performance under AI and RA cases with negligible complexity impact. Therefore, it is recommended to include the proposed method into the next version of ECM.

- 5 References

- ECM-7.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

- Yan Zhang, Bappaditya Ray, “Non-EE2: On MVD sign prediction”, JVET-X0132, October 2021.

- Zhipin Deng, Kai Zhang, Li Zhang, “EE2-3.2: Reconstruction-Reordered IBC for screen content coding,” JVET-AA0070, July 2022.

- M. Karczewicz, et. al., “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, February 2022.

- 6 Patent rights declaration(s)

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).