| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0074-v2-VSEI-OptionB |
| --- | --- |

| Title: | AHG9: On the VVC use of the NNPFC SEI message for picture rate upsampling (VSEI specification text for option B) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Technologies Tampere, Finland | Email: | miska.hannuksela@nokia.com |
| Source: | Nokia |  |  |

_____________________________

    - Neural-network post-filter characteristics SEI message semantics

The neural-network post-filter characteristics (NNPFC) SEI message specifies a neural network that may be used as a post-processing filter. The use of specified post-processing filters for specific pictures is indicated with neural-network post-filter activation SEI messages.

Use of this SEI message requires the definition of the following variable:

–	When nnpfc_auxiliary_inp_idc is equal to 1, a filtering strength control value StrengthControlVal that shall be a real number in the range of 0 to 1, inclusive.

Input pictures for the post-processing filter are derived for each value of i in the range of 0 to numInputPics − 1, inclusive, as follows:

–	If i is equal to 0, the source picture with index 0 is the cropped decoded output picture for which the post-processing filter is activated.

–	Otherwise (i is greater than 0), the following applies:

–	If nnpfc_purpose is equal to 5 and there is a first frame packing arrangement SEI message with fp_arrangement_type equal to 5 present for the source picture with index 0, the source picture with index i is the cropped decoded output picture that precedes, in output order, the source picture with index i − 1 and has a frame packing arrangement SEI message with fp_arrangement_type equal to 5 and fp_current_frame_is_frame0_flag equal to the value of the fp_current_frame_is_frame0_flag in the first frame packing arrangement SEI message.

–	Otherwise (nnpfc_purpose is not equal to 5 or no frame packing arrangement SEI message with fp_arrangement_type equal to 5 is present for the source picture with index 0), the source picture with index i is the cropped decoded output picture that precedes, in output order, the source picture with index i − 1.

–	If nnpfc_purpose is equal to 5, CroppedWidth or CroppedHeight is not respectively equal to the width or height of the source picture with index i greater than 0, and there is a second post-processing filter that has nnpfc_purpose equal to 3 and is activated by an NNPFA SEI message for the source picture with index i, the input picture with index i is the output of the neural-network inference of the second post-processing filter applied to the source picture with index i.

–	Otherwise (nnpfc_purpose is not equal to 5, or CroppedWidth and CroppedHeight are respectively equal to the width are height of the source picture with index i greater than 0, or there is no second post-processing filter that has nnpfc_purpose equal to 3 and is activated by an NNPFA SEI message for the source picture with index i), the input picture with index i is the source picture with index i.

Let luma sample array CroppedYPic[ idx ] and chroma sample arrays CroppedCbPic[ idx ] and CroppedCrPic[ idx ], when present, be the respective sample arrays of the input pictures with index idx in the range of 0 to numInputPics − 1, inclusive.

Input picture width and height in units of luma samples, denoted herein by CroppedWidth and CroppedHeight, respectively, are derived as follows:

–	If nnpfc_purpose is equal to 5 and there is a second post-processing filter that has nnpfc_purpose equal to 3 and is activated by an NNPFA SEI message for the source picture with index 0, the following applies:

–	CroppedWidth is equal to nnpfc_pic_width_in_luma_samples defined for the second post-processing filter.

–	CroppedHeight is equal to nnpfc_pic_height_in_luma_samples defined for the second post-processing filter.

–	Otherwise (nnpfc_purpose is not equal to 5 or there is no second post-processing filter that has nnpfc_purpose equal to 3 and is activated by an NNPFA SEI message for the source picture with index 0), the following applies:

–	CroppedWidth is equal to the width, in luma samples, of the source picture with index 0.

–	CroppedHeight is equal to the height, in luma samples, of the source picture with index 0.

The following constraints apply for each input picture with index i in the range of 1 to numInputPics − 1, inclusive:

–	If the width, in luma samples, of the source picture with index i is not equal to CroppedWidth, or the height, in luma samples, of the source picture with index i is not equal to CroppedHeight, nnpfc_purpose shall be equal to 5 and there shall be a second post-processing filter that has nnpfc_purpose equal to 3, is activated by an NNPFA SEI message for the source picture with index i, has nnpfc_pic_width_in_luma_samples equal to CroppedWidth and has nnpfc_pic_height_in_luma_samples equal to CroppedHeight.

Let BitDepthY be the bit depth for the luma sample array of the input pictures.

Let BitDepthC be the bit depth for the chroma sample arrays, if any, of the input pictures.

Let ChromaFormatIdc be the chroma format indicator for the input pictures as described in subclause ‎7.3.

The variables SubWidthC and SubHeightC are derived from ChromaFormatIdc as specified by Table 2.

[Ed. (MH): Global variables CroppedYPic[ idx ], CroppedCbPic[ idx ], CroppedCrPic[ idx ], CroppedWidth, CroppedHeight, BitDepthY, BitDepthC, ChromaFormatIdc, SubWidthC and SubHeightC should be changed to respective local variables croppedYPic[ idx ], etc.]