| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0074-v2 |
| --- | --- |

| Title: | AHG9: On the VVC use of the NNPFC SEI message for picture rate upsampling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Technologies Tampere, Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The picture rate upsampling purpose was added to the neural-network post-filter characteristics (NNPFC) SEI message in the JVET-AB meeting. A picture rate upsampling post-filter uses two or more input pictures to generate one or more interpolated pictures.

The contribution responds to the following asserted issues:

- Missing VVC specification text for inputting multiple input pictures for a picture rate upsampling post-filter.

- The following question that was left for further study in the JVET-AB meeting: Does the selection of the input pictures for a picture rate upsampling post-filter depend on TemporalId values?

- The use of temporal interleaving frame packing arrangement with a frame rate upsampling post-filter.

- The use of a picture rate upsampling post-filter when picture widths and/or heights of input pictures for the post-filter vary.

This contribution proposes VVC specification text for inputting multiple input pictures for a picture rate upsampling post-filter and has the following main ingredients:

- The input sample arrays CroppedYPic[ idx ], CroppedCbPic[ idx ] and CroppedCrPic[ idx ] for increasing order of idx are set in decreasing order of POC regardless of the TemporalId values, where idx equal to 0 corresponds to the current picture where the post-filter is activated.

- When a frame rate upsampling post-filter is applied with temporal interleaving frame packing arrangement, all the input pictures are selected to be either first constituent frames or second constituent frames.

- Either of the following is proposed:

  - Add a constraint that frame rate upsampling can only be used when all input pictures have the same dimensions.

  - Add a constraint that frame rate upsampling can only be used when all input pictures have the same dimensions after applying super resolution post-filter applicable to the input picture, if any.

Version 2 of the contribution relates to the following offline text drafting revisits of the BoG report JVET-AC0324-v1:

- Item 14 (clarification of existing intent): Adopt in spirit to clarify which pictures are input for picture rate upsampling. Revisit after offline text drafting.

- Item 16 (BF/cleanup): Adopt in spirit to clarify that when picture rate upsampling is applied together with the frame packing arrangement SEI message with fp_arrangement_type equal to 5 (temporal interleaving), the pictures are the pictures from the same view. Revisit after offline text drafting.

- Item 17 (sensibility constraint): Add a constraint that frame rate upsampling can only be used when all input pictures have the same cropped dimensions after applying the super resolution post-filter applicable to the input picture, if any. Revisit after offline text drafting.

Version 2 of the contribution proposes two options to implement the in-spirit agreement of the BoG:

- Option A: Addition of frame rate upsampling related constraints for the input variables in the VSEI text, and keeping the text proposed VVC text of JVET-AC0074-v1.

- Option B: Changing the "interface" variables CroppedYPic[ idx ], CroppedCbPic[ idx ], CroppedCrPic[ idx ], CroppedWidth, CroppedHeight, BitDepthY, BitDepthC, and ChromaFormatIdc to be local variables in the NNPFC SEI message and deriving the variables in the VSEI text.

The author prefers option A, because "interface" variables in the similar nature as currently for NNPFC SEI message have been used for other SEI messages too. Option B is therefore asserted to be a change in specification text writing principles.

# Problem Statement

## On missing text on multiple input pictures

As an outcome of JVET-AB0058, VSEI 2nd Ed. DAM 1 supports inputting multiple input pictures for the post-processing filter for frame rate upsampling using variables CroppedYPic[ idx ], CroppedCbPic[ idx ] and CroppedCrPic[ idx ]. However, the support for multiple input pictures has not been enabled in VVC 2nd Ed. DAM 1 in the subclause specifying the use of the neural-network post-filter characteristics (NNPFC) SEI message.

It is suggested to add support for inputting multiple input pictures for the post-processing filter using variables CroppedYPic[ idx ], CroppedCbPic[ idx ] and CroppedCrPic[ idx ] in the VVC subclause specifying the use of NNPFC SEI message.

## On TemporalId-based selection of input pictures

The following excerpt from the JVET meeting notes on JVET-AB0058 is relevant for the use of NNPFC SEI message for frame rate upsampling:

It was further commented that in terms of timing it would be required to associate the activation SEI with the last input picture, and the other input pictures are the corresponding preceding images in output order with temporal ID less than or equal to the TID of the last input picture. Decision: It was agreed to add this as purpose 5, adopt JVET-AB0058 with the changes suggested. It was further suggested that it might be considered to be used in temporal sublayers as well (further study was recommended on that aspect, and if that is relevant at all).

We studied whether and how TemporalId should impact the selection of input pictures to a frame rate upsampling with an example bitstream presented in Figure 1. The frame rate of this example is assumed to be 30 Hz. The figure contains some pictures in output order running from left to right. The pictures enclose their TemporalId values, and some potential prediction dependencies are indicated by prediction arrows.

Figure 1 – Example bitstream with TemporalId values and output order running from left to right.

Let us assume that there is an NNPFC SEI message that specifies a frame rate upsampling filter that generates an intermediate picture between the pair of input pictures. When that frame rate upsampling filter is used for each pair of frames, the frame rate is doubled to 60 Hz as illustrated in Figure 2.

Figure 2 – Frame rate upsampling to double the frame rate of the example bitstream.

To double the frame rate, this example bitstream contains a NNPFA SEI message in each picture unit. Specifically, when an NNPFA SEI message is carried in a picture unit with TemporalId 0 or 1, the other input picture for the filter is the preceding picture, in output order, with TemporalId equal to 2. It is therefore concluded that the input pictures to the frame rate upsampling post-filter should not be limited as suggested in the JVET-AB meeting notes ("the other input pictures are the corresponding preceding images in output order with temporal ID less than or equal to the TID of the last input picture").

## On temporal interleaving frame packing arrangement

The frame packing arrangement SEI message indicates, when fp_arrangement_type is equal to 5, that first and second constituent frames (usually, the left- and right-view pictures of the same time) are temporally interleaved. When a frame rate upsampling post-filter is applied with temporal frame packing arrangement, all the input pictures should be either first constituent frames or second constituent frames, since otherwise the output of the post-filter might be unpredictable.

## On pictures of varying picture width or height in input pictures

In VVC, the width and height of pictures can vary within a CLVS. However, the NNPFC SEI message semantics input only one pair of CroppedWidth and CroppedHeight values. Any of the following approaches should therefore be taken:

- Allow inputting picture-wise CroppedWidth and CroppedHeight. It is argued that it would require substantial changes in the semantics of the NNPFC SEI message semantics for constructing the input tensors of the post-filter.

- Add a constraint that frame rate upsampling can only be used when all input pictures have the same dimensions.

- Add a constraint that frame rate upsampling can only be used when all input pictures have the same dimensions after applying super resolution post-filter applicable to the input picture, if any.

# Overview of the proposal

This contribution has the following main ingredients:

- The input sample arrays CroppedYPic[ idx ], CroppedCbPic[ idx ] and CroppedCrPic[ idx ] for increasing order of idx are set in decreasing order of POC regardless of the TemporalId values, where idx equal to 0 corresponds to the current picture where the post-filter is activated.

- When a frame rate upsampling post-filter is applied with temporal interleaving frame packing arrangement, all the input pictures are selected to be either first constituent frames or second constituent frames.

- Either of the following is proposed:

  - Add a constraint that frame rate upsampling can only be used when all input pictures have the same dimensions. (In response to the in-spirit agreement in the BoG as documented in JVET-AC0324v1, the specification text for this option is no longer present in version 2 of this contribution.)

  - Add a constraint that frame rate upsampling can only be used when all input pictures have the same dimensions after applying super resolution post-filter applicable to the input picture, if any.

# Proposed specification text

The text that implements all ingredients of this contribution is presented below. Changed text for both options is indicated with gray highlight. For clarity, deleted text is not shown, the entire subclause is reproduced, and text that has not been changed is indicated without highlights.

The respective VSEI text for options A and B are included in the accompanying documents included in the zip package.

## Option A

### D.12.11	Use of the neural network post-filter characteristics SEI message

Let currCodedPic be the coded picture in which the post-processing filter defined by the NNPFC SEI message is activated by an NNPFA SEI message.

The variables currPic, specifying the picture to which the post-processing filter is applied, currWidth, specifying the width of the picture to which the post-processing filter is applied in luma samples, and currHeight, specifying the height of the picture to which the post-processing filter is applied in lumas samples, are derived as follows:

–	If nnpfc_purpose is equal to 5 and there is a second post-processing filter that is defined by at least one NNPFC SEI message, is activated by an NNPFA SEI message for currCodedPic, and has nnpfc_purpose equal to 3, the following applies:

–	currPic is set to be the output of the neural-network inference of the second post-processing filter with the cropped decoded output picture corresponding to currCodedPic as an input.

–	currWidth is set equal to nnpfc_pic_width_in_luma_samples.

–	currHeight is set equal to nnpfc_pic_height_in_luma_samples.

–	Otherwise, the following applies:

–	currPic is the cropped decoded output picture corresponding to currCodedPic.

–	currWidth is set equal to pps_pic_width_in_luma_samples − SubWidthC * ‌ ( pps_conf_win_left_offset + pps_conf_win_right_offset ) applying to currCodedPic..

–	currHeight is set equal to pps_pic_height_in_luma_samples − SubHeightC * ‌ ( pps_conf_win_top_offset + pps_conf_win_bottom_offset ) applying to currCodedPic.

When nnpfc_purpose is equal to 5, the variables numInputPics, specifying the number of input pictures for the post-processing filter, and the array inputPicPoc[ i ] for all values of i in the range of 0 to numInputPics − 1, inclusive, specifying the picture order count values for the input pictures for the post-processing filter, are derived as follows:

–	The variable numInputPics is set equal to nnpfc_num_input_pics_minus2 + 2.

–	The variable inputPicPoc[ i ] for all values of i in the range of 0 to numInputPics − 1, inclusive, is derived as follows:

–	inputPicPoc[ 0 ] is set equal to PicOrderCntVal of currCodedPic.

–	The following applies for each value of i in the range of 1 to numInputPics − 1, inclusive, in increasing order of i:

–	If currPic contains a constituent frame X (X being either 0 or 1) in temporal interleaving frame packing arrangement, inputPicPoc[ i ] is set equal to PicOrderCntVal of the picture that precedes, in output order, the picture associated with index i – 1 and contains a constituent frame X.

–	Otherwise (currPic does not contain a constituent frame in temporal interleaving frame packing arrangement), inputPicPoc[ i ] is set equal to PicOrderCntVal of the picture that precedes, in output order, the picture associated with index i – 1.

For purposes of interpretation of the NNPFC SEI message, the following variables are specified:

–	CroppedWidth is set equal to currWidth.

–	CroppedHeight is set equal to currHeight.

–	The luma sample array CroppedYPic[ 0 ] and the chroma sample arrays CroppedCbPic[ 0 ] and CroppedCrPic[ 0 ], when present, are set to be the 2-dimensional arrays of decoded sample values of the Y, Cb and Cr components, respectively, of currPic.

–	When nnpfc_purpose is equal to 5, the luma sample arrays CroppedYPic[ i ] and the chroma sample arrays CroppedCbPic[ i ] and CroppedCrPic[ i ], when present, are derived as follows for each value of i in the range of 1 to numInputPics − 1, inclusive:

–	Let sourcePic be the cropped decoded output picture that has PicOrderCntVal equal to inputPicPoc[ i ] in the CLVS containing currCodedPic.

–	The variable sourceWidth is set equal to pps_pic_width_in_luma_samples − ‌SubWidthC * ( pps_conf_win_left_offset + pps_conf_win_right_offset ) applying to sourcePic.

–	The variable sourceHeight is set equal to pps_pic_height_in_luma_samples − ‌SubHeightC * ( pps_conf_win_top_offset + pps_conf_win_bottom_offset ) applying to sourcePic.

–	If sourceWidth is equal to CroppedWidth and sourceHeight is equal to CroppedHeight, inputPic is set to be the same as sourcePic.

–	Otherwise (sourceWidth is not equal to CroppedWidth or sourceHeight is not equal to CroppedHeight), the following applies:

–	There shall be a post-processing filter, hereafter referred to as the super resolution filter, that is defined by at least one NNPFC SEI message, is activated by an NNPFA SEI message for sourcePic, and has nnpfc_purpose equal to 3, nnpfc_pic_width_in_luma_samples equal to CroppedWidth and nnpfc_pic_height_in_luma_samples equal to CroppedHeight.

–	inputPic is set to be the output of the neural-network inference of the super resolution filter with sourcePic being an input.

–	The luma array CroppedYPic[ i ] and the chroma sample arrays CroppedCbPic[ i ] and CroppedCrPic[ i ], when present, are set to be the 2-dimensional arrays of decoded sample values of the Y, Cb and Cr components, respectively, of inputPic.

–	BitDepthY and BitDepthC are both set equal to BitDepth.

–	ChromaFormatIdc is set equal to sps_chroma_format_idc.

–	StrengthControlVal is set equal to the value of SliceQpY ÷ 63 of the first slice of currCodedPic.

There shall not be more than two neural-network post-filter characteristics SEI messages present in a picture unit with the same value of nnpfc_id. When there are two neural-network post-filter characteristics SEI messages present in a picture unit with the same value of nnpfc_id, these SEI messages shall have different content. When two neural-network post-filter characteristics SEI messages with the same nnpfc_id and different content are present in the same picture unit, both of these neural-network post-filter characteristics SEI messages shall be in the same SEI NAL unit.

## Option B: Remove most of the input variables

### D.12.11	Use of the neural network post-filter characteristics SEI message

Let currCodedPic be the coded picture in which the post-processing filter defined by the NNPFC SEI message is activated by an NNPFA SEI message.

For purposes of interpretation of the NNPFC SEI message, the following variables are specified:

–	StrengthControlVal is set equal to the value of SliceQpY ÷ 63 of the first slice of currCodedPic.

There shall not be more than two neural-network post-filter characteristics SEI messages present in a picture unit with the same value of nnpfc_id. When there are two neural-network post-filter characteristics SEI messages present in a picture unit with the same value of nnpfc_id, these SEI messages shall have different content. When two neural-network post-filter characteristics SEI messages with the same nnpfc_id and different content are present in the same picture unit, both of these neural-network post-filter characteristics SEI messages shall be in the same SEI NAL unit.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).