| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0076-v1 |
| --- | --- |

| Title: | AHG9/AHG15: On the NNPFC SEI message for machine analysis |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela, Francesco Cricri Jukka I. Ahonen Honglei Zhang Nokia Technologies Tampere, Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The contribution reviews some earlier works on neural-network post-filtering targeted at improving machine analysis precision and concludes, based on the earlier works, that post-filters targeted at machine analysis can have different purposes (i.e., nnpfc_purpose values) in the neural-network post-filter characteristics (NNPFC) SEI message.

The contribution proposes to add the target usage of a post-processing filter in the NNPFC SEI message to indicate whether the filtered video is suitable for any usage, is intended for user viewing, or is expected to be provided as input to machine analysis.

The proposed target usage is intended to be used for selecting which post-filters are applied as follows:

- When a decoding device displays the video for user viewing rather than performs machine analysis, any post-processing filter that has only machine analysis as the target usage is suggested to be omitted.

- When a decoding device performs machine analysis rather than displays the video, any post-processing filter that has only user viewing as the target usage is suggested to be omitted.

# Introduction

Some neural-network post-processing filters may have been trained to improve machine analysis precision. Such filters may cause blurring of the background, changing of the colour mapping, and overemphasizing object edges, for instance.

Our work [1] studied neural-network post-filters aimed at improving machine analysis precision. We trained a task-agnostic enhancement (TAE) post-filter using a loss function combining both mean squared error (MSE) and a generic machine task loss. The trained post-filter was applied on images decoded by VVC. The TAE-filtered images were found superior in object detection and instance segmentation performance when compared to both the decoded images as such and the post-filtered images resulting from a post-filter trained with MSE only. Figure 1 reproduces an example from [1], where a decoded picture appears on top-left, the respective TAE-filtered picture on the top-right. It can be observed that the background is flatter and the colour mapping of the dogs appears smoother in the TAE-filtered video.

Figure 1. Examples of a decoded picture (top-left), the respective post-filtered picture (top-right), and their instance segmentation results.

JVET-AB0274 [2] includes another example of neural-network post-filters trained to improve machine analysis precision. JVET-AB0274 presents a VVC-based system targeted at machine analysis where video is pre-processed by downsampling prior to VVC-compliant encoding. Upsampling is performed as post-processing subsequent to VVC decoding. JVET-AB0274 uses several alternative pairs of neural networks for upsampling and downsampling, where some of the pre- and post-processing filters are jointly trained using a machine analysis loss.

The examples above demonstrate that a post-filter targeted at machine analysis can have different nnpfc_purpose values in the NNPFC SEI message: nnpfc_purpose would be set equal to 1 (visual quality improvement) for the TAE post-filter of [1] and equal to 3 (upsampling without changing the chroma format) in [2]. However, the currently adopted NNPFC SEI message does not specify whether the post-filter is targeted, for example, to displaying or to machine analysis.

# Overview of the proposal

This contribution defines the target usage of a post-processing filter to indicate whether the filtered video is suitable for any usage, is intended for user viewing, or is expected to be provided as input to machine analysis. It is proposed to add the target usage syntax element in the neural-network post-filter characteristics (NNPFC) SEI message.

The target usage is intended to be used for post-filter selection as follows: When the decoding device either displays the video or uses the video as input machine analysis opposite to what is indicated in the target usage of an NNPFC SEI message, the decoding device should omit the post-processing filter defined by the NNPFC SEI message. Consequently, the target usage indication can be used to avoid post-filtering that would likely make the filtered video worse than the unfiltered video for the applied usage.

# Proposed specification text

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_id | ue(v) |
| nnpfc_mode_idc | ue(v) |
| if( nnpfc_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_reserved_zero_bit_a | u(1) |
| nnpfc_tag_uri | st(v) |
| nnpfc_uri | st(v) |
| } |  |
| nnpfc_formatting_and_purpose_flag | u(1) |
| if( nnpfc_formatting_and_purpose_flag ) { |  |
| nnpfc_target_usage | ue(v) |
| nnpfc_purpose | ue(v) |
| … |  |

nnpfc_target_usage indicates the intended usage of the filtered output sample arrays resulting from the post-processing filter as specified in Table XX. The filtered output sample arrays may undergo further processing, such as colour space conversion, prior to their intended usage.

The value of nnpfc_target_usage shall be in the range of 0 to 2, inclusive, in bitstreams conforming to this edition of this document. Values of 3 to 255, inclusive, for nnpfc_target_usage are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_target_usage in the range of 3 to 255, inclusive. Values of nnpfc_target_usage greater than 255 shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use.

Table XX – Definition of nnpfc_target_usage

| Value | Intended usage of the filtered output sample arrays resulting from the post-processing filter |
| --- | --- |
| 0 | Any usage |
| 1 | User viewing |
| 2 | Machine analysis |

NOTE YY1– When nnpfc_target_usage is equal to 1, the post-processing filter is intended to improve fidelity but could have a negative impact on machine analysis precision. When nnpfc_target_usage is equal to 2, the post-processing filter is intended to improve machine analysis precision but could have a negative impact on subjective quality.

NOTE YY2– When a decoding device displays the video for user viewing rather than performs machine analysis, any post-processing filter that has nnpfc_target_usage equal to 2 is suggested to be omitted. When a decoding device performs machine analysis rather than displays the video, any post-processing filter that has nnpfc_target_usage equal to 1 is suggested to be omitted.

# References

- J. I. Ahonen, R. G. Youvalari, N. Le, H. Zhang, F. Cricri, H. Rezazadegan Tavakoli, M. M. Hannuksela and E. Rahtu, “Learned Enhancement Filters for Image Coding for Machines”, IEEE International Symposium on Multimedia, Nov. – Dec. 2021.

- S. Wang, B. Li, Z. Wang, Y. Ye, S. Wang, “Encoder-only algorithms in Alibaba and City University of Hong Kong’s response to the Video Coding for Machines CfP”, JVET-AB0274.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).