| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0079 |
| --- | --- |

| Title: | [AHG15] Effect of the perceptual QP adaptation (QPA) on machine task performance |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chulkeun Kim, Dong-Gyu Gwak Jaehyun Lim | Tel: Email: | chulkeun.kim@lge.com |
| Source: | LG electronics |  |  |

_____________________________

- Abstract

This contribution reports the impact of the perceptual QP adaptation (QPA) on machine task performance. VTM 12.0 with QPA enabled configuration is used for encoding open image dataset that was used in the evaluation of the VCM CfP and object detection was performed using the decoded results. Experimental results reportedly show 15.17% BD rate gain for object detection.

Introduction

At the 28th meeting of the ITU-T/ISO/IEC Joint Video Experts Team (JVET), an ad hoc group on optimization of encoders and receiving systems for machine analysis of coded video content (AHG 15) was established. There is a mandate about studying non-normative encoder and receiving systems technologies that enhance the performance of machine analysis tasks on coded video content. This contribution reports the machine task performance using the QPA that already exists in VVC. The experimental results show that the use of QPA improves the performance of machine tasks without normative changes.

Proposed method

In this contribution, the effect of subjective quality improvement on machine task is tested by using perceptual QP adaptation (QPA). The perceptual QP adaptation (QPA) that is integrated into the VTM can improve subjective coding quality [1]. Figure 1 represents the experiment process. This process is the same as the anchor generation process of VCM [2], except that ‘--PerceptQPA=1’ is added to the encoding configuration to enable QPA for testing. The decoded video is analyzed using the networks defined in the CTC for VCM [2].

Figure 1. Experiment process.

Experimental results

Table 1 presents the machine task performance comparison between whether QPA is enabled or not. Experimental results reportedly show 15.17% BD rate gain for object detection. Relatively higher performance compared to anchors has been observed at low-medium bit-rates.

Table 1. Machine task performance comparison between fixed QP and QPA.

|  | Anchor(QPA off) |  | Tested(QPA on) |  |
| --- | --- | --- | --- | --- |
| QP | BPP | mAP | BPP | mAP |
| 22 | 0.863 | 78.931 | 1.015 | 78.518 |
| 27 | 0.509 | 78.025 | 0.575 | 78.400 |
| 32 | 0.287 | 77.218 | 0.300 | 77.335 |
| 37 | 0.153 | 74.062 | 0.154 | 75.640 |
| 42 | 0.077 | 68.582 | 0.087 | 71.548 |
| 47 | 0.037 | 57.896 | 0.043 | 61.479 |
|  |  |  | BD-rate | -15.17% |

Figure 2 represents the effect of the QPA. Comparing figures 2 (a) and (b), subjective quality differences are observed in the background area. Quality degradation is observed in the grassy area in Figure 2 (b) with QPA turned on. This is due to the characteristics of QPA, which enhances the quality of regions with low visual activity and degrades the quality of regions with high visual activity. A similar tendency is observed in other example pairs. Specifically, Cat recognition fails with QPA off as shown in figure 2 (e), but succeeds with QPA on in figure 2 (f).

- QPA off (image id: 5854f72f67524d89, QP:37)

- QPA on (image id: 5854f72f67524d89, QP:37)

(c) QPA off (image id: 2fd31c13d1a5fdb3, QP:37)

(d) QPA on (image id: 2fd31c13d1a5fdb3, QP:37)

(e) QPA off (image id: aa243033bdaf2a25, QP:47)

(f) QPA on (image id: aa243033bdaf2a25, QP:47)

Figure 2. Visual effect of QPA

Conclusion and discussion

This contribution reports the impact of the perceptual QP adaptation (QPA) on machine task performance. When QPA is applied, it shows higher performance rather than QPA disabled case on machine tasks without any normative changes. Therefore, it is suggested to study on QPA for machine tasks at the AHG15.

References

- C. Helmrich, H. Schwarz, D. Marpe, T. Wiegand, “Improved perceptually optimized QP adaptation and associated distortion measure,” JVET-K0206, Ljubljana, SI, July 2018.

- ISO/IEC JTC 1/SC 29/WG 2, "Common Test Conditions and Evaluation Methodology for Video Coding for Machines," N00192, Online, April 2022.

Patent rights declaration(s)

LG electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).