| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0080-v1 |
| --- | --- |

| Title: | EE2-3.7: Chroma IBC method as in VTM-5.0 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yingbin Wang, Xiaozhong Xu, Xin Zhao, Roman Chernyak, Shan Liu | Email: | yingbinwang@tencent.com xiaozhongxu@tencent.com xinzzhao@tencent.com chernyak@tencent.com shanl@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

This contribution reports test results of EE2-3.7. In this test, the chroma IBC method in VTM5.0 is studied on top of ECM7.0. This method is used for chroma components when dual tree is activated in intra slice. For each 2x2 chroma sub-block, a block vector is derived from its co-located luma position and used for motion compensation. The overall performance impact of the proposed method on top of ECM-7.0 for SCC content is reported as below. 
For Class-F test sequence:

- AI:		-0.15%/-0.00%/-0.21% with 99% EncT and 99% DecT

- RA:	-0.15%/-0.40%/-0.38% with 99% EncT and 99% DecT

For Class-TGM test sequence:

- AI:		-1.46%/-1.95%/-2.38% with 100% EncT and 98% DecT

- RA:	-0.49%/-0.68%/-0.70% with 100% EncT and 100% DecT

Introduction

In VTM5.0, IBC mode can be applied for chroma component in dual tree partition. The chroma IBC mode does not need to signal BV, but directly derives the block vector from its co-located luma position. For each 2x2 chroma sub-block, a block vector (BV) is derived as followed and used for motion compensation:

bvC[ 0 ] = ( bvL[ 0 ]  >>  ( 3 + SubWidthC ) ) * 32

bvC[ 1 ] = ( bvL[ 1 ]  >>  ( 3 + SubHeightC ) ) * 32

where bvC is the chroma block vector of 2x2 sub-block in 1/32 fractional-sample accuracy, bvL is the block vector of the co-located luma samples in 1/16 fractional-sample accuracy. The variables SubWidthC and SubHeightC depend on the chroma format sampling structure. In YCbCr 4:2:0 sampling, the values of SubWidthC and SubHeightC are equal to 2.

If one of the sub-blocks cannot find a valid block vector, the chroma IBC mode will be disabled by the encoder. 1 additional bit is signalled to indicate whether the chroma IBC mode is used or not. Besides, the following modifications are made to harmonized with existing ECM methods:

1) Luma blocks in IntraTMP mode can be used to derive the chroma BV.

2) The flip-aware BV adjustment approach in [1] is applied to refine the chroma block vector, if the co-located luma position is inside a RRIBC coded block.

Simulation Results

The proposed method was implemented based on ECM-7.0 [2] under common test conditions [3]. The simulation results of the proposed method are provided in Table 1.

Table 1. Simulation results of EE2-3.7

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.15% | 0.00% | -0.21% | 99% | 99% |
| Class TGM | -1.46% | -1.95% | -2.38% | 100% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.15% | -0.40% | -0.38% | 99% | 99% |
| Class TGM | -0.49% | -0.68% | -0.70% | 100% | 100% |

Conclusions

In this contribution, the chroma IBC method in VTM5.0 is studied on top of ECM7.0. The experimental result shows that the proposed method achieves coding performance for screen contents under AI and RA configuration with negligible complexity impact. It is recommended to include the proposed method into the next version of ECM.

References

- Zhipin Deng, Kai Zhang, Li Zhang, “EE2-3.2: Reconstruction-Reordered IBC for screen content coding,” JVET-AA0070, July 2022.

- ECM-7.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-7.0

- M. Karczewicz, et. al., “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, February 2022.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).