| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0082-v2 |
| --- | --- |

| Title: | EE2-1.1 : Directional planar prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sunmi Yoo Jangwon Choi Junghak Nam Myungoh Hong Jaehyun Lim Seung Hwan Kim* 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea 10680 Treena St, San Diego, CA 92131, USA* | Tel: Email: | +82-2-6912-6477 sunmi.yoo@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In this proposal, modified horizontal and vertical planar prediction methods are suggested. First, the additional two modes are considered as horizontal and vertical prediction modes during the reference sample construction and transform processes. In addition, the directional flag is inferred either horizontal planar or vertical planar mode. Specifically, if the DIMD first mode is less than 34, the intra mode of the current block is inferred to be a horizontal planar mode. Otherwise, the intra mode of the current block is set to be a vertical planar mode. The experimental results of the proposed methods are shown below:

Proposed method

The directional planar prediction method was proposed at the last meeting [1]. The horizontal planar prediction block is constructed only using the horizontal interpolation of the planar prediction as shown below:

predx,y=W-1-x*rec-1,y+x+1*recW,-1+(W≫1)≫log2W

The vertical planar prediction is also performed using the top reference samples and the below-left reference sample as follow:

predx,y=H-1-y*recx,-1+y+1*rec-1,H+(H≫1)≫log2H

In the proposed method, the additional two interpolated predictions are considered as horizontal or vertical prediction during the reference sample construction and transform processes. Moreover, the proposed prediction modes are coded in the MPM process – when the MPM index is equal to 0, the planar mode is further truncated-binary coded to indicate whether the current block is the conventional planar or directional planar prediction, then the directional flag is inferred using the DIMD mode. The DIMD mode is always available in the current ECM for the intra prediction block since the DIMD mode is considered as one of the MPM candidates. If the current block is a planar predicted but not a conventional planar mode, and DIMD first mode is less than the mode 34, then the current block is inferred as a horizontal planar mode. Otherwise, if the DIMD first mode is equal to or greater than the mode 34, the current block is inferred as a vertical planar mode.

Experimental results

The proposed methods are implemented on top of ECM-7.0 [2] and the simulations are tested under ECM test condition with all intra and random access configurations [3].

Table 1 The results of the proposed method (AI)

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.08% | -0.04% | 103% | 98% |
| Class A2 | -0.02% | 0.01% | -0.04% | 103% | 99% |
| Class B | -0.03% | 0.02% | -0.16% | 103% | 99% |
| Class C | -0.03% | 0.01% | -0.08% | 103% | 100% |
| Class E | -0.08% | -0.13% | -0.12% | 103% | 100% |
| Overall | -0.04% | 0.00% | -0.10% | 103% | 99% |
| Class D | -0.02% | -0.06% | -0.01% | 103% | 100% |
| Class F | -0.08% | -0.04% | -0.01% | 103% | 98% |

Table 2 The results of the proposed method (RA)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.29% | 0.02% | 101% | 100% |
| Class A2 | -0.01% | 0.21% | 0.16% | 100% | 99% |
| Class B | -0.02% | -0.10% | -0.39% | 99% | 100% |
| Class C | 0.01% | -0.04% | 0.02% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.06% | -0.09% | 100% | 100% |
| Class D | -0.05% | -0.25% | 0.15% | 100% | 99% |
| Class F | -0.08% | 0.05% | 0.05% | 100% | 99% |

References

- S. Yoo, J. Choi, J. Nam, M. Hong, J. Lim, S. Kim, “EE2-related : On directional planar prediction,” JVET-AB0104, Oct. 2022.

- ECM-7.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Feb. 2022.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).