| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0083-v1 |
| --- | --- |

| Title: | EE2-1.4:Combination of directional planar prediction methods |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sunmi Yoo* Jangwon Choi* Junghak Nam* Myungoh Hong* Jaehyun Lim* Seung Hwan Kim* Kyungyong (Kyle) Kim** Dongcheol Kim** Ju-Hyung (John) Son ** Jin Sam Kwak ** Xinwei Li+ Ru-Ling Liao+ Jie Chen+ Yan Ye+ | Tel: Email: | sunmi.yoo@lge.com kyungyong.kim@wilusgroup.com sid.lxw@alibaba-inc.com |
| Source: | LG Electronics Inc*, WILUS Inc.**, Alibaba group+ |  |  |

_____________________________

- Abstract

In this proposal, some adjustments of horizontal and vertical planar prediction are suggested. Firstly, a transform kernel for planar horizontal and planar vertical mode is determined by intra prediction mode of the current block. If it is the planar horizontal/vertical mode, the vertical/horizontal intra prediction mode is used to derive a transform kernel in MTS and LFNST. Further, the directional flag is inferred either horizontal planar or vertical planar mode based on the DIMD mode. If the DIMD first mode is less than 34, the intra mode of the current block is inferred to be a horizontal planar mode. Otherwise, the intra mode of the current block is set to be a vertical planar mode. The proposed prediction method is restricted when the block size is greater than 32x32. The experimental results of the proposed methods are shown below:

  - AI: -0.06% / -0.04% / -0.04% for Y/U/V, 104% / 100% for EncT, DecT

  - RA: X.XX% / X.XX% / X.XX% for Y/U/V, XXX% / XXX% for EncT, DecT

Proposed method

The proposed method is based on the planar horizontal and vertical prediction, and two additional aspects are provided. Firstly, the transform kernel mapping method for the planar horizontal/vertical mode is proposed - this contribution proposes to use vertical/horizontal mode to derive a transform kernel in MTS set and LFNST set as shown in Fig. 1.

If an intra prediction mode of a current block is the planar vertical mode, the horizontal intra prediction mode is used to derive a transform kernel in MTS set and LFNST set. Also, if an intra prediction mode of a current block is the planar horizontal mode, the vertical intra prediction mode is used to derive a transform kernel in MTS set and LFNST set.

Secondly, the directional flag is inferred using the DIMD mode. The DIMD mode is always available in the current ECM for the intra prediction block since the DIMD mode is considered as one of the MPM candidates. If the current block is a planar predicted but not a conventional planar mode, and DIMD first mode is less than the mode 34, then the current block is inferred as a horizontal planar mode. Otherwise, if the DIMD first mode is equal to or greater than the mode 34, the current block is inferred as a vertical planar mode.

Lastly, if the current block size is greater than 32x32, the proposed directional planar prediction is not applied and the flag related to this proposed method is not signaled accordingly.

Figure 1. The proposed setting method of the intra prediction mode used to derive a transform kernel in MTS set and LFNST set

Experimental results

The proposed methods are implemented on top of ECM-7.0 and the simulations are tested under ECM test condition with all intra and random access configurations.

The experimental results of the proposed method are shown in Table 1.

Table 1 The experimental results of the proposed method (AI)

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.09% | -0.01% | 103% | 99% |
| Class A2 | -0.04% | -0.06% | 0.06% | 103% | 100% |
| Class B | -0.05% | -0.06% | -0.12% | 104% | 100% |
| Class C | -0.06% | -0.04% | -0.03% | 105% | 100% |
| Class E | -0.10% | 0.04% | -0.03% | 104% | 100% |
| Overall | -0.06% | -0.04% | -0.04% | 104% | 100% |
| Class D | -0.06% | -0.03% | -0.05% | 105% | 100% |
| Class F | -0.07% | 0.22% | -0.03% | 102% | 100% |

Table 2 The experimental results of the proposed method (RA)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.01% | 0.11% | -0.09% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.02% | -0.08% | -0.04% | 100% | 100% |
| Class F | -0.06% | -0.11% | -0.08% | 99% | 100% |

References

- S. Yoo, J. Choi, J. Nam, M. Hong, J. Lim, S. Kim (LGE), “EE2-related : On directional planar prediction,” JVET-AB0104, Oct. 2022. “

- K. Kim, D. Kim, J.-H. Son, J.-S. Kwak (WILUS), “EE2-1.15-related: Improvements on planar horizontal and planar vertical mode,” JVET-AB0110, Oct. 2022.

- X. Li, R.-L. Liao, J. Chen, Y. Ye, “EE2-related: On horizontal and vertical planar modes,” JVET-AB0162, Oct. 2022.

- ECM-7.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Feb. 2022.

Patent rights declaration(s)

LG Electronics Inc., WILUS Inc. and Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).