| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0085-v1 |
| --- | --- |

| Title: | AHG9: On neural-network post-filter characteristics (NNPFC) SEI message for temporal upsampling towards machine vision |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shurun Wang, Jie Chen, Yan Ye, Shiqi Wang | Email: | shurun.wsr@alibaba-inc.com jiechen.cj@alibaba-inc.com yan.ye@alibaba-inc.com shiqwang@cityu.edu.hk |
| Source: | Alibaba and City University of Hong Kong |  |  |

_____________________________

- Abstract

This contribution proposes to extend the current neural-network post-filter characteristic (NNPFC) SEI message to support temporal upsamling for machine vision..

Introduction

There was a call for proposal (CfP) response of video coding for machines (VCM) from us in last MPEG meeting, which demonstrated temporal resampling can be used to improve VCM performance [1]. The CfP response establishes a VVC based system, called JAST, which is composed of pre-analysis and process, temporal resampling and spatial resampling, and VVC is adopted as the core codec. The encoding diagram is shown in Fig.  1. First, the critical information for VCM of input data I could be extracted with pre-analysis and pre-process, which are content and QP adatptive encoder-only algorithms. The temporal and spatial resampling are further performed to reduce the redundancy for machine vision and generate some side information for decoder reconstruction, such as resampling factor and frame number etc. The bitstream b, the final output of the JAST encoder, is obtained by multiplexing the VVC bitstream and the side information.

Fig.  1 The encoding diagram of JAST

The JAST decoding process is shown in Fig.  2.The JAST bitstream b is parsed to extract the VVC bitstream for VVC decoding and the side information for spatial resampling and temporal resampling. According to the spatial and temporal resampling factors in the side information, spatial and temporal resampling are applied to reconstruct the ultimate output data I', which can be fed to any machine vision tasks.

Fig.  2 The decoding diagram of JAST

Whereas spatial filtering already is supported in NNPF, temporal filtering is not yet supported, so we want to add it as a new purpose for NNPF.

Proposal

In this section, we describe the proposed syntax and semantics modifications on the basis of current NNPFC SEI message [2].

Syntax of NNPFC SEI message for machine vision

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_id | ue(v) |
| nnpfc_mode_idc | ue(v) |
| nnpfc_purpose_and_formatting_flag | u(1) |
| if( nnpfc_purpose_and_formatting_flag ) { |  |
| nnpfc_purpose | ue(v) |
| if( nnpfc_purpose = = 2 ¦ ¦ nnpfc_purpose = = 4 ) |  |
| nnpfc_out_sub_c_flag | u(1) |
| if( nnpfc_purpose = = 3 ¦ ¦ nnpfc_purpose = = 4 ) { |  |
| nnpfc_pic_width_in_luma_samples | ue(v) |
| nnpfc_pic_height_in_luma_samples | ue(v) |
| } |  |
| if(nnpfc_purpose = = 5){ |  |
| nnpfc_temp_output_pic_no_minus1 | ue(v) |
| nnpfc_inp_range_div2_minus1 | ue(v) |
| } |  |
| /* input and output formatting */ |  |
| nnpfc_component_last_flag | u(1) |
| nnpfc_inp_format_flag | u(1) |
| if( nnpfc_inp_format_flag = = 1 ) |  |
| nnpfc_inp_tensor_bitdepth_minus8 | ue(v) |
| nnpfc_inp_order_idc | ue(v) |
| nnpfc_auxiliary_inp_idc | ue(v) |
| nnpfc_separate_colour_description_present_flag | u(1) |
| if( nnpfc_separate_colour_description_present_flag ) { |  |
| nnpfc_colour_primaries | u(8) |
| nnpfc_transfer_characteristics | u(8) |
| nnpfc_matrix_coeffs | u(8) |
| } |  |
| nnpfc_out_format_flag | u(1) |
| if( nnpfc_out_format_flag = = 1 ) |  |
| nnpfc_out_tensor_bitdepth_minus8 | ue(v) |
| nnpfc_out_order_idc | ue(v) |
| nnpfc_constant_patch_size_flag | u(1) |
| nnpfc_patch_width_minus1 | ue(v) |
| nnpfc_patch_height_minus1 | ue(v) |
| nnpfc_overlap | ue(v) |
| nnpfc_padding_type | ue(v) |
| if( nnpfc_padding_type = = 4 ){ |  |
| nnpfc_luma_padding_val | ue(v) |
| nnpfc_cb_padding_val | ue(v) |
| nnpfc_cr_padding_val | ue(v) |
| } |  |
| nnpfc_complexity_idc | ue(v) |
| if( nnpfc_complexity_idc > 0 ) |  |
| nnpfc_complexity_element( nnpfc_complexity_idc ) |  |
| if( nnpfc_mode_idc = = 2 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_reserved_zero_bit | u(1) |
| nnpfc_uri_tag[ i ] | st(v) |
| nnpfc_uri[ i ] | st(v) |
| } |  |
| } |  |
| /* filter specified or updated by ISO/IEC 15938-17 bitstream */ |  |
| if( nnpfc_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_reserved_zero_bit | u(1) |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| nnpfc_payload_byte[ i ] | b(8) |
| } |  |
| } |  |

| nnpfc_complexity_element( nnpfc_complexity_idc ) { | Descriptor |
| --- | --- |
| if( nnpfc_complexity_idc = = 1 ) { |  |
| nnpfc_parameter_type_idc | u(2) |
| if (nnpfc_parameter_type_idc ! = 2) |  |
| nnpfc_log2_parameter_bit_length_minus3 | u(2) |
| nnpfc_num_parameters_idc | u(6) |
| nnpfc_num_kmac_operations_idc | ue(v) |
| } |  |
| } |  |

Semantics of NNPFC SEI message for machine vision

nnpfc_purpose indicates the purpose of post-processing filter as specified in Table 20. The value of nnpfc_purpose shall be in the range of 0 to 232 − 2, inclusive. Values of nnpfc_purpose that do not appear in Table 20 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore SEI messages that contain reserved values of nnpfc_purpose.

Table 1 – Definition of nnpfc_purpose

| Value | Interpretation |
| --- | --- |
| 0 | Unknown or unspecified |
| 1 | Visual quality improvement |
| 2 | Chroma upsampling from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format |
| 3 | Increasing the width or height of the cropped decoded output picture without changing the chroma format |
| 4 | Increasing the width or height of the cropped decoded output picture and upsampling the chroma format |
| 5 | Perform the temporal upsampling to improve the machine vision performance |

nnpfc_temp_output_pic_no_minus1 plus 1 specifies the number of the upsampled pictures generated by the post-processing filter between the current picture and the previous picture in display order. The value of nnpfc_temp_output_pic_no_minus1 shall be in the range of 0 to 232 – 2, inclusive. When nnpfc_temp_output_pic_no_minus1 is not present, nnpfc_temp_output_pic_no_minus1 is inferred to be equal to 0..

NOTE – Specifically, if nnpfc_temp_output_pic_no_minus1 is 1, there should be 2 frames generated between the current picture and the previous picture in display order..

nnpfc_inp_range_div2_minus1  plus 1 specifies the half number of the input picture to the post-processing filter for temporal picture upsamling. The value of nnpfc_inp_range_div2_minus1 shall be in the range of 0 to 232 – 2, inclusive. If the index of the current picture is CurPicIdx in disaplay order, the input pictures are the pictures with index of CurPicIdx – (nnpfc_inp_range_div2_minus1+1), CurPicIdx - nnpfc_inp_range_div2_minus1-2, … CurPicIdx -1, CurPicIdx, CurPicIdx +1,…, CurPicIdx + nnpfc_inp_range_div2_minus1 in display order. When nnpfc_inp_range_div2_minus1 is not present, nnpfc_inp_range_div2_minus1 is inferred to be equal to 0. It is a requirement of bitstream conformance that the following constraints apply:

- (nnpfc_inp_range_div2_minus1+1)<<5shall be greater than (nnpfc_temp_output_pic_no_minus1+1)

- (nnpfc_temp_output_pic_no_minus1+1)<< 3 shall be greater than . (nnpfc_inp_range_div2_minus1+1)

NOTE – For example, if nnpfc_temp_output_pic_no_minus1+1 is 2 and the current reconstructed frame index is i, there should be 2 frames generated between decoded frame i-1 and decoded frame i by means of the temporal upsampling model.

The function InpSampleVal( y, x, picHeight, picWidth, croppedSeq, picIdx) with inputs being a vertical sample location y, a horizontal sample location x, a picture height picHeight, a picture width picWidth, and sample array croppedSeq returns the value of sampleVal derived as follows (croppedSeq is the sample array of the decoded video file with spatial-wise crop only before temporal upsampling, and the decoded frame indexes, denoted as picIdx, are in the highest dimension, which means the shape of croppedSeq is [picWidth, picHeight, picture_num] and picture_num is the picture number of the decoded sequence):

croppedPic= croppedSeq[:, :, picIdx]

if( nnpfc_padding_type  = =  0 )
	if( y < 0  | |  x < 0  | |  y >= picHeight  | |  x  >=  picWidth )
		sampleVal = 0
	else
		sampleVal = croppedPic[ x ][ y ]		(82)
else if( nnpfc_padding_type  = =  1 )
	sampleVal = croppedPic[ Clip3( 0, picWidth − 1, x ) ][ Clip3( 0, picHeight − 1, y ) ]
else if( nnpfc_padding_type  = =  2 ) 
	sampleVal = croppedPic[ Reflect( picWidth − 1, x ) ][ Reflect( picHeight − 1, y ) ]
else if( nnpfc_padding_type  = =  3 ) 
	if( y >= 0  &&  y < picHeight )
		sampleVal = croppedPic[ Wrap( picWidth − 1, x ) ][ y ] 
else if( nnpfc_padding_type  = =  4 ) 
	if( y < 0  | |  x < 0  | |  y >= picHeight  | |  x  >=  picWidth )
		sampleVal[ 0 ]  = nnpfc_luma_padding_val	  sampleVal[ 1 ]  = nnpfc_cb_padding_val
		sampleVal[ 2 ] = nnpfc_cr_padding_val
	else
		sampleVal = croppedPic[ x ][ y ]

The input of the temporal upsampling network should also be specified as follows, by a process called DeriveInputTensors( ). CurPicIdx indicates the picture index of the current picture in display order.

Table 2 – Process for deriving the input tensors inputTensor for a given vertical sample coordinate cTop and a horizontal sample coordinate cLeft specifying the top-left sample location for the patch of samples included in the input tensors

| nnpfc_inp_ order_idc | Process DeriveInputTensors( ) for deriving input tensors |
| --- | --- |
| 0 | for(idx_picture_input= (nnpfc_purpose==5? CurPicIdx- (nnpfc_inp_range_div2_minus1+1): CurPicIdx); idx_picture_input < (nnpfc_purpose==5? CurPicIdx + (nnpfc_inp_range_div2_minus1+1): CurPicIdx+1); idx_picture_input++){ for( yP = −overlapSize; yP < inpPatchHeight + overlapSize; yP++) for( xP = −overlapSize; xP < inpPatchWidth + overlapSize; xP++ ) { inpVal = InpY( InpSampleVal( cTop + yP, cLeft + xP, CroppedHeight, CroppedWidth, croppedSeq, idx_picture_input) ) if( nnpfc_component_last_flag = = 0 ) inputTensor[ 0 ][ idx_picture_input- CurPicIdx+ nnpfc_inp_range_div2_minus1+1][ yP + overlapSize ][ xP + overlapSize ] = inpVal else inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ idx_picture_input- CurPicIdx+ nnpfc_inp_range_div2_minus1+1 ] = inpVal if( nnpfc_auxiliary_inp_idc = = 1 ) if( nnpfc_component_last_flag = = 0 ) inputTensor[ 0 ][ 2* (nnpfc_inp_range_div2_minus1+1)][ yP + overlapSize ][ xP + overlapSize ] = 2( StrengthControlVal − 42 ) / 6 else inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 2* (nnpfc_inp_range_div2_minus1+1)] = 2( StrengthControlVal − 42 ) / 6 } } |
| 1 | for(idx_picture_input= (nnpfc_purpose==5 ? CurPicIdx- (nnpfc_inp_range_div2_minus1+1): CurPicIdx); idx_picture_input < (nnpfc_purpose==5 ? CurPicIdx + (nnpfc_inp_range_div2_minus1+1): CurPicIdx+1); idx_picture_input++){ for( yP = −overlapSize; yP < inpPatchHeight + overlapSize; yP++) for( xP = −overlapSize; xP < inpPatchWidth + overlapSize; xP++ ) { inpCbVal = InpC( InpSampleVal( cTop + yP, cLeft + xP, CroppedHeight / SubHeightC, CroppedWidth / SubWidthC, croppedSeq, idx_picture_input) ) inpCrVal = InpC( InpSampleVal( cTop + yP, cLeft + xP, CroppedHeight / SubHeightC, CroppedWidth / SubWidthC, croppedSeq, idx_picture_input) ) if( nnpfc_component_last_flag = = 0 ) { inputTensor[ 0 ][ idx_picture_input- CurPicIdx+ nnpfc_inp_range_div2_minus1+1 ][ yP + overlapSize ][ xP + overlapSize ] = inpCbVal inputTensor[ 0 ][ idx_picture_input- CurPicIdx+ (2+1)* (nnpfc_inp_range_div2_minus1+1)][ yP + overlapSize ][ xP + overlapSize ] = inpCrVal } else { inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ idx_picture_input- CurPicIdx+ nnpfc_inp_range_div2_minus1+1 ] = inpCbVal inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ idx_picture_input- CurPicIdx+ (2+1)* (nnpfc_inp_range_div2_minus1+1) ] = inpCrVal } if(nnpfc_auxiliary_inp_idc = = 1) if( nnpfc_component_last_flag = = 0 ) inputTensor[ 0 ][ 4* (nnpfc_inp_range_div2_minus1+1) ][ yP + overlapSize ][ xP + overlapSize ] = 2( StrengthControlVal − 42 ) / 6 else inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 4* (nnpfc_inp_range_div2_minus1+1) ] = 2( StrengthControlVal − 42 ) / 6 } |
| 2 | for(idx_picture_input= (nnpfc_purpose==5 ? CurPicIdx- (nnpfc_inp_range_div2_minus1+1): CurPicIdx); idx_picture_input < (nnpfc_purpose==5 ? CurPicIdx + (nnpfc_inp_range_div2_minus1+1): CurPicIdx+1); idx_picture_input++){ for( yP = −overlapSize; yP < inpPatchHeight + overlapSize; yP++) for( xP = −overlapSize; xP < inpPatchWidth + overlapSize; xP++ ) { yY = cTop + yP xY = cLeft + xP yC = yY / SubHeightC xC = xY / SubWidthC inpYVal = InpY( InpSampleVal( yY, xY, CroppedHeight, CroppedWidth, croppedSeq, idx_picture_input) ) inpCbVal = InpC( InpSampleVal( yC, xC, CroppedHeight / SubHeightC, CroppedWidth / SubWidthC, croppedSeq, idx_picture_input ) ) inpCrVal = InpC( InpSampleVal( yC, xC, CroppedHeight / SubHeightC, CroppedWidth / SubWidthC, croppedSeq, idx_picture_input ) ) if( nnpfc_component_last_flag = = 0 ) { inputTensor[ 0 ][ idx_picture_input- CurPicIdx+ nnpfc_inp_range_div2_minus1+1 ][ yP + overlapSize ][ xP + overlapSize ] = inpYVal inputTensor[ 0 ][ idx_picture_input- CurPicIdx+(2+1)* (nnpfc_inp_range_div2_minus1+1) ][ yP + overlapSize ][ xP + overlapSize ] = inpCbVal inputTensor[ 0 ][ idx_picture_input- CurPicIdx+ (4+1)* (nnpfc_inp_range_div2_minus1+1) ][ yP + overlapSize ][ xP + overlapSize ] = inpCrVal } else { inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ idx_picture_input- CurPicIdx+ nnpfc_inp_range_div2_minus1+1 ] = inpYVal inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ idx_picture_input- CurPicIdx+(2+1)*(nnpfc_inp_range_div2_minus1+1) ] = inpCbVal inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ idx_picture_input- CurPicIdx+(4+1)* (nnpfc_inp_range_div2_minus1+1) ] = inpCrVal } if(nnpfc_auxiliary_inp_idc = = 1) if( nnpfc_component_last_flag = = 0 ) inputTensor[ 0 ][ 6* (nnpfc_inp_range_div2_minus1+1) ][ yP + overlapSize ][ xP + overlapSize ] = 2( StrengthControlVal − 42 ) / 6 else inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 6* (nnpfc_inp_range_div2_minus1+1) ] = 2( StrengthControlVal − 42 ) / 6 } } |
| 3 | for(idx_picture_input= (nnpfc_purpose==5 ? CurPicIdx- (nnpfc_inp_range_div2_minus1+1): CurPicIdx); idx_picture_input < (nnpfc_purpose==5 ? CurPicIdx + (nnpfc_inp_range_div2_minus1+1): CurPicIdx+1); idx_picture_input++){ for( yP = −overlapSize; yP < inpPatchHeight + overlapSize; yP++) for( xP = −overlapSize; xP < inpPatchWidth + overlapSize; xP++ ) { yTL = cTop + yP * 2 xTL = cLeft + xP * 2 yBR = yTL + 1 xBR = xTL + 1 yC = cTop / 2 + yP xC = cLeft / 2 + xP inpTLVal = InpY( InpSampleVal( yTL, xTL, CroppedHeight, CroppedWidth, croppedSeq, idx_picture_input ) ) inpTRVal = InpY( InpSampleVal( yTL, xBR, CroppedHeight, CroppedWidth, croppedSeq, idx_picture_input) ) inpBLVal = InpY( InpSampleVal( yBR, xTL, CroppedHeight, CroppedWidth, croppedSeq, idx_picture_input ) ) inpBRVal = InpY( InpSampleVal( yBR, xBR, CroppedHeight, CroppedWidth, croppedSeq, idx_picture_input ) ) inpCbVal = InpC( InpSampleVal( yC, xC, CroppedHeight / 2, CroppedWidth / 2, croppedSeq, idx_picture_input ) ) inpCrVal = InpC( InpSampleVal( yC, xC, CroppedHeight / 2, CroppedWidth / 2, croppedSeq, idx_picture_input ) ) if( nnpfc_component_last_flag = = 0 ) { inputTensor[ 0 ][ idx_picture_input- CurPicIdx+ nnpfc_inp_range_div2_minus1+1 ][ yP + overlapSize ][ xP + overlapSize ] = inpTLVal inputTensor[ 0 ][ idx_picture_input- CurPicIdx+(1+2)* (nnpfc_inp_range_div2_minus1+1) ][ yP + overlapSize ][ xP + overlapSize ] = inpTRVal inputTensor[ 0 ][ idx_picture_input- CurPicIdx+(1+4)* (nnpfc_inp_range_div2_minus1+1) ][ yP + overlapSize ][ xP + overlapSize ] = inpBLVal inputTensor[ 0 ][ idx_picture_input- CurPicIdx+(1+6)* (nnpfc_inp_range_div2_minus1+1) ][ yP + overlapSize ][ xP + overlapSize ] = inpBRVal inputTensor[ 0 ][ idx_picture_input- CurPicIdx+(1+8)* (nnpfc_inp_range_div2_minus1+1) ][ yP + overlapSize ][ xP + overlapSize ] = inpCbVal inputTensor[ 0 ][ idx_picture_input- CurPicIdx+(1+10)* (nnpfc_inp_range_div2_minus1+1) ][ yP + overlapSize ][ xP + overlapSize ] = inpCrVal } else { inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ idx_picture_input- CurPicIdx+ nnpfc_inp_range_div2_minus1+1 ] = inpTLVal inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ idx_picture_input- CurPicIdx+ (1+2)* (nnpfc_inp_range_div2_minus1+1) ] = inpTRVal inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ idx_picture_input- CurPicIdx+ (1+4)* (nnpfc_inp_range_div2_minus1+1) ] = inpBLVal inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ idx_picture_input- CurPicIdx+(1+6)* (nnpfc_inp_range_div2_minus1+1) ] = inpBRVal inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ idx_picture_input- CurPicIdx+ (1+8)* (nnpfc_inp_range_div2_minus1+1) ] = inpCbVal inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ idx_picture_input- CurPicIdx+ (1+10)* (nnpfc_inp_range_div2_minus1+1) ] = inpCrVal } if(nnpfc_auxiliary_inp_idc = = 1) if( nnpfc_component_last_flag = = 0 ) inputTensor[ 0 ][ 10* (nnpfc_inp_range_div2_minus1+1) ][ yP + overlapSize ][ xP + overlapSize ] = 2( StrengthControlVal − 42 ) / 6 else inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 10* (nnpfc_inp_range_div2_minus1+1) ] = 2( StrengthControlVal − 42 ) / 6 } } |
| 4..255 | Reserved |

Moreover, the process for deriving sample values in the filtered output sample arrays is also specified as below.

Table 3 – Process for deriving sample values in the filtered output sample arrays FilteredYPic, FilteredCbPic, and FilteredCrPic from the output tensors outputTensor for a given vertical sample coordinate cTop and a horizontal sample coordinate cLeft specifying the top-left sample location for the patch of samples included in the input tensors

| nnpfc_out_ order_idc | Process StoreOutputTensors( ) for deriving sample values in the filtered picture from the output tensors |
| --- | --- |
| 0 | for(cur_out_idx=0; cur_out_idx< nnpfc_temp_output_pic_no_minus1+1; cur_out_idx++){ for( yP = 0; yP < outPatchHeight; yP++) for( xP = 0; xP < outPatchWidth; xP++ ) { yY = cTop * outPatchHeight / inpPatchHeight + yP xY = cLeft * outPatchWidth / inpPatchWidth + xP if ( yY < nnpfc_pic_height_in_luma_samples && xY < nnpfc_pic_width_in_luma_samples ) if( nnpfc_component_last_flag = = 0 ) FilteredYPic[ xY ][yY ][ cur_out_idx] = OutY( outputTensor[ 0 ][ cur_out_idx ][ yP ][ xP ] ) else FilteredYPic[ xY ][ yY ][ cur_out_idx] = OutY( outputTensor[ 0 ][ yP ][ xP ][ cur_out_idx ] ) } } |
| 1 | for(cur_out_idx=0; cur_out_idx< nnpfc_temp_output_pic_no_minus1+1; cur_out_idx++){ for( yP = 0; yP < outPatchCHeight; yP++) for( xP = 0; xP < outPatchCWidth; xP++ ) { xSrc = cLeft * horCScaling + xP ySrc = cTop * verCScaling + yP if ( ySrc < nnpfc_pic_height_in_luma_samples / outSubHeightC && xSrc < nnpfc_pic_width_in_luma_samples / outSubWidthC ) if( nnpfc_component_last_flag = = 0 ) { FilteredCbPic[ xSrc ][ ySrc ][ cur_out_idx] = OutC( outputTensor[ 0 ][ cur_out_idx ][ yP ][ xP ] ) FilteredCrPic[ xSrc ][ ySrc ][ cur_out_idx] = OutC( outputTensor[ 0 ][ nnpfc_temp_output_pic_no_minus1+1 +cur_out_idx ][ yP ][ xP ] ) } else { FilteredCbPic[ xSrc ][ ySrc ] = OutC( outputTensor[ 0 ][ yP ][ xP ][ cur_out_idx ] ) FilteredCrPic[ xSrc ][ ySrc ] = OutC( outputTensor[ 0 ][ yP ][ xP ][ nnpfc_temp_output_pic_no_minus1+1 +cur_out_idx ] ) } } } |
| 2 | for(cur_out_idx=0; cur_out_idx< nnpfc_temp_output_pic_no_minus1+1; cur_out_idx++){ for( yP = 0; yP < outPatchHeight; yP++) for( xP = 0; xP < outPatchWidth; xP++ ) { yY = cTop * outPatchHeight / inpPatchHeight + yP xY = cLeft * outPatchWidth / inpPatchWidth + xP yC = yY / outSubHeightC xC = xY / outSubWidthC yPc = ( yP / outSubHeightC ) * outSubHeightC xPc = ( xP / outSubWidthC ) * outSubWidthC if ( yY < nnpfc_pic_height_in_luma_samples && xY < nnpfc_pic_width_in_luma_samples) if( nnpfc_component_last_flag = = 0 ) { FilteredYPic[ xY ][ yY ] = OutY( outputTensor[ 0 ][ cur_out_idx ][ yP ][ xP ] ) FilteredCbPic[ xC ][ yC ] = OutC( outputTensor[ 0 ][ cur_out_idx+ nnpfc_temp_output_pic_no_minus1+1 ][ yPc ][ xPc ] ) FilteredCrPic[ xC ][ yC ] = OutC( outputTensor[ 0 ][ cur_out_idx+2* (nnpfc_temp_output_pic_no_minus1+1) ][ yPc ][ xPc ] ) } else { FilteredYPic[ xY ][ yY ] = OutY( outputTensor[ 0 ][ yP ][ xP ][ cur_out_idx ] ) FilteredCbPic[ xC ][ yC ] = OutC( outputTensor[ 0 ][ yPc ][ xPc ][ cur_out_idx+ nnpfc_temp_output_pic_no_minus1+1 ] ) FilteredCrPic[ xC ][ yC ] = OutC( outputTensor[ 0 ][ yPc ][ xPc ][ cur_out_idx+2* (nnpfc_temp_output_pic_no_minus1+1) ] ) } } } |
| 3 | for(cur_out_idx=0; cur_out_idx< nnpfc_temp_output_pic_no_minus1+1; cur_out_idx++){ for( yP = 0; yP < outPatchHeight; yP++ ) for( xP = 0; xP < outPatchWidth; xP++ ) { ySrc = cTop / 2 * outPatchHeight / inpPatchHeight + yP xSrc = cLeft / 2 * outPatchWidth / inpPatchWidth + xP if ( ySrc < nnpfc_pic_height_in_luma_samples / 2 && xSrc < nnpfc_pic_width_in_luma_samples / 2 ) if( nnpfc_component_last_flag = = 0 ) { FilteredYPic[ xSrc * 2 ][ ySrc * 2 ] = OutY( outputTensor[ 0 ][ cur_out_idx ][ yP ][ xP ] ) FilteredYPic[ xSrc * 2 + 1 ][ ySrc * 2 ] = OutY( outputTensor[ 0 ][ cur_out_idx+ nnpfc_temp_output_pic_no_minus1+1 ][ yP ][ xP ] ) FilteredYPic[ xSrc * 2 ][ ySrc * 2 + 1 ] = OutY( outputTensor[ 0 ][ cur_out_idx+ 2*(nnpfc_temp_output_pic_no_minus1+1) ][ yP ][ xP ] ) FilteredYPic[ xSrc * 2 + 1][ ySrc * 2 + 1 ] = OutY( outputTensor[ 0 ][ cur_out_idx+ 3*(nnpfc_temp_output_pic_no_minus1+1) ][ yP ][ xP ] ) FilteredCbPic[ xSrc ][ ySrc ] = OutC( outputTensor[ 0 ][ cur_out_idx+ 4*(nnpfc_temp_output_pic_no_minus1+1) ][ yP ][ xP ] ) FilteredCrPic[ xSrc ][ ySrc ] = OutC( outputTensor[ 0 ][ cur_out_idx+ 5*(nnpfc_temp_output_pic_no_minus1+1) ][ yP ][ xP ] ) } else { FilteredYPic[ xSrc * 2 ][ ySrc * 2 ] = OutY( outputTensor[ 0 ][ yP ][ xP ][ cur_out_idx ] ) FilteredYPic[ xSrc * 2 + 1 ][ ySrc * 2 ] = OutY( outputTensor[ 0 ][ yP ][ xP ][ cur_out_idx+ nnpfc_temp_output_pic_no_minus1+1 ] ) FilteredYPic[ xSrc * 2 ][ ySrc * 2 + 1 ] = OutY( outputTensor[ 0 ][ yP ][ xP ][ cur_out_idx+ 2*(nnpfc_temp_output_pic_no_minus1+1) ] ) FilteredYPic[ xSrc * 2 + 1][ ySrc * 2 + 1 ] = OutY( outputTensor[ 0 ][ yP ][ xP ][ cur_out_idx+ 3*(nnpfc_temp_output_pic_no_minus1+1) ] ) FilteredCbPic[ xSrc ][ ySrc ] = OutC( outputTensor[ 0 ][ yP ][ xP ][ cur_out_idx+ 4*(nnpfc_temp_output_pic_no_minus1+1) ] ) FilteredCrPic[ xSrc ][ ySrc ] = OutC( outputTensor[ 0 ][ yP ][ xP ][ cur_out_idx+ 5*(nnpfc_temp_output_pic_no_minus1+1) ] ) } } } |
| 4..255 | Reserved |

conclusion

The current neural-network post-filter characteristics SEI message is extended to support temporal upsampling towards machine vision in this contriution. It is suggested to consider the adoption of the proposed NNPFC SEI extension in future version of VSEI.

References

- Shurun Wang, Binzhe Li, Zhao Wang, Shiqi Wang, Yan Ye, “[VCM] Video Coding for Machines CfP Response from Alibaba and City University of Hong Kong,” MPEG-m60737, Oct. 2022.

- Sean McCarthy, Takeshi Chujoh, Miska Hannuksela, Gary Sullivan, " Additional SEI messages for VSEI (Draft 2)," JVET-AA2006, Jul. 2022.

Patent rights declaration(s)

Alibaba and City University of Hong Kong may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).