| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0085-v2 |
| --- | --- |

| Title: | AHG9: On neural-network post-filter characteristics (NNPFC) SEI message for temporal upsampling towards machine vision |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shurun Wang, Jie Chen, Yan Ye, Shiqi Wang | Email: | shurun.wsr@alibaba-inc.com jiechen.cj@alibaba-inc.com yan.ye@alibaba-inc.com shiqwang@cityu.edu.hk |
| Source: | Alibaba and City University of Hong Kong |  |  |

_____________________________

- Abstract

This contribution proposes to refine the current neural-network post-filter characteristic (NNPFC) SEI message on picture rate upsamling for machine vision. Specifically, constraints on the input number and output number of pictures for picture rate upsampling are proposed based on the performance and implementation cost consideration.

Introduction

There was a call for proposal (CfP) response of video coding for machines (VCM) from us in last MPEG meeting, which demonstrated temporal resampling can be used to improve VCM performance [1]. The CfP response establishes a VVC based system, called JAST, which is composed of pre-analysis and process, temporal resampling and spatial resampling, and VVC is adopted as the core codec. The encoding diagram is shown in Fig.  1. First, the critical information for VCM of input data I could be extracted with pre-analysis and pre-process, which are content and QP adatptive encoder-only algorithms. The temporal and spatial resampling are further performed to reduce the redundancy for machine vision and generate some side information for decoder reconstruction, such as resampling factor and frame number etc. The bitstream b, the final output of the JAST encoder, is obtained by multiplexing the VVC bitstream and the side information.

Fig.  1 The encoding diagram of JAST

The JAST decoding process is shown in Fig.  2.The JAST bitstream b is parsed to extract the VVC bitstream for VVC decoding and the side information for spatial resampling and temporal resampling. According to the spatial and temporal resampling factors in the side information, spatial and temporal resampling are applied to reconstruct the ultimate output data I', which can be fed to any machine vision tasks.

Fig.  2 The decoding diagram of JAST

Whereas the picture rate resampling was adopted at JVET-AB meeting[2] and could be used for VCM purpose, the relationship between the input picture number and output picture number has not been considered. The unlimited input and output number of the pictures could degrade the performance for VCM and will also introduce implementation issues to a real post-processing product.

Proposal

In version 2, based on the current design of picture rate upsampling of NNPFC SEI in VSEI draft 3[2], it is proposed to impose two constraints on numInputPics and numOutputPics.

Specifically, according to JVET-AB2006[2], when the purpose of NNPFC is for picture rate upsampling (nnpfc_purpose = = 5), the picture number of the input and output for the filter, denoted as numInputPics and numOutputPics respectively, could be deduced based on nnpfc_num_input_pics_minus2 and nnpfc_interpolated_pics, which are shown as follows,

if( nnpfc_purpose  = =  5 ) {
	numInputPics = nnpfc_num_input_pics_minus2 + 2
	for( i = 0, numOutputPics = 0; i  <=  numInputPics − 2; i++ ) 	(1)
		numOutputPics  +=  nnpfc_interpolated_pics[ i ]
}

Constraint 1-upper bound for picture rate upsampling

16× numInputPics >= numOutputPics

Constraint 1 tends to constraint the upper bound for picture rate upsampling in terms of the ratio between numOutputPics and numInputPics to avoid unlimited output number of the pictures, as an unlimited output number of pictures is not feasible for a real product considering computational capability and the implementation cost. Moreover, the performance of picture rate upsampling could be inferior if the output picture number is too much larger than the input picture number, especially for the high temporal complexity scenarios.

Constraint 2-lower bound for picture rate upsampling

numInputPics <=16× numOutputPics

Similarly, Constraint 2 tends to constraint the lower bound for picture rate upsampling. If the temporal complexity of the video sequence is high, more input pictures are required for better picture rate upsampling performance, as constrained in Constraint 1. On the contrary, if the temporal complexity is low, too many input picture for temporal rate upsampling is not necessary, which is constrained by Constraint 2.

conclusion

Two constraints are proposed for the relationship between the input and output picture number for picture rate upsampling in NNPFC SEI. It is suggested to consider the adoption of the proposed constraint(s) in the next version of VSEI.

References

- Shurun Wang, Binzhe Li, Zhao Wang, Shiqi Wang, Yan Ye, “[VCM] Video Coding for Machines CfP Response from Alibaba and City University of Hong Kong,” MPEG-m60737, Oct. 2022.

- Sean McCarthy, Takeshi Chujoh, Miska Hannuksela, Gary Sullivan, Yekui Wang, " Additional SEI messages for VSEI (Draft 3)," JVET-AB2006, Oct. 2022.

Patent rights declaration(s)

Alibaba and City University of Hong Kong may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).