| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0087-v1 |
| --- | --- |

| Title: | Non-EE2: Intra TMP with half-pel precision |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xinwei Li Ru-Ling Liao Jie Chen Yan Ye | Tel: Email: | {sid.lxw, ruling.lrl, jiechen.cj, yan.ye}@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

In this contribution, half-pel precision is supported in Intra TMP. Specifically, half-pel positions in 8 directions adjacent to the optimal integer-pel position obtained by template matching are further enabled to Intra TMP. The selected precision and directions are signaled in CU level. A 4-tap DCT-IF interpolation filter is used for half-pel interpolation. It is reported that on top of ECM-7.0, the overall coding performance impact for {Y, U, V, EncT, DecT} is {-x.xx%, -x.xx %, -x.xx %, xxx%, xxx%} for AI and {-x.xx%, -x.xx %, -x.xx %, xxx%, xxx%} for RA.

Introduction

Intra template matching prediction (Intra TMP) is a newly added intra prediction mode in ECM [1]. Intra TMP generates the predicted values of the current block by copying the reconstructed values of the matching block in the current frame, whose position is determined by template matching at both the encoder side and the decoder side. The template matching is performed based on the L-shaped template of the current block and SAD is used as a cost function. In ECM-7.0, the Intra TMP supports not only screen contents, but also camera capture contents [2]. To speed-up the template matching, the search region is sub-sampled by a factor of 2. After finding the best match, a refinement process is performed in which another template matching search is performed around the best match with a reduced search range.

However, Intra TMP only supports integer-pel precision, which may limit the prediction accuracy, especially on camera-captured contents which have richer texture.

Proposed method

It is proposed to enable half-pel precision in Intra TMP. The template matching process is not changed, which will find an integer-pel matching position. In the proposed method, the encoder will further try 8 adjacent half-pel positions around the integer-pel position in 8 directions as shown in the Figure 1, and select one of the 9 positions (1 integer-pel position + 8 half-pel positions) by RDO.

If Intra TMP mode is selected for the current block, a flag is further signaled to indicate whether to use integer-pel or half-pel precision. When using half-pel precision, an index is further signaled to indicate the direction of the half-pel position. A 4-tap DCT-IF interpolation filter, [-5, 37, 37, -5], is used for half-pel interpolation in Intra TMP.

Figure 1. The adjacent half-pel positions in 8 directions.

Simulation results

The proposed method is implemented on top of ECM-7.0 [3]. The simulation results are tested under ECM test condition [4] with all intra and random access configurations as shown in Table 1.

Table 1. Simulation results of the proposed method

|  |  |  |  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Over ECM-7.0 |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  |  | V |  | EncT |  |  | DecT |  |  |
| Class A1 | -0.01% |  | -0.06% |  |  | -0.02% |  | 102% |  |  | 100% |  |  |
| Class A2 | #VALUE! |  | #VALUE! |  |  | #VALUE! |  | #NUM! |  |  | #NUM! |  |  |
| Class B | -0.12% |  | -0.12% |  |  | -0.20% |  | 104% |  |  | 100% |  |  |
| Class C | -0.02% |  | -0.11% |  |  | -0.15% |  | 102% |  |  | 101% |  |  |
| Class E | -0.29% |  | -0.28% |  |  | -0.49% |  | 103% |  |  | 102% |  |  |
| Overall | #VALUE! |  | #VALUE! |  |  | #VALUE! |  | #NUM! |  |  | #NUM! |  |  |
| Class D | 0.00% |  | -0.20% |  |  | -0.09% |  | 101% |  |  | 100% |  |  |
| Class F | -0.04% |  | 0.12% |  |  | 0.07% |  | 103% |  |  | 101% |  |  |
| Class TGM | -0.03% |  | 0.01% |  |  | 0.01% |  | 105% |  |  | 100% |  |  |
|  |  |  |  | Random access Main10 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Over ECM-7.0 |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  |  | V |  | EncT |  |  | DecT |  |  |
| Class A1 | -0.04% |  | -0.08% |  |  | 0.12% |  | 101% |  |  | 100% |  |  |
| Class A2 | #VALUE! |  | #VALUE! |  |  | #VALUE! |  | #DIV/0! |  |  | #DIV/0! |  |  |
| Class B | -0.07% |  | -0.13% |  |  | -0.25% |  | 101% |  |  | 100% |  |  |
| Class C | -0.01% |  | 0.04% |  |  | -0.05% |  | 101% |  |  | 100% |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | #VALUE! |  | #VALUE! |  |  | #VALUE! |  | #DIV/0! |  |  | #DIV/0! |  |  |
| Class D | -0.01% |  | -0.06% |  |  | 0.06% |  | 101% |  |  | 101% |  |  |
| Class F | #VALUE! |  | #VALUE! |  |  | #VALUE! |  | #DIV/0! |  |  | #DIV/0! |  |  |
| Class TGM | #VALUE! |  | #VALUE! |  |  | #VALUE! |  | #DIV/0! |  |  | #DIV/0! |  |  |

References

- K. Naser, T. Poirier, F. Le Léannec and G. Martin-Cocher, “EE2: Intra Template Matching”, JVET-V0130, April 2021.

- K. Naser, T. Poirier, F. Galpin and A. Robert, “E2-1.14: IntraTMP adaptation for camera-captured content”, JVET-AB0130, Oct. 2022.

- ECM-7.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Feb. 2022.

Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).