| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0088-v1 |
| --- | --- |

| Title: | AHG9: Generative Face Video SEI Message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bolin Chen, Jie Chen, Shurun Wang, Yan Ye, Shiqi Wang | Email: | chenbolin.chenboli@alibaba-inc.com jiechen.cj@alibaba-inc.com shurun.wsr@alibaba-inc.com yan.ye@alibaba-inc.com shiqwang@cityu.edu.hk |
| Source: | Alibaba and City University of Hong Kong |  |  |

_____________________________

- Abstract

This contribution proposes for a generative face video SEI messages. The proposed SEI has two functions: one is to reconstruct high-quality talking face video at ultra-low bitrate and the another is to manipulate the talking face video towards personalized characterization. As such, the proposed SEI is applicable to video conferencing, live entertainment, face animation and metaverse-related functionalities.

Introduction

Recently, deep generative models, especially for generative adversarial network (GAN)[1], have shown great potentials in face video compression[2]-[5]. Follwing the philosophy of early model-based coding[6]-[8], the generative compression codecs can economically model the input face video into the transmitted symbols (i.e., semantic parameters or facial landmarks) based on the analysis model, and subsequently the decoded symbols can greatly facilitate to reconstruct face images via the synthesis model. It should be mentioned that a traditional video codec such as the VVC or HEVC codec can be employed to compress the first picture (or a set of key pictures) of a talking face video sequence as the base picture for plentiful texture reference. Then, the subsequent face pictures can be effectively reconstructed based on such base pictures and motion parameters from the GAN models. Figure 1 illustrates the general block diagram of generative compression codec.

Figure1: General block diagram of generative compression codec.

In 1999, the first 3D Morphable Model (3DMM) model[9] was proposed for 3D face reconstruction. Based on such a parametric face model, the input picture can be projected into highly disentangled facial parameters (i.e., 3D head location, head rotation, head translation, eye blinking and mouth motion). As such, the disentangled 3DMM facial parameters enjoy the advantages of ultra-low bitrate, high flexibility and vivid reality in face reconstruction and interaction. In this contribution, we propose to carry these 3DMM parameters in the form of a new SEI message. This proposed SEI message can be used to animate the base pictures coded using VVC or HEVC, and to reconstruct high quality face video at extremely low bitrate. Figure 2 gives examples of how such SEI can be used to generate and manipulate talking face video using one or more images.

- Flowchart of generative face video SEI message

- Examples of generative face video SEI message for different types of base pictures (natural face or virtual face)

Figure2: Examples of how such SEI can be used to generate and manipulate talking face video using one or more images.

Proposal

In this section, we describe the proposed SEI message.

Proposed SEI message scope

Table 4 – Persistence scope of SEI messages (informative)

| SEI message | Persistence scope |
| --- | --- |
| … | … |
| Generative_face_video | The PU containing the SEI message |

Syntax of generative face video SEI message

| Generative_face_video ( payloadSize ) { | Descriptor |
| --- | --- |
| gfv_id | ue(v) |
| gfv_num_set_of_parameter | ue(v) |
| gfv_quantization_factor | ue(v) |
| gfv_head_location_present_flag | u(1) |
| gfv_head_rotation_present_flag | u(1) |
| gfv_head_translation_present_flag | u(1) |
| gfv_eye_blinking_present_flag | u(1) |
| gfv_mouth_motion_present_flag | u(1) |
| for( i=0; i< gfv_num_set_of_parameter;i++){ |  |
| if(gfv_head_location_present_flag==1){ |  |
| gfv_location[i] | ue(v) |
| } |  |
| if(gfv_head_rotation_present_flag==1){ |  |
| gfv_rotation_roll [i] | ue(v) |
| gfv_rotation_pitch[i] | ue(v) |
| gfv_rotation_ yaw[i] | ue(v) |
| } |  |
| if(gfv_head_translation_present_flag==1){ |  |
| gfv_translation_x[i] | ue(v) |
| gfv_translation_y[i] | ue(v) |
| gfv_translation_z[i] | ue(v) |
| } |  |
| if(gfv_eye_blinking_present_flag==1){ |  |
| gfv_eye[i] | ue(v) |
| } |  |
| if(gfv_mouth_motion_present_flag==1){ |  |
| gfv_mouth_para1[i] | ue(v) |
| gfv_mouth_para2[i] | ue(v) |
| gfv_mouth_para3[i] | ue(v) |
| gfv_mouth_para4[i] | ue(v) |
| gfv_mouth_para5[i] | ue(v) |
| gfv_mouth_para6[i] | ue(v) |
| } |  |
| } |  |
| } |  |

Semantics of generative face video SEI message

This SEI message carries face information parameters that may be used for face generative compression. More specifically, every SEI message has a base picture that isthe picuture contained in the same PU as the SEI message. Based on the base picture, the face information parameters in the SEI message can be used to reconstruct face pictures, and each set of the parameters in the SEI message is used to generate one face picture For these face information parameters in the SEI message, there are 5 corresponding face information present flags (i.e., gfv_head_location_present_flag, gfv_head_rotation_present_flag, gfv_head_translation_present_flag, gfv_eye_blinking_present_flag and gfv_mouth_motion_present_flag) to determine whether the related face information parameters (i.e., gfv_location[i], gfv_rotation_roll [i], gfv_rotation_pitch [i], gfv_rotation_yaw[i], gfv_translation_x [i], gfv_translation_y[i], gfv_translation_z[i], gfv_eye[i], gfv_mouth_para1[i], gfv_mouth_para2[i], gfv_mouth_para3[i], gfv_mouth_para4[i], gfv_mouth_para5[i] and gfv_mouth_para6[i]) are transmitted or not. If not, the corresponding face information parameters abstracted from the based picture are used to generate the subsequent face pictures..

gfv_id contains an identifying number that may be used to identify a generative face video filter. The value of gfv_id shall be in the range of 0 to 232 − 2, inclusive.

gfv_num_set_of_parameter specifies the number of parameter set in the SEI message. One set of parameters is used to generate one face picture.. The value of gfv_num_set_of _parameter shall be in the range of 0 to 210 , inclusive.

gfv_quantization_factor specifies quantization factor to process the face information paramter (i.e., gfv_location[i], gfv_rotation_roll [i], gfv_rotation_pitch [i], gfv_rotation_yaw[i], gfv_translation_x [i], gfv_translation_y[i], gfv_ translation_z[i], gfv_eye[i], gfv_mouth_para1[i], gfv_mouth_para2[i], gfv_mouth_para3[i], gfv_mouth_para4[i], gfv_mouth_para5[i] and gfv_mouth_para6[i]). The values of paramaters used for face generation are equal to the values of corresponding syntax elements divided by gfv_quantization_factor..

Note:For example, if the value of gfv_location[i] is 1234, and the value of gfv_quantization_factor is 10000, the parameter actually used for gfv_location[i] is 0.1234.

gfv_head_location_present_flag equal to 1 indicates gfv_location[i] is present. gfv_head_location_present_flag equal to 0 indicates gfv_location[i] is not present.

gfv_location[i], when i is not equal to 0, specifies the quantized residual corresponding to head location between the i-th face picture and (i-1)-th face picture  via gfv_quantization_factor in display order. When i is equal to 0, gfv_location[0] specifies the quantized head location parameter of 0-th face picture.

gfv_head_rotation_present_flag equal to 1 indicates  gfv_rotation_roll[i], gfv_rotation_pitch[i], gfv_rotation_ yaw[i]are present and gfv_head_rotation_flag equal to 0 indicates gfv_rotation_roll[i], gfv_rotation_pitch[i], gfv_rotation_ yaw[i]are not present.

gfv_rotation_roll[i], when i is not equal to 0, specifies the quantized residual corresponding to head rotation around the front-to-back axis (called roll) between the i-th face picture and (i-1)-th face picture via gfv_quantization_factor in display order. When i is equal to 0, gfv_rotation_roll[0] specifies the quantized front-to-back-axis head rotation parameter of 0-th face picture.

gfv_rotation_ pitch[i], when i is not equal to 0, specifies the quantized residual corresponding to head rotation around the side-to-side axis (called pitch) between the i-th face picture and (i-1)-th face picture via gfv_quantization_factorin display order. When i is equal to 0, gfv_rotation_pitch [0] specifies the quantized side-to-side-axis head rotation parameter of 0-th face picture.

gfv_rotation_yaw[i], when i is not equal to 0, specifies the quantized residual corresponding to head rotation around the vertical axis (called yaw) between the i-th face picture and (i-1)-th face picture via gfv_quantization_factor in display order. When i is equal to 0, gfv_rotation_yaw[0] specifies the quantized vertical-axis head rotation parameter of 0-th face picture.

gfv_head_translation_present_flag equal to 1 indicates gfv_translation_x[i], gfv_translation_y[i] and gfv_translation_z[i] are present and gfv__head_translation_flag equal to 0 indicates gfv_translation_x[i], gfv_translation_y[i] and gfv_translation_z[i] are not present.

gfv_translation_x[i], when i is not equal to 0, specifies the quantized residual corresponding to head translation around the x axis between the i-th face picture and (i-1)-th face picture via gfv_quantization_factor in display order. When i is equal to 0, gfv_translation_x[0] specifies the quantized x-axis head translation parameter of 0-th face picture.

gfv_translation_y[i], when i is not equal to 0, specifies the quantized residual corresponding to head translation around the y axis between the i-th face picture and (i-1)-th face picture via gfv_quantization_factor in display order. When i is equal to 0, gfv_translation_y[0] specifies the quantized y-axis head translation parameter of 0-th face picture.

gfv_translation_z[i], when i is not equal to 0, specifies the quantized residual corresponding to head translation around the z axis between the i-th face picture and (i-1)-th face picture via gfv_quantization_factor in display order. When i is equal to 0, gfv_translation_z[0] specifies the quantized z-axis head translation parameter from 0-th face picture.

gfv_eye_blinking_present_flag equal to 1 indicates gfv_eye[i] is present and gfv_eye_blinking_present_flag is equal to 0 indicates gfv_eye[i] is not present.

gfv_eye[i], when i is not equal to 0, specifies the quantized residual corresponding to eye blinking degree between the i-th face picture and (i-1)-th face picture via gfv_quantization_factor in display order. When i is equal to 0, gfv_eye[0] specifies the quantized eye blinking parameter of 0-th face picture.

gfv_mouth_motion_present_flag is equal to 1 indicates gfv_mouth_para1[i], gfv_mouth_para2[i], gfv_mouth_para3[i], gfv_mouth_para4[i], gfv_mouth_para5[i] and gfv_mouth_para6[i] are present and gfv_mouth_motion_present_flag  equal to 0 indicates gfv_mouth_para1[i], gfv_mouth_para2[i], gfv_mouth_para3[i], gfv_mouth_para4[i], gfv_mouth_para5[i] and gfv_mouth_para6[i] are  not present.

gfv_mouth_para1[i], when i is not equal to 0, specifies the quantized residual corresponding to mouth motion between the i-th face picture and (i-1)-th face picture via gfv_quantization_factor in display order. When i is equal to 0, gfv_mouth_para1[0] specifies the quantized mouth motion parameter of 0-th face picture.

gfv_mouth_para2[i], when i is not equal to 0, specifies the quantized residual corresponding to mouth motion between the i-th face picture and (i-1)-th face picture via gfv_quantization_factor in display order. When i is equal to 0, gfv_mouth_para2[0] specifies the quantized mouth motion parameter of 0-th face picture.

gfv_mouth_para3[i], when i is not equal to 0, specifies the quantized residual corresponding to mouth motion between the i-th face picture and (i-1)-th face picture via gfv_quantization_factor in display order. When i is equal to 0, gfv_mouth_para3[0] specifies the quantized mouth motion parameter of 0-th face picture.

gfv_mouth_para4[i], when i is not equal to 0, specifies the quantized residual corresponding to mouth motion between the i-th face picture and (i-1)-th face picture via gfv_quantization_factor in display order. When i is equal to 0, gfv_mouth_para4[0] specifies the quantized mouth motion parameter from 0-th face picture.

gfv_mouth_para5[i], when i is not equal to 0,  specifies the quantized residual corresponding to mouth motion between the i-th face picture and (i-1)-th face picture via gfv_quantization_factor in display order. When i is equal to 0, gfv_mouth_para5[0] specifies the quantized mouth motion parameter from 0-th face picture.

gfv_mouth_para6[i], when i is not equal to 0, specifies the quantized residual corresponding to mouth motion between the i-th face picture and (i-1)-th face picture via gfv_quantization_factor in display order. When i is equal to 0, gfv_mouth_para6[0] specifies the quantized mouth motion parameter from 0-th face picture.

Conclusion

A new generative face video SEI message is proposed in this contribution. This proposed SEI message can be used to animate the base pictures coded using VVC or HEVC, and to reconstruct high quality face video at extremely low bitrate. Therefore, the proposed SEI can be applicable to video conferencing, live entertainment, face animation and metaverse-related functionalities. It is suggested to consider the proposed SEI message in future version of VSEI.

References

- I. Goodfellow, J. Pouget-Abadie, M. Mirza, B. Xu, D. Warde-Farley, S. Ozair, A. Courville, and Y. Bengio, “Generative adversarial nets,” Advances in neural information processing systems, vol. 27, 2014.

- M. Oquab, P. Stock, O. Gafni, D. Haziza, T. Xu, P. Zhang, O. Celebi, Y. Hasson, P. Labatut, B. Bose-Kolanu, T. Peyronel, and C. Couprie, “Low bandwidth video-chat compression using deep generative models,” in IEEE/CVF Conference on Computer Vision and Pattern Recognition Workshop, 2021.

- G. Konuko, G. Valenzise, and S. Lathuili`ere, “Ultra-low bitrate video onferencing using deep image animation,” in 2021 IEEE International onference on Acoustics, Speech and Signal Processing (ICASSP), 2021, pp. 4210–4214.

- A. Tang, Y. Huang, J. Ling, Z. Zhang, Y. Zhang, R. Xie, and L. Song, “Generative compression for face video: A hybrid scheme,” in 2022 IEEE International Conference on Multimedia and Expo (ICME), 2022, pp. 1–6.

- B. Chen, Z. Wang, B. Li, R. Lin, S. Wang, and Y. Ye, “Beyond eypoint coding: Temporal evolution inference with compact feature representation for talking face video compression,” in Proceedings of the IEEE Data Compression Conference, 2022.

- D. Pearson and J. Robinson, “Visual communication at very low data rates,” Proceedings of the IEEE, vol. 73, no. 4, pp. 795–812, 1985.

- H. G. Musmann, M. H¨otter, and J. Ostermann, “Object-oriented analysis-synthesis coding of moving images,” Signal Processing: Image Communication, vol. 1, no. 2, pp. 117–138, 1989.

- Y. Nakaya, Y. Chuah, and H. Harashima, “Model-based/waveform hybrid coding for videotelephone images,” in [Proceedings] ICASSP 91: 1991 International Conference on Acoustics, Speech, and Signal Processing, 1991, pp. 2741–2744 vol.4.

- V. Blanz and T. Vetter, “A morphable model for the synthesis of 3d faces,” in 26th International Conference on Computer Graphics and Interactive Techniques, 1999, p. 187–194.

Patent rights declaration(s)

Alibaba and City University of Hong Kong may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).