| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0089-v2 |
| --- | --- |

| Title: | EE1-1.5: Combined intra and inter models for luma and chroma |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Du Liu, Jacob Ström, Mitra Damghanian, Per Wennersten, Kenneth Andersson | Email: | {du.liu, jacob.strom, mitra.damghanian, per.wennersten, kenneth.r.andersson } @ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution reports the results of EE1-1.5, where one luma model is used for intra and inter luma slices and one chroma model is used for intra and inter chroma slices. In Test 1.5.1, the training crosscheck is performed. In Test 1.5.2, the IPB idea is extended to the chroma model and the chroma model is retrained with AI and RA data. In Test 1.5.3, a chroma model without the IPB input is retrained with AI and RA data. Both Test 1.5.2 and Test 1.5.3 have only one luma model and one chroma model.

Test 1.5.2 The luma IPB model and the chroma IPB model. Worst case 677 kMAC/pixel (block-basis) and num. of para. 3.11M.

Integer:

- Vs. NNVC-3.0, BDR Y/U/V: 
RA -9.60%/-20.97%/-22.07, LDB -8.28%/-20.23/-21.44%, AI -7.39%/-18.28%/-20.26%

- Vs. NNVC-3.0 filter set #1, BDR Y/U/V: 
RA -0.13%/-0.30%/-2.18%, LDB 0.24%/-5.33%/-8.36%, AI 0.15%/2.36%/0.63%

Floating point:

- Vs. NNVC-3.0, BDR Y/U/V: 
RA -9.60%/-21.25%/-21.96%, LDB -8.28%/-20.86%/-20.62%, AI -7.39%/-18.17%/-19.99%

- Vs. NNVC-3.0 filter set #1, BDR Y/U/V: 
RA -0.14%/-0.66%/-1.95/, LDB 0.22%/-6.17%/-6.96%, AI 0.15%/2.41%/0.68%

Test 1.5.3 The luma IPB model and the chroma model without IPB. Worst case 673 kMAC/pixel (block-basis) and num. of para. 3.10M.

Integer:

- Vs. NNVC-3.0, BDR Y/U/V: 
RA -9.60%/-20.93%/-21.49%, LDB -8.28%/-20.55%/-21.14%, AI -7.39%/-18.34%/-19.94%

- Vs. NNVC-3.0 filter set #1, BDR Y/U/V: 
RA -0.13%/-0.24%/-1.41%, LDB 0.25%/-5.74%/-8.04%, AI 0.15%/2.30%/0.81%

Floating point:

- Vs. NNVC-3.0, BDR Y/U/V: 
RA -9.59%/-21.00%/-21.50%, LDB -8.25%/-20.78%/-21.45%, AI -7.39%/-18.21%/-19.93%

- Vs. NNVC-3.0 filter set #1, BDR Y/U/V: 
RA -0.13%/-0.29%/-1.36%, LDB -0.24%/-6.09%/-7.92%, AI 0.15%/2.38%/0.78%

V2 includes results for the training crosscheck of the inter luma model from stage 1.

# Introduction

In JVET-AA0111 [1] and NNVC-3.0 [2], four CNN models are used for in-loop filtering: One model for intra luma pictures, one model for inter luma pictures, one model for intra chroma pictures, and one model for inter chroma pictures. In JVET-AB0052 [3], a combined intra and inter luma model is proposed on top of the inter luma model from JVET-AA0111, where three models are needed: One model for luma pictures, one model for intra chroma pictures, and one model for inter chroma pictures. The luma model uses the block type I/P/B information as an additional input.

In this contribution, a combined intra and inter chroma model is tested, where only two models are needed: a luma model and a chroma model.

There are three tests in EE1-1.5:

•	Test 1.5.1 is the training crosscheck.

•	Test 1.5.2 tests the chroma model with the input IPB.

•	Test 1.5.3 tests the chroma model without the IPB input.

# Proposed method

## Structure of the model

The structure of the luma model is depicted in Figures 1, 2, and 3. Figure 1 shows the five inputs, which are the reconstructed luma samples (rec), the prediction luma samples (pred), the block boundary strength (bs), QP, and the block type (IPB). Figure 2 shows the k-th residual block. In this contribution, there are eight residual blocks. Figure 3 shows the last part of the network.

Figure 1. Head of luma network. The inputs are combined to form the input y to the next part of the network.

Figure 2. The k-th residual block (k=0..7). The output y of the head is fed into a first residual block with input z0=y, which also takes the inputs rec, pred, part, bs, qp, and IPB. The output z1 is then fed into another such residual block.

Figure 3. The output of the last residual block is fed into this last part of the network.

The chroma model has a similar structure as the luma model, where the main difference is that the inputs include reconstructed luma samples (recY), reconstructed chroma samples (recUV), predicted chroma samples (predUV), boundary strength (bsUV), QP, and the tested block type IPB. The residual blocks and the last part of the network are the same as the luma model.

## Complexity

The complexity measured in kMAC/sample and the number of parameters were calculated using the thop library [5]. version 0.0.31-200524190.

In both JVET-AA0111 and NNVC-3.0, four NN models are used.

In Test 1.5.2, the luma IPB model and the chroma IPB model is used. The total number of parameters is 3.13M and the total memory is 6.23×106 bytes in 16-bit integer, which are about half of those in JVET-AA0111 and NNVC-3.0 filter set #1.

In Test 1.5.3, the luma model IPB model is used, and the chroma model without IPB is used. The total number of parameters is 3.12M, which is slightly lower than Test 1.5.2, and the total memory is 6.21×106 bytes in 16-bit integer.

Table 1: Summary of complexity and performance

| Test/Luma model | Nbr. of models | kMAC/pix (YUV) worst case (frame-basis) | kMAC/pix (YUV) worst case (block-basis) | Num. of param. in total (106) | Total mem. in float (106 bytes) | Total mem. in int16 (106 bytes) | RA BDR-Y/U/V int16 (vs. NNVC-3.0) | RA BDR-Y/U/V int16 (vs. NNVC-3.0 filter set #1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NNVC-3.0 filter set #1 | 4 | 525 | 664 | 6.21 | 24.85 | 12.42 | -9.49%/ -20.71%/ -20.40% | 0 |
| Test 1.5.2: luma IPB + chroma IPB | 2 | 535 | 677 | 3.11 | 12.46 | 6.23 | -9.60%/ -20.97%/ -22.07% | -0.13%/ -0.30%/ -2.18% |
| Test 1.5.3: IPB luma + chroma no IPB | 2 | 532 | 673 | 3.10 | 12.42 | 6.21 | -9.60%/ -20.93%/ -21.49% | -0.13%/ -0.24%/ -1.41% |

The intra chroma model in NNVC-3.0 filter set #1 has a complexity of 110 kMAC/sample and 1.57M parameters. The inter chroma model in NNVC-3.0 filter set #1 has a complexity of 107 kMAC/sample and 1.56M parameters.

## Training

To train the model to fit to both intra and inter cases, both the DIV2K and the BVI-DVC datasets are used.

| Table 2. Network Information for NN-based video coding tool testing in training stage for the chroma model. |  |  |
| --- | --- | --- |
| Network Information in Training Stage |  |  |
| Mandatory | GPU Type | GPU: NVIDIA A100-PCIE-40GB |
|  | Framework: | PyTorch v1.9 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 400 |
|  | Batch size: | 32 |
|  | Loss function: | Weighted L1 and L2 |
|  | Training time: | 10 days (Test 1.5.2) 10 days (Test 1.5.3) |
|  | Training data information: | DIV2K, BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0, JVET-W-EE1-1.6, qp {17, 22, 27, 32, 37, 42} |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 256x256 |
|  | Learning rate: | 1e-4 and 1e-5 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Mini-batch selection process: |  |
|  | Other information: |  |

## Inference

The decoder will extract an input IPB patch the same size as the rec input, e.g., 144x144 samples. An IPB sample will be 0 if the corresponding rec sample belongs to a block that is intra-predicted, 0.5 if the block is uni-predicted and 1 for a bi-predicted block.

| Table 3. Network Information for NN-based video coding tool testing in inference stage for the chroma model. |  |  |
| --- | --- | --- |
| Network Information in Inference Stage |  |  |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | SADL [6] |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | See Table 1 |
|  | Parameter Precision (Bits) | Int16, fp32 |
|  | Memory Parameter (MB) | See Table 1 |
|  | Multiplay Accumulate (MAC) | See Table 1 |
| Optional |  |  |
|  | Total Conv. Layers |  |
|  | Total FC Layers |  |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 128x128, 256x256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

# Experimental result

The proposed models are tested on top of NCS-1.0 [7]. The results are the same as on top of NNVC-3.0. The parameter selection and residue scaling are kept unchanged, where the only change is the NN model and the corresponding inputs. The results are in comparison to the NNVC CTC [4].

The models are converted to SADL [6] in floating point and 16-bit integer. All the simulations are run with GCC 8.2.0. The anchors are NNVC-3.0 and NNVC-3.0 filter set #1.

## Test 1.5.2 luma IPB and chroma IPB

Int16 over NNVC-3.0

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-3.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -8.71% | -16.92% | -21.40% | 204% | 38004% |
| Class A2 | -9.88% | -20.12% | -16.70% | 193% | 36538% |
| Class B | -9.11% | -22.08% | -23.34% | 207% | 37435% |
| Class C | -10.66% | -23.26% | -24.99% | 181% | 36276% |
| Class E |  |  |  |  |  |
| Overall | -9.60% | -20.97% | -22.07% | 196% | 37054% |
| Class D | -12.74% | -23.71% | -27.40% | 169% | 32824% |
| Class F | -5.49% | -14.42% | -13.81% | 277% | 17658% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-3.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -7.33% | -19.08% | -20.29% | 191% | 34203% |
| Class C | -9.17% | -21.66% | -22.88% | 173% | 33079% |
| Class E | -8.67% | -16.07% | -23.31% | 345% | 26315% |
| Overall | -8.28% | -20.23% | -21.44% | 214% | 31678% |
| Class D | -10.99% | -22.07% | -26.59% | 170% | 31510% |
| Class F | -5.43% | -14.27% | -10.14% | 258% | 16735% |

|  | All intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-3.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -6.54% | -16.48% | -17.90% | 197% | 27707% |
| Class A2 | -6.71% | -19.10% | -15.10% | 158% | 21639% |
| Class B | -6.69% | -17.66% | -21.18% | 156% | 21504% |
| Class C | -7.42% | -18.79% | -21.75% | 140% | 16068% |
| Class E | -10.02% | -19.60% | -23.06% | 171% | 26015% |
| Overall | -7.39% | -18.28% | -20.06% | 161% | 21726% |
| Class D | -7.30% | -18.75% | -22.75% | 134% | 13891% |
| Class F | -5.18% | -14.97% | -14.43% | 131% | 18375% |

Int16 over NNVC-3.0 filter set #1

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-3.0_filterset1 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.19% | -1.27% | -3.40% | 102% | 104% |
| Class A2 | 0.39% | -0.01% | 0.89% | 104% | 106% |
| Class B | -0.08% | 1.35% | -2.22% | 108% | 101% |
| Class C | -0.84% | -1.87% | -3.51% | 111% | 110% |
| Class E |  |  |  |  |  |
| Overall | -0.13% | -0.30% | -2.18% | 107% | 105% |
| Class D | -1.48% | -0.03% | -4.40% | 110% | 112% |
| Class F | -0.52% | -3.19% | -3.65% | 111% | 119% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-3.0_filterset1 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.54% | -3.79% | -7.33% | 103% | 105% |
| Class C | -0.05% | -7.25% | -9.64% | 106% | 103% |
| Class E | 0.13% | 1.36% | -7.38% | 101% | 92% |
| Overall | 0.24% | -5.33% | -8.36% | 104% | 101% |
| Class D | -0.21% | -3.33% | -9.71% | 112% | 108% |
| Class F | -0.53% | -5.69% | -4.11% | 105% | 117% |

|  | All intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-3.0_filterset1 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.05% | 2.08% | 2.76% | 111% | 115% |
| Class A2 | 0.15% | 1.23% | 2.96% | 108% | 111% |
| Class B | 0.14% | 5.59% | 0.67% | 110% | 117% |
| Class C | 0.20% | -0.13% | 0.06% | 106% | 117% |
| Class E | 0.21% | 1.74% | -3.13% | 113% | 119% |
| Overall | 0.15% | 2.36% | 0.63% | 109% | 116% |
| Class D | 0.17% | 0.35% | -2.20% | 111% | 120% |
| Class F | 0.11% | -3.61% | -3.78% | 112% | 124% |

Float over NNVC-3.0

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-3.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -8.72% | -17.60% | -21.24% | 263% | 64767% |
| Class A2 | -9.92% | -20.45% | -16.61% | 246% | 60115% |
| Class B | -9.10% | -22.30% | -23.31% | 263% | 63392% |
| Class C | -10.65% | -23.28% | -24.82% | 220% | 59653% |
| Class E |  |  |  |  |  |
| Overall | -9.60% | -21.25% | -21.96% | 248% | 61979% |
| Class D | -12.71% | -23.95% | -27.37% | 208% | 53313% |
| Class F | -5.43% | -14.51% | -13.70% | 371% | 28460% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-3.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -7.36% | -19.73% | -19.50% | 244% | 58399% |
| Class C | -9.14% | -22.28% | -22.02% | 212% | 54025% |
| Class E | -8.64% | -17.04% | -22.56% | 490% | 44937% |
| Overall | -8.28% | -20.86% | -20.62% | 277% | 53295% |
| Class D | -11.08% | -23.50% | -26.35% | 207% | 50748% |
| Class F | -5.34% | -14.61% | -10.39% | 349% | 26645% |

|  | All intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-3.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -6.53% | -16.50% | -17.75% | 239% | 44686% |
| Class A2 | -6.70% | -19.26% | -15.23% | 179% | 35375% |
| Class B | -6.70% | -17.10% | -20.99% | 172% | 34641% |
| Class C | -7.42% | -18.84% | -21.74% | 155% | 27111% |
| Class E | -10.02% | -19.66% | -22.99% | 190% | 42917% |
| Overall | -7.39% | -18.17% | -19.99% | 182% | 35595% |
| Class D | -7.30% | -18.85% | -22.80% | 143% | 22956% |
| Class F | -5.18% | -14.97% | -14.39% | 139% | 29637% |

Float over NNVC-3.0 filter set #1

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-3.0_filterset1 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.15% | -1.97% | -3.14% | 106% | 106% |
| Class A2 | 0.35% | -0.45% | 0.92% | 104% | 101% |
| Class B | -0.05% | 1.06% | -1.99% | 108% | 103% |
| Class C | -0.84% | -2.00% | -3.14% | 109% | 108% |
| Class E |  |  |  |  |  |
| Overall | -0.14% | -0.66% | -1.95% | 107% | 104% |
| Class D | -1.46% | -0.50% | -4.31% | 107% | 107% |
| Class F | -0.47% | -3.41% | -3.43% | 109% | 114% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-3.0_filterset1 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.50% | -4.52% | -6.03% | 100% | 106% |
| Class C | -0.02% | -8.23% | -8.13% | 104% | 105% |
| Class E | 0.07% | -0.67% | -6.45% | 106% | 102% |
| Overall | 0.22% | -6.17% | -6.96% | 103% | 104% |
| Class D | -0.33% | -5.45% | -10.04% | 111% | 110% |
| Class F | -0.47% | -6.27% | -4.44% | 104% | 113% |

|  | All intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-3.0_filterset1 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.02% | 1.88% | 2.93% | 111% | 112% |
| Class A2 | 0.16% | 0.93% | 2.62% | 105% | 104% |
| Class B | 0.13% | 6.17% | 0.92% | 107% | 108% |
| Class C | 0.20% | -0.16% | 0.06% | 112% | 112% |
| Class E | 0.21% | 1.62% | -3.09% | 111% | 118% |
| Overall | 0.15% | 2.41% | 0.68% | 109% | 110% |
| Class D | 0.17% | 0.38% | -2.20% | 111% | 113% |
| Class F | 0.11% | -3.64% | -3.78% | 111% | 116% |

## Test 1.5.3 luma IPB and chroma no IPB

Int16 over NNVC-3.0

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-3.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -8.73% | -16.47% | -20.68% | 226% | 41638% |
| Class A2 | -9.89% | -20.17% | -16.07% | 211% | 40529% |
| Class B | -9.10% | -22.25% | -22.92% | 222% | 40392% |
| Class C | -10.66% | -23.19% | -24.35% | 188% | 36863% |
| Class E |  |  |  |  |  |
| Overall | -9.60% | -20.93% | -21.49% | 211% | 39686% |
| Class D | -12.75% | -24.00% | -27.26% | 173% | 32452% |
| Class F | -5.48% | -14.30% | -12.50% | 292% | 18429% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-3.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -7.37% | -19.69% | -20.45% | 215% | 37845% |
| Class C | -9.17% | -21.62% | -21.99% | 186% | 35686% |
| Class E | -8.59% | -17.10% | -19.72% | 396% | 30299% |
| Overall | -8.28% | -20.55% | -21.14% | 238% | 35104% |
| Class D | -10.99% | -24.67% | -27.82% | 180% | 32430% |
| Class F | -5.35% | -14.31% | -10.84% | 284% | 18129% |

|  | All intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-3.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -6.54% | -16.45% | -18.07% | 217% | 29465% |
| Class A2 | -6.70% | -19.06% | -14.98% | 166% | 23493% |
| Class B | -6.69% | -17.68% | -21.24% | 156% | 22214% |
| Class C | -7.43% | -18.91% | -21.74% | 158% | 16908% |
| Class E | -10.01% | -19.82% | -22.18% | 165% | 25831% |
| Overall | -7.39% | -18.34% | -19.94% | 168% | 22683% |
| Class D | -7.30% | -18.83% | -22.69% | 150% | 14988% |
| Class F | -5.18% | -15.17% | -13.89% | 140% | 19177% |

Int16 over NNVC-3.0 filter set #1

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-3.0_filterset1 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.18% | -0.65% | -2.50% | 114% | 114% |
| Class A2 | 0.38% | -0.04% | 1.71% | 114% | 118% |
| Class B | -0.06% | 1.14% | -1.63% | 116% | 109% |
| Class C | -0.84% | -1.79% | -2.68% | 115% | 112% |
| Class E |  |  |  |  |  |
| Overall | -0.13% | -0.24% | -1.41% | 115% | 112% |
| Class D | -1.49% | -0.48% | -4.27% | 113% | 110% |
| Class F | -0.51% | -3.04% | -2.17% | 117% | 124% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-3.0_filterset1 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.50% | -4.52% | -7.56% | 115% | 116% |
| Class C | -0.05% | -7.27% | -8.65% | 114% | 112% |
| Class E | 0.22% | 0.10% | -2.79% | 116% | 105% |
| Overall | 0.25% | -5.74% | -8.04% | 115% | 112% |
| Class D | -0.20% | -6.50% | -11.20% | 118% | 112% |
| Class F | -0.44% | -5.69% | -4.85% | 116% | 127% |

|  | All intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-3.0_filterset1 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.05% | 2.18% | 2.60% | 122% | 122% |
| Class A2 | 0.15% | 1.28% | 3.12% | 113% | 121% |
| Class B | 0.13% | 5.57% | 0.59% | 110% | 121% |
| Class C | 0.20% | -0.29% | 0.10% | 120% | 123% |
| Class E | 0.21% | 1.43% | -1.96% | 109% | 119% |
| Overall | 0.15% | 2.30% | 0.81% | 114% | 121% |
| Class D | 0.17% | 0.19% | -2.15% | 124% | 130% |
| Class F | 0.11% | -3.81% | -3.18% | 120% | 129% |

Float over NNVC-3.0

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-3.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -8.71% | -16.76% | -20.66% | 271% | 66848% |
| Class A2 | -9.90% | -20.27% | -16.18% | 253% | 63953% |
| Class B | -9.09% | -22.22% | -22.92% | 267% | 65563% |
| Class C | -10.64% | -23.20% | -24.35% | 219% | 59304% |
| Class E |  |  |  |  |  |
| Overall | -9.59% | -21.00% | -21.50% | 251% | 63762% |
| Class D | -12.73% | -24.15% | -27.42% | 204% | 52707% |
| Class F | -5.42% | -14.25% | -12.60% | 370% | 28318% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-3.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -7.35% | -20.01% | -20.92% | 272% | 64059% |
| Class C | -9.16% | -21.75% | -22.11% | 233% | 59530% |
| Class E | -8.57% | -17.07% | -19.98% | 540% | 52566% |
| Overall | -8.25% | -20.78% | -21.45% | 307% | 59497% |
| Class D | -11.07% | -25.83% | -27.85% | 219% | 54569% |
| Class F | -5.31% | -13.97% | -10.93% | 382% | 28998% |

|  | All intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-3.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -6.54% | -16.41% | -18.09% | 263% | 49043% |
| Class A2 | -6.69% | -19.18% | -15.12% | 195% | 38757% |
| Class B | -6.70% | -17.11% | -21.10% | 187% | 37804% |
| Class C | -7.43% | -18.98% | -21.78% | 163% | 28520% |
| Class E | -10.02% | -19.86% | -22.17% | 204% | 45835% |
| Overall | -7.39% | -18.21% | -19.93% | 196% | 38453% |
| Class D | -7.31% | -18.95% | -22.77% | 152% | 24520% |
| Class F | -5.17% | -15.18% | -13.91% | 144% | 31466% |

Float over NNVC-3.0 filter set #1

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-3.0_filterset1 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.17% | -0.82% | -2.42% | 109% | 109% |
| Class A2 | 0.37% | -0.14% | 1.48% | 107% | 107% |
| Class B | -0.04% | 1.19% | -1.47% | 110% | 106% |
| Class C | -0.82% | -1.87% | -2.57% | 108% | 108% |
| Class E |  |  |  |  |  |
| Overall | -0.13% | -0.29% | -1.36% | 109% | 107% |
| Class D | -1.48% | -0.80% | -4.46% | 105% | 106% |
| Class F | -0.46% | -3.10% | -2.19% | 108% | 113% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-3.0_filterset1 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.52% | -4.85% | -7.65% | 111% | 116% |
| Class C | -0.04% | -7.64% | -8.26% | 115% | 115% |
| Class E | 0.15% | -0.72% | -3.19% | 117% | 119% |
| Overall | 0.24% | -6.09% | -7.92% | 114% | 116% |
| Class D | -0.32% | -8.19% | -11.80% | 117% | 119% |
| Class F | -0.44% | -5.49% | -4.96% | 114% | 123% |

|  | All intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-3.0_filterset1 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.02% | 2.06% | 2.55% | 122% | 123% |
| Class A2 | 0.17% | 1.04% | 2.77% | 114% | 114% |
| Class B | 0.12% | 6.17% | 0.79% | 117% | 118% |
| Class C | 0.20% | -0.34% | 0.04% | 118% | 118% |
| Class E | 0.22% | 1.33% | -2.01% | 119% | 126% |
| Overall | 0.15% | 2.38% | 0.78% | 118% | 119% |
| Class D | 0.17% | 0.19% | -2.19% | 118% | 121% |
| Class F | 0.11% | -3.86% | -3.25% | 115% | 123% |

# Conclusions

This contribution contains two tests of the combined intra and inter model for chroma. It is reported that, under RA, Test 1.5.2 the performance of luma IPB and chroma IPB gives Y/U/V -9.60%/-20.97%/-22.07% with the integer model over NNVC-3.0, and Test 1.5.3 the performance of luma IPB and chroma without IPB gives Y/U/V -9.60%/-20.93%/-21.49% with the integer model.

The performance of the chroma model with IPB and chroma model without IPB are similar. One possible reason is that the chroma model uses luma samples as input, and the luma model has the IPB as an input. The benefit of using IPB for chroma is therefore not significant.

It is proposed to adopt Test 1.5.3 the luma IPB model and the chroma model without IPB into NNVC-3.0.

# References

[1] EE1-1.6: Deep In-Loop Filter With Fixed Point Implementation, JVET-AA0111, Y. Li, K. Zhang, J. Li, L. Zhang, H. Wang, M. Coban, A.M. Kotra, M. Karczewicz, F. Galpin, K. Andersson, J. Ström, D. Liu, R. Sjöberg, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 27th Meeting, by teleconference, 13–22 July 2022

[2] https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/NNVC-3.0

[3] EE1-1.5: One luma model with IPB and/or skip for filtering intra and inter luma slices, JVET-AB0052, D. Liu, J. Ström, M. Damghanian, P. Wennersten, K. Andersson, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 28th Meeting, Mainz, 21–28 Oct. 2022

[4] Common Test Conditions and evaluation procedures for neural network-based video coding technology, E. Alshina, R.-L. Liao, S. Liu, A. Segall, JVET-X2016, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 24th Meeting, by teleconference, 6–15 October 2021

[5] https://pypi.org/project/thop/

[6] AHG11: Small Ad-hoc Deep-Learning Library (SADL) update, JVET-AA0086, F. Galpin, T. Dumas, P. Bordes, E. François, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 27th Meeting, by teleconference, 13–22 July 2022

[7] https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/NCS-1.0

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).