| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0090 |
| --- | --- |

| Title: | AHG11: Neural Network-based Reference CU Quality Enhancement for Motion Compensation Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yanhan Chu Zhikui Wang Wen Zhang Shuai Li | Email: | yanhan_c@mail.sdu.edu.cn wangzhikui@hisense.com zhangwen12@hisense.com shuaili@sdu.edu.cn |
| Source: | Hisense Visual Technology Co.,Ltd. |  |  |

_____________________________

- Abstract

This contribution proposes to enhance the Coding Unit (CU) before motion compensation with neural network. The proposed method is mandatorily performed on CU with minimum size of 16×16. Experimental results demonstrate that, compared with VTM-11.0_nnvc-2.0, the proposed method achieves 0.86% BD-rate reductions for Y component on Class D under the LDP configuration.

Introduction

In this contribution, a neural network based motion compensation enhancement method is proposed. The proposed method imposes neural networks on CU before motion compensation. Currently, the proposed method is applied on the luma component of CUs with size of 64×64, 64×32, 32×64, 32×32, 16×64, 64×16, 32×16, 16×32, and 16×16.

Proposed method

Framework

In VVC, Motion compensation includes conventional motion compensation and affine motion compensation. Our proposed neural network needs to be integrated between motion estimation and motion compensation.

Fig. 1 (a) is the process of integrating neural network into conventional motion compensation while Fig. 1 (b) is the process of integrating the neural network into affine motion compensation. In the conventional motion compensation, the VVC standard method finds the reference CU from the current CU according to the MV. Since the fractional interpolation filter will reduce the image size, it is necessary to expand the boundary of the reference CU by 4 pixels and then perform DCTIF to ensure that the CU size is consistent before and after filtering. The residual CU is obtained by subtracting the filtered prediction CU from the original CU, then it is transformed and quantized. Our mothed is designed to input the padded image patch into neural network, and the output image patch is then interpolated and filtered. In the affine motion compensation, the VVC standard method is to split the current CU into several 4 × 4 sub-CUs, and calculate the MV of each sub-CU according to the 4-parameter or 6-parameter affine models. The reference sub-CUs will then be found through MV. By performing padding, fractional interpolation on the reference sub-CUs, the final predicted sub-CUs can be obtained. Finally, the predicted sub-CUs will be combined to gets the predicted CU. In our method, we first get the max region containing all the reference sub-CUs, and then pad the region and input it to the neural network. Then, based on the corresponding position of the output image patch, the padding sub-CUs can be filtered by DCTIF to get the predicted sub-CUs.

We trained three networks, one for the CU block with size 64×64, one for the CU block with size 64×32, 32×64, 32×32, and the last one for the CU block with size 16×64, 64×16, 32×16, 16×32, 16×16.

(a) Conventional motion compensation

(b)  Affine motion compensation

Fig. 1   motion compensation in VVC

Neural Network Architecture

Fig. 2 presents the structure of the proposed neural network. It takes the extended reference block as the input image patch. The input image patch is first fed into a convolutional layer to extract the feature maps. After that, the feature maps pass through 10 residual convolutional blocks (RCB) [1] and a convolutional layer. RCB is composed of two convolution layers and an activation layer, and its input adds to the output to form a residual connection. Finally, the neural network output is obtained by adding the input block and the learned residual information.

For all the convolutional layers in network, kernel size of 3×3 is used. The number of the feature maps in each convolutional layer except the last layer is set to 64. The last convolutional layer produces 1 feature map. ReLU is used as the activation function. The network structure is the same as that of JVET-Y0090[4].

Fig. 2  The structure of the proposed neural network.

Neural Network Training

In the training stage, PyTorch 1.8.0 was used. We trained three networks (namely Net1, Net2, and Net3) to enhance the quality of different CU blocks. Net1 for CU Blocks with size of 64×64, Net2 for CU Blocks with size of 64×32, 32×64, 32×32. Net3 for CU Blocks with size of 16×64, 64×16, 32×16, 16×32, 16×16. 650 videos choose from BVI-DVC [3] dataset were used to train the neural network. The video sequences were compressed by VTM-11.0_nnvc-2.0 with LDP configurations (QP=22). We used blocks with size of 64×64, 32×32, and 16×16 as input to train the Net1, Net2, Net3, respectively. The training information is summarized in Table 1 as suggested by JVET-X2016 [2].

Table 1. Training information.

| Training information |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | NVIDIA Geforce RTX 3090 |
|  | CPU Type | Intel(R) Core(TM) i9-10900X CPU @ 3.70GHz |
|  | Framework: | PyTorch 1.8.0 |
|  | Number of GPUs per Task: | 1 |
|  | Epoch: | 240 |
|  | Batch size: | 64 |
|  | Total Parameter Number: | 738.43k/model (3 models in total) |
|  | Parameter Precision (Bits): | 32 (F) |
|  | Training data information: | BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | QP =22 |
| Optional | Patch size | 64x64, 32x32, 16x16 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Loss function: | MSE |

Neural Network Information

In inference stage, PyTorch 1.8.0 was used for performing the proposed neural network based motion compensation enhancement in VTM-11.0_nnvc-2.0. The inference information is summarized in Table 2 as suggested by JVET-X2016 [2].

Table 2. Inference information.

| Inference information |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | N/A |
|  | CPU Type | Intel(R) Core(TM) i9-10900X CPU @ 3.70GHz |
|  | Framework: | PyTorch 1.8.0 |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 738.43k/model (3 models in total) |
|  | Parameter Precision (Bits) | 32 (F) |
|  | Memory Parameter (KB) | 2611K/model (3 models in total) |
| Optional | Total Conv. Layers | 22 |
|  | Total FC Layers | 0 |
|  | Batch size: | 1 block at a time |
|  | Patch size | Depending on the block (64x64, 32x32, 16x16) |

Results

The common test conditions defined in JVET-X2016 [2] were used to evaluate the proposed method. Due to the limited time, we only encode the first 32 frames of the video sequences in Class D. The experimental results on VTM-11.0_nnvc-2.0 under LDP configuration are shown in Table 3 from which we can see that an average 0.86% BD Rate reduction can be achieved for Y component.

Table 3. Experimental results on VTM-11.0_nnvc-2.0

| Class D | Low delay P Over VTM-11.0_nnvc-2.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate (Y) | BD-rate (U) | BD-rate (V) | Encoding Time | Decoding Time |
| BasketballPass | -1.09% | 1.72% | -0.28% | 69314% | 18650% |
| BQSquare | -1.28% | 1.66% | 0.51% | 62348% | 30851% |
| BlowingBubbles | -0.70% | 0.59% | 1.59% | 61608% | 26800% |
| RaceHorses | -0.38% | 1.81% | 0.13% | 54884% | 25098% |
| Overall | -0.86% | 1.44% | 0.44% | 57620% | 24264% |

Conclusions

This contribution presents a neural network based motion compensation enhancement method. Experimental results demonstrate that the proposed method achieves coding performance improvement compared with VTM-11.0_nnvc-2.0.

References

- Kaiming He, Xiangyu Zhang, Shaoqing Ren, and Jian Sun, “Deep residual learning for image recognition," in Proceedings of the IEEE conference on computer vision and pattern recognition, 2016, pp. 770-778.

- S. Liu, A. Segall, E. Alshina, and R.-L. Liao, “JVET common test conditions and evaluation procedures for neural network-based video coding technology”, JVET-X2016, 24th Meeting: by teleconference, 6 – 15 October 2021.

- Ma, Di, Fan Zhang, and David R. Bull. "BVI-DVC: A Training Database for Deep Video Compression." arXiv preprint arXiv:2003.13552 (2020).

- Changyue Ma , Ru-Ling Liao , Yan Ye, “AHG11: Neural Network Based Motion Compensation Enhancement for Video Coding”,JVET-Y0090, 25th Meeting, by teleconference, 12–21 January 2022.

Patent rights declaration(s)

Hisense Visual Technology Co.,Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).