| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0097-v2 |
| --- | --- |

| Title: | Non-EE2: SGPM combined with IntraTMP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheng Fang, Shuang Peng, Dong Jiang，Jucai Lin, Xue Zhang | Tel: Email: | jiang_dong@dahuatech.com lin_jucai@dahuatech.com |
| Source: | Dahua (ZheJiang Dahua Technology CO.,LTD.) |  |  |

_____________________________

- Abstract

This contribution introduces IntraTMP to the SGPM design. In this method, the two geometric blocks can not only use intra prediction modes, but also use IntraTMP mode. The proposed method was implemented on top of ECM-7.0, it reportedly provides {Y, U, V} BD-rate reduction with encoder and decoder runtimes as follows:

AI: -0.04%, -0.05%, -0.05%, 102% (EncT), 115% (DecT)

Introduction

In ECM-7.0[1], the Spatial GPM method was adopted to intra prediction. SGPM consists of one partition mode and two associated intra prediction modes for two geometric blocks. To express the necessary partition and prediction information more efficiently in the bit-stream, a candidate list is employed and only the candidate index is signaled in the bit-stream. Each candidate in the list can derive a combination of one partition mode and two intra prediction modes. The length of the candidate list of combinations is set equal to 16.

In SGPM, there are 26 partition modes. For each partition mode, an IPM list is derived for each geometric block. The IPM list size is 3. The two intra prediction modes are selected from the IPM list.

A one line template is used to generate this candidate list.  For each possible combination of one partition mode and two intra prediction modes from the IPM list, a prediction is generated for the template with the partitioning weight extended to the template. These combinations are ranked in ascending order of their SATD between the prediction and reconstruction of the template.

Proposed method

In this contribution, we introduce IntraTMP to the SGPM design. One of the geometric blocks can use IntraTMP to do the prediction, but we only apply IntraTMP to the geometric block which has more  corresponding template pixels, as shown in Fig.1.

SGPM combined with IntraTMP

Since the IntraTMP mode is added, the number of possible combination also increases by 1*3*26=78. These additional 78 combinations only join the template rank process, so the candidate list length is still 16. So no additional syntax elements are required.

Experimental results

The proposed method is tested on top of ECM-7.0 under the common test conditions[2].

Results of the proposed method under AI configuration

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.14% | 0.03% | 101% | 115% |
| Class A2 | -0.02% | 0.03% | 0.01% | 100% | 116% |
| Class B | -0.03% | -0.06% | -0.09% | 105% | 120% |
| Class C | -0.04% | -0.04% | -0.13% | 101% | 115% |
| Class E | -0.08% | -0.07% | -0.01% | 100% | 106% |
| Overall | -0.04% | -0.05% | -0.05% | 102% | 115% |
| Class D | 0.00% | -0.09% | -0.02% | 103% | 109% |
| Class F | -0.07% | 0.05% | 0.03% | 101% | 109% |

Conclusions

This contribution introduces IntraTMP to the SGPM design. The results show that the proposed method can bring more coding gains.

References

- ECM-7.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-7.0.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

Patent rights declaration(s)

Dahua Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).