| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0098-v1 |
| --- | --- |

| Title: | EE2-1.8: Location-dependent DIMD |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Saverio Blasi Jani Lainema | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This document reports on experiment EE2-1.8 on Location-dependent Decoder Side Intra Mode Derivation (DIMD).

# Introduction

In test EE2-1.8 [1], sample-based blending of DIMD predictors is tested. When using Location-dependent DIMD [2], usage of sample-based blending is inferred during the DIMD mode derivation process, by analysing the histograms computed above and on the left of the current block. The weights used to blend the DIMD predictors are derived based on the histogram calculation process. In case sample-based blending is not used, conventional DIMD blending is used to blend the DIMD predictors.

# Experimental results (proposed method)

The tables below summarize the coding efficiency and runtime impact of the proposed method in ECM 7.0 using the JVET common test conditions.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.17% | -0.16% | 101% | 102% |
| Class A2 | -0.03% | -0.02% | -0.03% | 101% | 101% |
| Class B | -0.04% | -0.04% | -0.14% | 100% | 98% |
| Class C | -0.05% | -0.06% | -0.08% | 100% | 97% |
| Class E | -0.08% | -0.17% | -0.09% | 100% | 103% |
| Overall | -0.05% | -0.08% | -0.10% | 100% | 100% |
| Class D | -0.05% | -0.16% | -0.07% | 100% | 102% |
| Class F | -0.02% | 0.10% | -0.19% | 101% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.26% | -0.08% | 99% | 99% |
| Class A2 | -0.02% | 0.17% | -0.04% | 100% | 99% |
| Class B | -0.02% | -0.08% | -0.20% | 100% | 99% |
| Class C | 0.02% | 0.00% | 0.06% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.04% | -0.08% | 100% | 99% |
| Class D | -0.03% | -0.19% | -0.02% | 100% | 99% |
| Class F | -0.07% | -0.15% | -0.15% | 100% | 98% |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] V. Seregin, J. Chen etc, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2)”, JVET-AB2024, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 28th Meeting: Mainz, DE, 20–28 October 2022.

[2] S. Blasi, J. Lainema, “AHG12 - Location-dependent Decoder-side Intra Mode Derivation”, JVET-AB0116, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 28th Meeting: Mainz, DE, 20–28 October 2022.