| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0104-v2 |
| --- | --- |

| Title: | EE2-3.3: Block Vector Difference Prediction for IBC blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Filippov, V. Rufitskiy, | Tel: Email: | afilippov@ofinno.com vrufitskiy@ofinno.com |
| Source: | Ofinno LLC |  |  |

_____________________________

# Abstract

This contribution presents a technique to predict signs and magnitudes of x- and y-components of block vector differences (BVDs) for IBC blocks. In particular, BVD signs and suffix bins of exponential Golomb code used to represent BVD magnitudes are predicted by estimating template matching cost of candidate blocks to take advantage of regular CABAC mode rather than its by-pass mode at the entropy coding stage. On top of ECM-7.0, the following BD-rate gain results are reportedly obtained for the sub-tests of EE2-3.3:

- Test EE2-3.3a: AI configuration: {-0.42%, -0.29%, -0.46%} for class F and {-1.14%, -1.11%, -1.13%} for class TGM; RA configuration: {-0.35%, -0.38%, -0.39%} for class F and {-0.71%, -0.72%, -0.64%} for class TGM; LDB configuration: {-0.11%, -0.56%, -1.32%} for class F and {-0.56%, -0.44%, -0.47%} for class TGM;

- Test EE2-3.3b: AI configuration: {-0.46%, -0.32%, -0.45%} for class F and {-1.30%, -1.28%, -1.25%} for class TGM; RA configuration: {-0.38%, -0.39%, -0.34%} for class F and {-0.85%, -0.89%, -0.76%} for class TGM; LDB configuration: {-0.34%, -0.13%, -0.91%} for class F and {-0.83%, -0.80%, -0.73%} for class TGM;

- Test EE2-3.3c: AI configuration: {-0.50%, -0.40%, -0.51%} for class F and {-1.38%, -1.36%, -1.34%} for class TGM; RA configuration: {-0.43%, -0.66%, -0.35%} for class F and {-0.96%, -0.94%, -0.87%} for class TGM;

- Test EE2-3.3d: AI configuration: {-0.51%, -0.48%, -0.67%} for class F and {-1.42%, -1.39%, -1.42%} for class TGM; RA configuration: {-0.41%, -0.43%, -0.28%} for class F and {-0.94%, -0.95%, -0.82%} for class TGM.

- Test EE2-3.3e: AI configuration: {-0.85%, -0.73%, -0.99%} for class F and {-3.37%, -3.50%, -4.05%} for class TGM; RA configuration: {-0.55%, -0.67%, -0.87%} for class F and {-1.48%, -1.48%, -1.61%} for class TGM.

- Test EE2-3.3f: AI configuration: {-0.81%, -0.65%, -0.92%} for class F and {-3.36%, -3.42%, -3.98%} for class TGM; RA configuration: {-0.60%, -0.82%, -0.81%} for class F and {-1.44%, -1.36%, -1.49%} for class TGM.

# Introduction

This contribution presents results of test EE2-3.3 [1] that aims at predicting BVD sign and magnitude suffix bins that are coded using by-pass mode in ECM-7.0. The technique investigated in this EE2 test was originally proposed in [2] and includes 4 sub-tests that differ from each other by the maximum number of BVD bins available for prediction. Besides, results for a combination of test EE2-3.1 and test EE2-3.3a are reported [3].

# 2	Proposed methods

We proposed to apply sign prediction to BVD and further extend this approach for predicting suffix bins of BVD magnitudes. Suffix bins are also derived at the decoder side by comparing values of signalled bins with the bins of the best candidates obtained with template matching. The maximum number of bins to be predicted for a PU is controlled by a macro. By setting this macro, we investigate 4 configurations that corresponds the 4 sub-tests where the following number of BVD bins are predicted:

- Test EE2-3.3a: up to 2 BVD signs and 4 BVD suffix bins;

- Test EE2-3.3b: up to 2 BVD signs and 6 BVD suffix bins;

- Test EE2-3.3c: up to 2 BVD signs and 8 BVD suffix bins;

- Test EE2-3.3d: up to 2 BVD signs and 10 BVD suffix bins.

The most significant bins of magnitude suffixes of BVD horizontal and vertical components are predicted, and the prediction match result is coded in the bitstream using CABAC context mode. The less significant bins of magnitude suffixes of horizontal and vertical BVD components are coded in by-pass mode.

Besides, results for tests combinations are reported:

- Test EE2-3.3e: Test EE2-3.3a and Test EE2-3.1a;

- Test EE2-3.3f: Test EE2-3.3a and Test EE2-3.1b;

# Simulation results

The proposed method was implemented on top of ECM-7.0 and are tested under CTC for enhanced compression tool testing [4] with All-Intra, Random-Access, and Low-Delay B configurations. The simulation results of tests EE2-3.3a-f are shown in Tables 1-6, respectively.

Table 1. The simulation results of Test EE2-3.3a

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.42% | -0.29% | -0.46% | 100% | 101% |
| Class TGM | -1.14% | -1.11% | -1.13% | 100% | 102% |
| Overall | -0.78% | -0.70% | -0.79% | 100% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.35% | -0.38% | -0.39% | 100% | 100% |
| Class TGM | -0.71% | -0.72% | -0.64% | 100% | 101% |
| Overall | -0.53% | -0.55% | -0.52% | 100% | 100% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.11% | -0.56% | -1.32% | 100% | 100% |
| Class TGM | -0.56% | -0.44% | -0.47% | 101% | 101% |
| Overall | -0.33% | -0.50% | -0.89% | 101% | 101% |

Table 2. The simulation results of Test EE2-3.3b

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.46% | -0.32% | -0.45% | 101% | 105% |
| Class TGM | -1.30% | -1.28% | -1.25% | 101% | 114% |
| Overall | -0.88% | -0.80% | -0.85% | 101% | 109% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.38% | -0.39% | -0.34% | 101% | 101% |
| Class TGM | -0.85% | -0.89% | -0.76% | 101% | 105% |
| Overall | -0.61% | -0.64% | -0.55% | 101% | 103% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.34% | -0.13% | -0.91% | 100% | 101% |
| Class TGM | -0.83% | -0.80% | -0.73% | 102% | 102% |
| Overall | -0.58% | -0.47% | -0.82% | 101% | 101% |

Table 3. The simulation results of Test EE2-3.3c

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.50% | -0.40% | -0.51% | 101% | 108% |
| Class TGM | -1.38% | -1.36% | -1.34% | 102% | 134% |
| Overall | -0.94% | -0.88% | -0.92% | 101% | 120% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.43% | -0.66% | -0.35% | 101% | 102% |
| Class TGM | -0.96% | -0.94% | -0.87% | 102% | 107% |
| Overall | -0.70% | -0.80% | -0.61% | 101% | 105% |

Table 4. The simulation results of Test EE2-3.3d

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.51% | -0.48% | -0.67% | 101% | 112% |
| Class TGM | -1.42% | -1.39% | -1.42% | 101% | 151% |
| Overall | -0.96% | -0.93% | -1.05% | 101% | 130% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.41% | -0.43% | -0.28% | 101% | 104% |
| Class TGM | -0.94% | -0.95% | -0.82% | 101% | 111% |
| Overall | -0.67% | -0.69% | -0.55% | 101% | 107% |

Table 5. The simulation results of Test EE2-3.3e, a combination of Test EE2-3.3a and Test EE2-3.1a

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.85% | -0.73% | -0.99% | 100% | 101% |
| Class TGM | -3.37% | -3.50% | -4.05% | 100% | 102% |
| Overall | -2.11% | -2.11% | -2.52% | 100% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.55% | -0.67% | -0.87% | 100% | 100% |
| Class TGM | -1.48% | -1.48% | -1.61% | 100% | 101% |
| Overall | -1.01% | -1.08% | -1.24% | 100% | 101% |

Table 6. The simulation results of Test EE2-3.3f, a combination of Test EE2-3.3a and Test EE2-3.1b

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.81% | -0.65% | -0.92% | 100% | 101% |
| Class TGM | -3.36% | -3.42% | -3.98% | 101% | 102% |
| Overall | -2.09% | -2.04% | -2.45% | 101% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.60% | -0.82% | -0.81% | 100% | 100% |
| Class TGM | -1.44% | -1.36% | -1.49% | 100% | 101% |
| Overall | -1.02% | -1.09% | -1.15% | 100% | 100% |

# Conclusion

This contribution reports the simulation results of test EE2-3.3. The experimental results show significant bitrate saving with different gain/complexity trade-offs. In particular, the BD-rate gain for test EE2-3.3a is achieved with no encoding run-time increase and with a negligible growth of decoding run-time. For tests EE2-3.3b-d, the encoding run-time increase is negligible whereas decoding run-time can be further reduced (e.g., by using SIMD optimizations). Thus, we recommend adopting the proposed technique into the next ECM release.

# References

- V. Seregin et al., “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” JVET-AB2024, October 2022.

- A. Filippov and V. Rufitskiy, “Non-EE2: Block Vector Difference Prediction for IBC blocks,” JVET-AB0170, October 2022.

- J.-Y. Huo, X. Hao, Y.-Z. Ma, F.-Z. Yang (Xidian Univ.), M. Li, L. Zhang, J. Ren (OPPO), “EE2-3.1: Direct block vector mode for chroma prediction,” JVET-AC0071, January 2023

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Feb. 2022.

# Patent rights declaration(s)

Ofinno LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).