| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0105-v2 |
| --- | --- |

| Title: | EE2-1.2: Improvements on planar horizontal and planar vertical mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kyungyong (Kyle) Kim Dongcheol Kim Ju-Hyung (John) Son Jin Sam Kwak | Email: | kyungyong.kim@wilusgroup.com |
| Source: | WILUS Inc. |  |  |

_____________________________

- Abstract

This contribution reports EE2-1.2 test results of two new additional planar modes. The two new additional planar modes were first proposed in JVET-AA0104 [1] and was further investigated in JVET-AB0110 [2]. In Test 1.2, a derivation method of a transform kernel for planar horizontal and planar vertical mode is introduced. If an intra prediction mode of a current block is the planar horizontal/vertical mode, the vertical/horizontal intra prediction mode is used to derive a transform kernel in MTS set and LFNST set.

On top of ECM-7.0, simulation results are reported as follows:

AI: {-0.08%, -0.02%, -0.02%, 102%, 99%}

RA: {-0.03%, -0.02%, -0.06%, 100%, 102%}

Introduction

In JVET-AA0104 [1], two new additional planar modes are proposed where only the horizontal interpolation or only the vertical interpolation are used to obtain the predicted samples.

For planar horizontal mode, only the horizontal linear interpolation is performed based on the left reference sample and the top-right reference sample to predict the current sample as:

predx,y=W-1-x*rec-1,y+x+1*recW,-1+(W≫1)≫log2W

For planar vertical mode, only the vertical linear interpolation is performed based on the above reference sample and the bottom-left reference sample to predict the current sample as:

predx,y=H-1-y*recx,-1+y+1*rec-1,H+(H≫1)≫log2H

Proposal

In Test 1.2, a derivation method of a transform kernel for planar horizontal and planar vertical mode [2] as shown in Fig. 1 is tested. If an intra prediction mode of a current block is the planar vertical mode, the horizontal intra prediction mode is used to derive a transform kernel in MTS set and LFNST set. Also, if an intra prediction mode of a current block is the planar horizontal mode, the vertical intra prediction mode is used to derive a transform kernel in MTS set and LFNST set.

Figure 1. The proposed setting method of the intra prediction mode used to derive a transform kernel in MTS set and LFNST set

In Test 1.2, encoder-side optimization method is implemented to reduce complexity of additional planar modes.

Test Result

The proposed combination methods were implemented on top of the ECM-7.0 [3] software and tested based on the common test condition as specified in the Exploration Experiments 2 document (JVET-AB2024 [4]). The test results are summarized below.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.12% | 0.01% | 100% | 99% |
| Class A2 | -0.07% | -0.01% | 0.01% | 101% | 99% |
| Class B | -0.06% | -0.05% | -0.08% | 103% | 99% |
| Class C | -0.06% | 0.04% | 0.00% | 104% | 100% |
| Class E | -0.13% | 0.05% | -0.03% | 102% | 99% |
| Overall | -0.08% | -0.02% | -0.02% | 102% | 99% |
| Class D | -0.05% | -0.16% | -0.03% | 104% | 99% |
| Class F | -0.13% | 0.09% | 0.09% | 101% | 99% |
| Class TGM | -0.01% | 0.03% | 0.00% | 101% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.11% | 0.03% | 100% | 102% |
| Class A2 | -0.01% | 0.18% | -0.07% | 100% | 102% |
| Class B | -0.03% | -0.06% | -0.16% | 100% | 102% |
| Class C | -0.02% | -0.04% | -0.01% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.02% | -0.06% | 100% | 102% |
| Class D | -0.09% | 0.11% | 0.14% | 102% | 102% |
| Class F | -0.05% | -0.01% | 0.04% | 98% | 102% |
| Class TGM | -0.01% | -0.02% | 0.15% | 98% | 101% |

Reference

[1] X. Li, R.L. Liao, J. Chen and Y. Ye (Alibaba), “Non-EE2: On planar horizontal mode and planar vertical mode”, JVET-AA0104, July 2022.

[2] K.Y. Kim, and et al., “EE2-1.15-related: Improvements on planar horizontal and planar vertical mode,” JVET-AB0110, October 2022.

[3] M. Coban, and et al., “Algorithm description of Enhanced Compression Model 7 (ECM 7),” JVET-AB2025, October 2022.

[4] M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, January 2022.

Patent rights declaration(s)

WILUS Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).