| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0107-v1 |
| --- | --- |

| Title: | AHG12: Fusion of Intra Template Matching |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jeeva Raj Arumugam, Ashwin Natesan, Vaibhav Valvaiker, Jay N. Shingala (Ittiam) Taoran Lu, Fangjun Pu, Peng Yin, Tong Shao, Arjun Arora, Sean McCarthy (Dolby) | Email: | {jeeva.raj, jay.shingala }@ittiam.com {tlu, fpu, pyin}@dolby.com |
| Source: | Ittiam Systems, Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes adaptive intra template fusion to further improve the coding gain of intra template matching. The fusion techniques are based on fusion of TIMD: the two searched templates from intra TM are adaptively fused with weights decided by TM cost.

The method was implemented on top of ECM 7.0. The Y, U, V BDRate, along with encoder and decoder runtime, are reported as follows:

AI: {-0.07%, -0.06%, -0.13%, 10X%,100%}

RA: {-0.0X%, -0.0X%, -0.XX%, 10X%,10X%}

# Introduction

Algorithmic descriptions of TIMD fusion and Intra TMP in ECM7.0 are presented below for quick reference.

Fusion for TIMD is adopted in ECM from JVET-W0123 [1]. For each intra prediction mode in MPMs, the SATD between the prediction and reconstruction samples of the template is calculated, as shown in Figure 1. The two intra prediction modes with the minimum SATD are selected as the TIMD modes. These two TIMD modes are fused with the adaptive weights.

The costs of the two selected modes are compared with a threshold. In the ECM 7, the cost factor of 2 is applied as follows:

costMode2 < 2*costMode1.

If this condition is true, the fusion is applied, otherwise only mode1 is used.

Weights of the modes are computed from their SATD costs as follows:

weight1 = costMode2/(costMode1+ costMode2)

weight2 = 1 - weight1

Figure 1 Template and its reference samples used in TIMD

In the previous JVET meeting, Intra template matching was shown to provide gain for camera-captured content [2]. Intra template matching prediction (Intra TMP) copies the best prediction block from the reconstructed part of the current frame, whose L-shaped template matches the current template. For a predefined search range, as shown in Figure 2, the encoder searches for the most similar template to the current template in a reconstructed part of the current frame and uses the corresponding block as a prediction block. The encoder then signals the usage of this mode, and the same prediction operation is performed at the decoder side.

Figure 2 Intra TM search area (R1: current CTU, R2: top-left CTU, R3: above CTU, R4: left CTU)

# Proposed methods

Current intra TMP uses the reconstructed data from the reference region of the current picture corresponding to the lowest TM cost as predicted data for the current CU. This contribution proposes adaptive intra TMP fusion with up to two prediction blocks corresponding to the best and second-best TM cost. It uses the similar technique as fusion for TIMD, as shown in Figure 3, where P1 is the prediction data corresponding to the best match with the lowest TM cost and P2 is the prediction data corresponding to the second-best match with the second-lowest TM cost. The fusion condition and the weights derivation are the same as in TIMD except that the cost factor used is 1.5 instead of 2.

Another condition for fusion using fused template cost is added as described below.

Fused template cost is computed as follows where fusion operation is same as used for P1 and P2

Fused Left template, Lf = fusion of L1 and L2

Fused Top template, Tf = fusion of T1 and T2

costLeft = SAD between L and Lf

costTop = SAD between T and Tf

costFusion = costLeft + costTop

Final fusion of P1 and P2 is performed only if costFusion < lowest TM cost corresponding to P1

Figure 3 Intra TMP fusion

# Simulation results

The proposal is implemented on top of ECM 7.0. The average BDrate gain of AI is {-0.07%, -0.06%, -0.13%}, with negligible encoder/decoder runtime increase.

Table 1 AI performance of proposal against ECM 7.0

|  | AI Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over ECM 7.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -0.01% | 0.00% | -0.08% | #DIV/0! | 102% |
| Class A2 | -0.10% | -0.08% | -0.03% | #DIV/0! | 100% |
| Class B | -0.10% | -0.06% | -0.20% | #DIV/0! | 100% |
| Class C | -0.02% | -0.03% | -0.12% | 100% | 99% |
| Class E | -0.14% | -0.16% | -0.20% | 101% | 101% |
| Overall | -0.07% | -0.06% | -0.13% | #DIV/0! | 100% |
| Class D | 0.01% | -0.13% | -0.07% | 100% | 97% |

Table 2 RA performance of proposal against ECM 7.0

|  | RA Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over ECM 7.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class A2 | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class B | -0.04% | -0.15% | -0.34% |  |  |
| Class C | 0.01% | -0.07% | 0.00% |  |  |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class D | -0.01% | -0.28% | -0.07% |  |  |

# Conclusion

This contribution proposes adaptive fusion for intra TMP. It is recommended to study the proposed method in the EE2 exploration.

# References

- JVET-W0123, EE2-related: Fusion for template-based intra mode derivation, K. Cao, N. Hu, V. Seregin, M. Karczewicz (Qualcomm), Y. Wang, K. Zhang, L. Zhang (ByteDance)

- JVET-AB0130, EE2-1.14: IntraTMP adaptation for camera-captured content, K, Naser, T. Poirier, F. Galpin, A. Robert (InterDigital)

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ittiam Systems may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).