| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0108-v1 |
| --- | --- |

| Title: | Non-EE2: Extend search area in Intra Template Matching Prediction(IntraTMP) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junyan Huo, Hongqing Du, Hongli Zhang, Zhenyao Zhang, Wenhan Qiao, Yanzhuo Ma, Fuzheng Yang, | Email: | jyhuo@mail.xidian.edu.cn hqdu@stu.xidian.edu.cn hlz@stu.xidian.edu.cn zhyzhang@stu.xidian.edu.cn whqiao@stu.xidian.edu.cn yzma@mail.xidian.edu.cn fzhyang@mail.xidian.edu.cn |
| Source: | Xidian University |  |  |

_____________________________

- Abstract

In ECM7.0, IntraTMP uses the template to search the most similar template in predefined search area and uses its corresponding reference block as the prediction block. IntraTMP divides the search area into four rectangular regions, i.e. R1, R2 , R3, and R4. This contribution proposes to modify two of the four rectangular regions which lets the adjacent neighboring region of the coding block being checked. On top of ECM-7.0, simulation results are reported as follows:

AI(Y/U/V): -0.13%/-0.13%/-0.18%, EncT 100%, DecT 100%

RA(Y/U/V): -0.xx%/-0.xx%/-0.xx%, EncT xx%, DecT xx%

Introduction

In ECM7.0[1], IntraTMP is enabled for both camera-captured videos and screen contents. With the predefined serach range in the current picture, IntraTMP uses the template to search the most similar template accrodding to SAD cost and takes its corresponding reference block as the prediction block. Specifically, the best matching template is first obtained in the predefined search regions by step size 2, then a refined search by step size 1 is performed around the best match in best region to obtain the final best matching template.

The predefined search range is divided into four rectangular regions as illustrated in Fig.1 below.

In Fig.1, the light gray area represents reconstructed samples in current picture, and the set of search points includes  R1, R2, R3, and R4 region, which is limited by searchRangeWidth, searchRangeHeight, template size and picture size. The search point represents the search position of the top-left pixel of the current block in the search area. In IntraTMP, searchRangeWidth and searchRangeHeight are set proportional to the block dimension and the contral factor is set equal to 5. The template size is 4.

Fig.1. The four rectangular search regions in IntraTMP

(color region: candidate search area of the left-top pixel of the coding block)

Proposed Methods

In this contribution, it is proposed to extend R3 and R4 regions due to the vertical upper side and horizontal left side of the current block have also been reconstrcted. It is assumed that the adjacent regions of the coding block has a higher spatial correlation with the coding block. Fig. 2 depicts the extended regions based on IntraTMP.

Fig.2 The extended regions based on IntraTMP

Experimental results

The proposed method is implemented on ECM-7.0, tests are performed following test conditions specified in JVET-Y2017[2]. Table 1 shows the test results in AI and RA configurations. More detail results can be found in the accompanying Excel tables. Note that the decoding time is reliable, and the encoding time is unreliable.

Table 1. Simulation results of the proposed method

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.00% | -0.11% | 96% | 99% |
| Class A2 | -0.22% | -0.33% | -0.31% | 101% | 100% |
| Class B | -0.14% | -0.12% | -0.23% | 101% | 102% |
| Class C | -0.06% | -0.06% | -0.03% | 96% | 100% |
| Class E | -0.24% | -0.18% | -0.26% | 110% | 100% |
| Overall | -0.13% | -0.13% | -0.18% | 100% | 100% |
| Class D | -0.19% | -0.54% | -0.27% | 85% | 99% |
| Class F | -0.10% | 0.21% | 0.04% | 108% | 102% |
| Class TGM | -0.09% | -0.02% | -0.08% | 99% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |

Conclusions

In this contribution, an extension of search area in IntraTMP is proposed. Compared to ECM-7.0, the contribution achieves BD-rate saving of {-0.13% (Y), -0.13% (U), -0.18% (V)} for the AI configuration and {x.xx% (Y), x.xx% (U), x.xx% (V)} for the RA configuration. The encoding and decoding time is similar to the anchor scheme. It is suggested to investigate in EE.

- 5 References

- ECM-7.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

- M. Karczewicz, et. al., “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, February 2022.

- 6 Patent rights declaration(s)

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).