| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0109-v1 |
| --- | --- |

| Title: | Non-EE2: Intra template matching (Intra TMP) based on linear filter model |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junyan Huo, Wenhan Qiao, Xue Hao, Zhenyao Zhang, Hongqing Du, Yanzhuo Ma, Fuzheng Yang, | Email: | jyhuo@mail.xidian.edu.cn whqiao@stu.xidian.edu.cn xhao@stu.xidian.edu.cn zhyzhang@stu.xidian.edu.cn hqdu@stu.xidian.edu.cn yzma@mail.xidian.edu.cn fzhyang@mail.xidian.edu.cn |
| Source: | Xidian University. |  |  |

_____________________________

- Abstract

In ECM7.0, Intra Template Matching Prediction (IntraTMP) is applied to camera-captured contents. This contribution proposes to apply a linear filter model for the prediction of Intra TMP (Intra TMP-FLM). The 6-tap linear filter consists of 5 spatial luma samples in the reference block and a bias term. Filter coefficients are derived for each block using the regression based the minimized MSE on samples between the reference template and current template. The test results on top of ECM-7.0 are as follows:

AI(Y/U/V): -0.12%/-0.14%/-0.18%, EncT 101%, DecT 101%

RA(Y/U/V): -0.xx%/-0.xx%/-0.xx%, EncT xx%, DecT xx%

Introduction

In ECM7.0[1], IntraTMP is enabled for both camera-captured videos and screen contents. With the predefined serach range in current picture, IntraTMP uses the current template to search the most similar template accrodding to SAD cost and takes its corresponding reference block as the prediction block. This contribution proposes to build a linear filter model between the reference template and current template and apply the linear model to reference block. Then the filtered reference block is used as the prediction block.

Proposed Methods

The proposed 6-tap filter consist of a 5-tap plus sign shape spatial component and a bias term. The input to the spatial 5-tap component of the filter consists of a center (C) sample in the reference block which is at corresponding locations with the sample in the current block to be predicted and its above/north (N), below/south (S), left/west (W) and right/east (E) neighbors as illustrated below.

Figure 1: Spatial part of the filter

The bias term B represents a scalar offset between the input and output and is set to middle luma value (512 for 10-bit content).

Output of the filter is calculated as follows:

predLumaVal = c0C + c1N + c2S + c3E + c4W + c5B

The filter coefficients ci are calculated by minimising the MSE between the reference template and current template, as shown in Fig2. The extensions to the area shown in blue area needed to support the “side samples” of the plus shaped spatial filter and are padded when in unavailable areas.

Figure 2: Reference area used to derive the filter coefficients

The MSE minimization is performed by calculating autocorrelation matrix for the reference template input and current template output. Autocorrelation matrix is LDL decomposed and the final filter coefficients are calculated using back-substitution.

Usage of the Intra TMP-FLM mode is signalled coded CU level flag. Specifically, Intra TMP-FLM is considered a sub-mode of Intra TMP. That is, Intra TMP-FLM flag is only signalled if Intra TMP flag is true.

Experimental results

The proposed method was implemented based on ECM-7.0 under common test condition [2]. Table 2 presents a summary of the results with all intra and random access configurations.

Table 2. Simulation results of the proposed method

|  |  |  | All Intra Main 10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.04% | 0.00% | 93% | 100% |
| Class A2 | -0.17% | -0.17% | -0.17% | 100% | 101% |
| Class B | -0.15% | -0.15% | -0.32% | 104% | 101% |
| Class C | -0.04% | -0.03% | -0.05% | 101% | 100% |
| Class E | -0.24% | -0.35% | -0.30% | 104% | 101% |
| Overall | -0.12% | -0.14% | -0.18% | 101% | 101% |
| Class D | -0.01% | -0.23% | -0.04% | 93% | 100% |
| Class F | -0.34% | -0.23% | -0.49% | 102% | 101% |
| Class TGM | -0.11% | -0.07% | -0.09% | 90% | 101% |

Table 2. Simulation results of the proposed method

|  |  |  | Random Access Main 10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Conclusions

In this contribution, a linear filter model based Intra TMP (Intra TMP-FLM) is propoesed. The performance of the proposed method is -0.12% (AI) and xxx% (RA) over ECM 7.0. It is suggested to investigate in EE.

- 5 References

- ECM-7.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-X2017, Oct. 2021.

- 6 Patent rights declaration(s)

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).