| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0110-v1 |
| --- | --- |

| Title: | Non-EE2: A Fusion method of Intra Template Matching Prediction(Intra TMP) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junyan Huo, Hongqing Du, Hongli Zhang, Wenhan Qiao, Yanzhuo Ma, Fuzheng Yang | Email: | jyhuo@mail.xidian.edu.cn hqdu@stu.xidian.edu.cn hlz@stu.xidian.edu.cn whqiao@stu.xidian.edu.cn yzma@mail.xidian.edu.cn fzhyang@mail.xidian.edu.cn |
| Source: | Xidian University |  |  |

_____________________________

- Abstract

In ECM7.0, Intra Template Matching Prediction(IntraTMP) is applied to camera-captured contents. This contribution proposes a IntraTMP Fusion method that blends several reference blocks to derive the final prediction block. These reference blocks are obtained by Block Vectors(BVs) in template matching search process. The test results on top of ECM-7.0 are as follows:

AI(Y/U/V): -0.11%/-0.11%/-0.16%, EncT 104%, DecT 102%

RA(Y/U/V): -0.xx%/-0.xx%/-0.xx%, EncT xx%, DecT xx%

Introduction

In ECM7.0[1], IntraTMP is enabled for both camera-captured videos and screen contents. With the predefined serach range in current picture, IntraTMP uses the current template to search the most similar template accrodding to SAD cost and takes its corresponding reference block as the prediction block. Specifically, the best matching template is first obtained in the predefined search regions with step size 2, and then a refined search with step size 1 is performed around the best match to obtain the final best matching template. The offset between the best matching template and the current template is Block Vector(BV).

Proposed Methods

In this contribution, a IntraTMP Fusion method is proposed. Fig. 1 depicts the procedure of IntraTMP Fusion.

Fig.1 The procedure of IntraTMP Fusion

First, the BVs corresponding to 5 matching templates are searched with step size 2; then, the 5 candidate reference blocks corresponding to the 5 candidate matching templates are obtained through BVs; last, the candidate reference blocks are fused according to weights as the prediction of the current block.

The fusion method is proposed as follows:

predSamples=n=04wn*refBlockn+w5*midValue

where predSamples represent the final predicted block, refBlockn is one of the 5 candidate reference blocks. For 10-bit content, midValue is 512. The weights wn are derived from candidate matching templates and current template using LDL decomposition method used in CCCM.

In ECM7.0, usage of the IntraTMP is signalled with a CU level flag. Specifically, the new IntraTMP Fusion method is considered as a sub-mode of IntraTMP and is signalled with IntraTMP_Fusion_flag.

Experimental results

The proposed method was implemented based on ECM-7.0 under common test conditions [2]. The results of All Intra and Random Access configuration are summarized in the Table 1. More detail results can be found in the accompanying Excel tables.

Table 1. Simulation results of the proposed method

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.05% | -0.01% | 102% | 100% |
| Class A2 | -0.16% | -0.16% | -0.18% | 104% | 103% |
| Class B | -0.13% | -0.09% | -0.24% | 103% | 103% |
| Class C | -0.04% | 0.00% | -0.01% | 100% | 99% |
| Class E | -0.19% | -0.27% | -0.33% | 111% | 101% |
| Overall | -0.11% | -0.11% | -0.16% | 104% | 102% |
| Class D | 0.01% | -0.12% | 0.04% | 82% | 99% |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |

Conclusions

In this contribution, a new IntraTMP Fusion method is proposed. Compared to ECM-7.0, the contribution achieves BD-rate saving of {-0.11% (Y), -0.11% (U), -0.16% (V)} for the AI configuration and {0.xx% (Y), 0.xx% (U), 0.xx% (V)} for the RA configuration. It is suggested to investigate in EE.

- 5 References

- ECM-7.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

- M. Karczewicz, et. al., “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, February 2022.

- 6 Patent rights declaration(s)

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).