| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0114-v1 |
| --- | --- |

| Title: | AHG11: Deep Reference Frame Generation for Inter Prediction Enhancement |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianghao Jia, Yuantong Zhang, Han Zhu, Zhenzhong Chen Wuhan University Zizheng Liu, Xiaozhong Xu, Shan Liu Tencent | Email: | {jiajh2021, yuantongzhang, zhuhanlyx, zzchen}@whu.edu.cn {zizhengliu, xiaozhongxu, shanl} @tencent.com |
| Source: | Wuhan University, Tencent |  |  |

_____________________________

- Abstract

This contribution presents a deep reference frame generation (DRF) method for VVC inter prediction enhancement. During the encoding and decoding process, the network receives two reconstructed frames from the decoded picture buffer as inputs and generates a new frame. The generated frame will be put into the reference picture lists as an additional reference for the current encoding picture, which enhances the inter prediction. Different from the previous work JVET-AB0114, this contribution optimizes the network architecture and modifies the TMVP strategy for better performance and less complexity. It is reported that this method can achieve {LDB: -3.18%/-11.89%/-9.21%, RA: -4.32%/-10.71%/-10.07%} bitrate savings for Y/U/V components respectively based on VTM-11.0_nnvc-2.0.

Introduction

In VVC, the reconstructed pictures stored in decoded picture buffer (DPB) are organized as reference picture list (RPL) for current encoding picture. During inter prediction procedure, the current encoding frame will select frames from RPL as reference picture for the motion estimation. The performance of inter prediction is highly dependent on the content of reference frames. Reference frames with fewer compression artifacts or more relevant content can boost prediction performance, which will lead to smaller residual coding consumption in the bitstream. This motivates some methods to facilitate inter prediction by synthesizing better reference pictures.

Recently, deep neural network (DNN) has been widely used to solve challenging video tasks including video interpolation and prediction. A variety of researches have explored to devote the DNN to video coding. This contribution proposes a deep reference frame generation (DRF) method for VVC inter prediction enhancement. Specifically, a pair of previously encoded frames are processed by the reference frame synthesis network to generate a new frame which can be regarded as the prediction of current encoding frame. Subsequently, the generated frame is integrated into the reference picture list exploited by inter prediction.

This method has been presented in JVET-X0060 [1], JVET-Y0096 [2], JVET-AA0082 [3], and JVET-AB0114 [4] at previous meetings. Different from the previous work, there are several modifications in this contribution:

a) Optimize the network architecture with more efficiency and fewer parameters.

b) Modify the TMVP strategy while using DRF.

Proposed Method

Framework

Fig. 1: The frame work of the proposed DFR method

The framework of the proposed DRF method is shown in Fig. 1. In this method, two reconstructed frames selected from DPB together with the QP maps are fed into the deep reference frame generation network to synthesize a new frame. This generated frame will be consequently inserted to both Reference List0 and List1 with the same POC of the current encoding frame as an alternative reference. DRF can be used both for LDB and RA configuration with some differences in network architecture. It should be noted that the ME process is not modified, and no extra information signaled into the bitstream. Other details can be found in our previous work.

Network architecture

Fig. 2 shows the architecture of the proposed DFR network. The reconstructed frames and their corresponding QP maps are fed into a three-level hierarchical structure to extract features in different granularities. An additional module PFRB [5] is employed to further explore the relationship of the features. Besides, we utilize the IFRNet [6] to estimate the optical flows among the intermediate frame and the two input frames. The estimated optical flows will be also down sampled into the similar pyramid structure as the extracted features. The three levels of feature and optical-flows are synthesized by optical-flow-based calculation. Finally, the GridNet and ResBlocks will output the generated reference frame with the synthesized features as input.

Fig. 2: The architecture of the DRF network

Modification of TMVP

In VVC, TMVP is considered when establishing merge/AMVP candidate list or using SbTMVP. The motion vector prediction derived from collocated CU will be scaled according to the POC distances among current picture, collocated picture, and corresponding references. In our previous design, we skip the TMVP construction when the generated reference frame is used to avoid division by zero error.

In this contribution, we have made some modification to unify the TMVP construction when DRF is enabled as shown in Table 1. In general, TMVP will be leveraged without scaling when DRF is used for both current CU and collocated CU.

Table 1: Modification of TMVP when DRF is enabled

|  | Reference of the current CU is DRF | Reference of the collocated CU is DRF | Use TMVP | The derivation of TMVP |
| --- | --- | --- | --- | --- |
| 1 | × | × | √ | Scale the MV of collocated reference CU. |
| 2 | × | √ | × | - |
| 3 | √ | × | × | - |
| 4 | √ | √ | √ | Copy the MV of collocated reference CU. |

Network information in training stage

Table 2: Network information for the NN-based video coding tool testing in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GTX 1080ti x 8 x 12GB |
|  | Framework | PyTorch v1.9.0 |
|  | Number of GPUs per Task | 8 |
|  | Epoch | 200 |
|  | Batch size | 3Kx24 (RA) 4Kx18 (LDB) |
|  | Loss function | L1 |
|  | Training time (for 1 model) | 168h |
|  | Training data information | Vimeo (compressed) |
|  | Training configurations for generating compressed training data (if different to VTM CTC) | QP = 22, 27, 32, 37, 42 |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 256x256 |
|  | Learning rate | 1e-4 |
|  | Learning rate update strategy |  |
|  | Optimizer | ADAM |
|  | Preprocessing |  |
|  | Mini-batch selection process |  |
|  | Training data update strategy |  |
|  | Other information |  |

Network information in inference stage

Table 3: Network information for the NN-based video coding tool testing in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment |  |
|  | GPU Type | CPU only |
|  | Framework | PyTorch v1.9.0 |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 7797K (RA) 11512K (LDB) |
|  | Total Number of Parameters (All Models) | 19309K |
|  | Parameter Precision (Bits) | 32 |
|  | Memory Parameter (MB) | 29.8 (RA) 44.0 (LDB) |
|  | Multiply Accumulate (kMAC/pixel) | 329 (RA) 398(LDB) |
| Optional |  |  |
|  | Total Conv. Layers |  |
|  | Total FC Layers |  |
|  | Total Memory (MB) |  |
|  | Batch size |  |
|  | Patch size | 256x256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information |  |

Results

This method was implemented on the VTM-11.0_nnvc-2.0. Following the JVET NNVC CTC [7], we conducted the compression experiments to evaluate the performance of the proposed DFR method. Table 4 and Table 5 show the experimental results of the proposed DRF method compared to the anchor results in LDB and RA configurations, respectively. According to the experimental results, the proposed DRF method can achieve {-3.18%/-11.89%/-9.21%, -4.32%/-10.71%/-10.07%} coding efficiency gains for YUV components on LDB and RA configuration respectively.

Table 4: Performance of the proposed method (LDB, full length)

|  | Low delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | -2.57% | -9.73% | -6.96% | -3.18% | -10.50% | -9.35% | 179% | 132685% |
| Class C | -3.36% | -14.58% | -12.02% | -4.76% | -14.15% | -9.24% | 140% | 74601% |
| Class E | -3.94% | -6.88% | -2.96% | -4.68% | -4.86% | -1.40% | 307% | 238090% |
| Overall | -3.18% | -11.89% | -9.21% | -4.08% | -12.12% | -9.30% | 189% | 126749% |
| Class D | -3.59% | -8.85% | -9.22% | -4.42% | -1.46% | -3.97% | 129% | 49557% |

Table 5: Performance of the proposed method (RA, full length)

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -3.54% | -7.84% | -8.49% | -4.15% | -9.36% | -9.82% | 156% | 87495% |
| Class A2 | -4.74% | -12.00% | -7.80% | -5.49% | -12.90% | -8.08% | 148% | 80970% |
| Class B | -3.62% | -10.19% | -10.20% | -4.17% | -11.75% | -12.00% | 159% | 101732% |
| Class C | -5.46% | -12.55% | -12.79% | -6.87% | -12.70% | -13.96% | 124% | 61673% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -4.32% | -10.71% | -10.07% | -5.15% | -11.75% | -11.31% | 146% | 82522% |
| Class D | -7.15% | -12.43% | -13.85% | -6.53% | -13.01% | -13.66% | 113% | 41698% |

Conclusions

This contribution presents a deep reference frame generation (DRF) method for VVC inter prediction enhancement. By applying the proposed method, additional BD-rate saving can be achieved.

Reference

[1] Z. Liu, X. Xu, S. Liu, Y. Guo, Z. Chen. AHG11: NN-based Reference Frame Interpolation for VVC Hierarchical Coding Structure. JVET-X0060, 24th Meeting, by teleconference, 6–15 Oct. 2021.

[2] Z. Liu, X. Xu, S. Liu, Y. Guo, Z. Chen. AHG11: NN-based Reference Frame Interpolation for VVC Hierarchical Coding Structure. JVET-Y0096, 25th Meeting, by teleconference, 9–18 Jan. 2022.

[3] Z. Liu, X. Xu, S. Liu, J. Jia, Z. Chen. AHG11: Deep Reference Frame Generation for Inter Prediction Enhancement. JVET-AA0082, 27th Meeting, by teleconference, 13-22 July 2022.

[4] J. Jia, Y. Zhang, H. Zhu, Z. Chen, Z. Liu, L. Wang, X. Xu, S. Liu. AHG11: Deep Reference Frame Generation for Inter Prediction Enhancement. JVET-AB0114, 28th Meeting, Mainz, DE, 20–28 October 2022.

[5] P. Yi, Z. Wang, K. Jiang, J. Jiang, and J. Ma. Progressive fusion video super-resolution network via exploiting non-local spatio-temporal correlations. IEEE/CVF International Conference on Computer Vision, 2019, pp. 3106-3115.

[6] L. Kong, B. Jiang, D. Luo, W. Chu, X. Huang, Y. Tai, C. Wang, and J. Yang. IFRNet: Intermediate feature refine network for efficient frame interpolation. IEEE/CVF Conference on Computer Vision and Pattern Recognition, 2022, pp. 1959-1968.

[7] E. Alshina, R.-L. Liao, S. Liu, A. Segall. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-AA2016, 27th Meeting, by teleconference, 13-22 July 2022.

Patent rights declarations

Wuhan University does not have any current or pending patent rights relating to the technology described in this contribution.

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).