| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0117-v1 |
| --- | --- |

| Title: | Cross-check of EE2-1.13: CCCM using non-downsampled luma samples |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | T. Dumas | Tel: Email: | thierry.dumas@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution reports the cross-check of the exploration experiment EE2-1.13: "CCCM using non-downsampled luma samples". EE2-1.13 arises from the contribution JVET-AB0180. JVET-AB0180 proposed, in CCCM, to directly predict the chroma samples from the reconstructed luma samples, without downsampling.

It is confirmed that the provided source code conforms to the technical description of JVET-AB0180. The cross-checked rate-distortion results and encoding/decoding runtimes of EE2-1.13 are reported.

# Introduction

In JVET-AB0180, a new CCCM filter is proposed. This CCCM filter comprises 6-tap spatial terms, two nonlinear terms, and a bias term. The main difference between the regular CCCM filter and the one in JVET-AB0180 lies in the arrangement of the 6 neighboring luma samples to the current chroma sample to be predicted, each neighboring luma sample corresponding to a different term of the 6-tap spatial terms.

As for the regular CCCM, up to 6 rows/columns of chroma samples above and on the left side of the current pair of chroma CBs are used to derive the filter coefficients. The LDL decomposition used to derive the coefficients of the new CCCM filter is identical to the one used to derive the coefficients of the regular CCCM filter.

For the current pair of two chroma CBs, if CCCM is selected for prediction, a single flag is signaled to indicate whether the regular CCCM model or the CCCM model introduced in JVET-AB0180 applies.

# Cross-check report

The source code of EE2-1.13 has been carefully analyzed and it is confirmed it sticks to the technical description provided in JVET-AB0180. The source code has been run under JVET EE2 coding test conditions [1], as shown in Table 1 and Table 2.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.06% | -0.08% | 101% | 102% |
| Class A2 | -0.23% | -0.35% | -0.28% | 100% | 102% |
| Class B | 0.01% | -0.27% | -0.33% | 96% | 98% |
| Class C | -0.02% | -0.27% | -0.35% | 100% | 101% |
| Class E | 0.04% | 0.09% | -0.06% | 101% | 100% |
| Overall | -0.03% | -0.19% | -0.24% | 99% | 100% |
| Class D | 0.00% | -0.46% | -0.33% | 103% | 99% |
| Class F | -0.22% | -0.96% | -0.64% | 99% | 99% |
| Class TGM |  |  |  |  |  |

Table 1: EE2-1.13 with respect to ECM-7.0 in AI all frames.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | -0.16% | -0.22% | 101% | 96% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.01% | -0.09% | -0.34% | 103% | 102% |
| Class C | 0.00% | -0.31% | -0.21% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | -0.17% | -0.13% | 101% | 104% |
| Class F | -0.26% | -0.88% | -0.68% | 100% | 102% |
| Class TGM |  |  |  |  |  |

Table 2: EE2-1.13 with respect to ECM-7.0 in RA all frames.

# References

[1] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Interdigital does not have any current or pending patent rights relating to the technology described in this contribution.