| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0118-v2 |
| --- | --- |

| Title: | EE1-1.8: QP-based loss function design for NN-based in-loop filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal, Information |  |  |
| Author(s) or Contact(s): | Chuan Zhou Zhuoyi Lv Jinrong Zhang (vivo) | Email: | chuan.zhou@vivo.com zhuoyi.lv@vivo.com |
| Source: | vivo Mobile Communication Co., Ltd |  |  |

_____________________________

- Abstract

This contribution presents the EE test results of JVET-AB0141. In this test, a new loss function is defined where quantization parameter is used as a factor for the loss calculation in the training stage. The proposed CNN model is implemented on top of NNVC common software (NCS1.0) using SADL int16 precision.

In EE1-1.8.2, both the epochs of training luma and chroma model are 50. In EE1-1.8.2, the epochs of training luma and chroma model are 350 and 50. BD-rate changes of {Y, Cb, Cr} are summarized as follows:

EE1-1.8.1:

- Compared with NNVC-3.0,

AI: {-7.21%, -20.61%, -21.30%}, EncT 154%, DecT 31084%

- Compared with NNVC-3.0 filter set #1

AI: {0.34%, -0.60%, -0.93%}, EncT 106%, DecT 123%

EE1-1.8.2:

- Compared with NNVC-3.0,

AI: {-7.49%, -20.60%, -21.28%}, EncT 153%, DecT 31176%

- Compared with NNVC-3.0 filter set #1

AI: {0.04%, -0.60%, -0.91%}, EncT 106%, DecT 123%

Introduction

In NN-based in-loop filter, the commonly used loss functions, L1 and L2 have not considered the information related to video coding.

MAEx,y= i=1n|yi-f(xi)|n

MSEx,y= i=1n(yi-f(xi))2n

In EE1-1.8.2, the number of epochs is studied for training luma model.

Proposed method

Since the training samples with different quantization parameters have different training difficulties, the new designed loss function can be formulated as follows:

Loss= w* MAEx,y   or

Loss= w* MSEx,y

w= 1- 11+exp30-qp/6

where w denotes the weight assigned to the conventional loss function according to the quantization parameters. The following figure shows the function curve of w(qp).

Inference

In the inference stage, SADL [1] is used as the inference library. All the models are quantized into int16 precision.

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 1.56M/model, 4 models in total |
|  | Parameter Precision (Bits) | 16 (I) |
|  | Memory Parameter (MB) | 3.1MB/model, 4 models in total |
|  | Multiply Accumulate (kMAC/pixel) | 539K/pixel (assuming frame-level input); 682K/pixel (assuming block-level input) |
| Optional |  |  |
|  | Total Conv. Layers | 25+16 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 128×128, 256×256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage |  |
|  | Other information: |  |

Training

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: Tesla-A100 40GB |
|  | Framework: | PyTorch v1.11 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 1.8.1: 50 luma; 50 for chroma 1.8.2: 350 luma; 50 for chroma |
|  | Batch size: | 64 |
|  | Training time: | 1.8.1: ~33h for luma; ~12h for chroma 1.8.2: ~230h for luma; ~12h for chroma |
|  | Training data information: | DIV2K |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0 + new MCTF, QP {17, 22, 27, 32, 37, 42} |
|  | Loss function: | L1, L2 |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 128×128 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

Experimental results

The proposed method is tested on top of NCS1.0 under the common test conditions in [2].

Table 1. The performance of EE1-1.8.1 (Anchor is NNVC-3.0)

|  | All intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -6.32% | -17.91% | -19.40% | 198% | 40267% |
| Class A2 | -6.41% | -20.47% | -19.57% | 158% | 33014% |
| Class B | -6.41% | -22.10% | -22.19% | 149% | 31461% |
| Class C | -7.54% | -20.01% | -22.72% | 131% | 21492% |
| Class E | -9.82% | -21.77% | -21.55% | 154% | 36217% |
| Overall | -7.21% | -20.61% | -21.30% | 154% | 31084% |
| Class D | -7.52% | -20.51% | -22.58% | 125% | 17888% |
| Class F | -3.83% | -13.19% | -12.42% | 124% | 19397% |

Table 2. The performance of EE1-1.8.1 (Anchor is NNVC-3.0 filter set #1)

|  | All intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.30% | 0.15% | 0.68% | 110% | 123% |
| Class A2 | 0.47% | 0.01% | -2.48% | 107% | 124% |
| Class B | 0.45% | -0.21% | -0.65% | 106% | 122% |
| Class C | 0.08% | -1.68% | -1.11% | 104% | 123% |
| Class E | 0.42% | -1.18% | -1.23% | 107% | 123% |
| Overall | 0.34% | -0.60% | -0.93% | 106% | 123% |
| Class D | -0.07% | -1.85% | -1.99% | 103% | 123% |
| Class F | 1.56% | -1.57% | -1.66% | 104% | 92% |

Table 3. The performance of EE1-1.8.2 (Anchor is NNVC-3.0)

|  | All intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -6.59% | -17.84% | -19.37% | 197% | 40325% |
| Class A2 | -6.67% | -20.63% | -19.63% | 157% | 33004% |
| Class B | -6.69% | -22.06% | -22.18% | 148% | 31539% |
| Class C | -7.80% | -19.95% | -22.65% | 130% | 21672% |
| Class E | -10.12% | -21.77% | -21.54% | 154% | 36261% |
| Overall | -7.49% | -20.60% | -21.28% | 153% | 31176% |
| Class D | -7.69% | -20.22% | -22.45% | 125% | 18184% |
| Class F | -4.03% | -13.20% | -12.43% | 123% | 22128% |

Table 4. The performance of EE1-1.8.2 (Anchor is NNVC-3.0 filter set #1)

|  | All intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.29% | 0.75% | 109% | 123% |
| Class A2 | 0.19% | -0.29% | -2.59% | 106% | 124% |
| Class B | 0.14% | -0.15% | -0.63% | 105% | 122% |
| Class C | -0.21% | -1.60% | -1.01% | 104% | 124% |
| Class E | 0.09% | -1.19% | -1.21% | 106% | 123% |
| Overall | 0.04% | -0.60% | -0.91% | 106% | 123% |
| Class D | -0.25% | -1.52% | -1.84% | 103% | 125% |
| Class F | 1.35% | -1.58% | -1.67% | 103% | 105% |

References

- F. Galpin, T. Dumas, P. Bordes, E. François. AHG11: Small Ad-hoc Deep-Learning Library (SADL) update. JVET-AA0086, 27th Meeting, by teleconference, 13–22 July 2022.

- E. Alshina, R.-L. Liao, S. Liu, A. Segall. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-AB2016, 28th Meeting, by teleconference, Oct. 2022.

Patent rights declaration(s)

vivo Mobile Communication Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).