| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0120-v1 |
| --- | --- |

| Title: | Non-EE2: template based intra MPM list construction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal, Information |  |  |
| Author(s) or Contact(s): | Chuan Zhou Zhuoyi Lv Jinrong Zhang (vivo) | Email: | chuan.zhou@vivo.com zhuoyi.lv@vivo.com |
| Source: | vivo Mobile Communication Co., Ltd |  |  |

_____________________________

- Abstract

This contribution proposed an intra most probable modes optimization method. The intra modes derived from neighbouring blocks and DIMD are sorted with SAD cost. Then the directional modes with added offset are obtained from the first several available directional modes with the smallest cost. The experimental results are summarized as follows:

On top of ECM-7.0

AI: -0.xx% (Y), -0.xx% (U), -x.xx% (V),  10x% (EncT),  10x% (DecT)

Introduction

In ECM7.0 [1], a general MPM list with 22 entries is constructed first, and then the first 6 entries in this general MPM list are included into the PMPM list, and the rest of entries form the SMPM list. The first entry in the general MPM list is the planar mode. The remaining entries are composed of the intra modes derived from neighbouring blocks and DIMD, the directional modes with added offset from the first two available directional modes of neighbouring blocks, and the default modes.

Proposed Method

In this contribution, the MPM list is constructed by the following three steps:

- The planar mode and the intra modes derived from neighbouring blocks and DIMD are included into list0.

- The planar mode is added into the MPM list first. Then the remaining modes in the list0 are sorted in the ascending order, and up to 5 modes with the smallest SAD cost are added into the MPM list. Now the size of the list is 6, which is same as that of the primary MPM (PMPM) list.

- The offset directional modes obtained from the first several directional modes in the MPM list are added into MPM list until the size of the list is 22.

More specifically, the SAD cost is computed between prediction and reconstruction samples of the template. SMPM list is equally divided into four groups. The group index is encoded using truncated unary binarization. Three bins of the group index are coded with context and bypass coding is used for the mode index in the group with fixed two bits.

Experimental results

The proposed method is tested on top of ECM7.0 under the common test conditions [2].

Table 2. Simulation results of the proposed method

|  | All intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.11% | -0.06% | -0.21% | 103% | 106% |
| Class E | -0.17% | -0.15% | -0.14% | 103% | 103% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.15% | -0.25% | -0.09% | 103% | 106% |
| Class F | -0.11% | -0.06% | -0.21% | 103% | 106% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

References

- M. Coban, F. Le Léannec, M. Sarwer, and J. Ström, “Algorithm description of Enhanced Compression Model 7 (ECM 7),” JVET-AB2025, Oct. 2022.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

Patent rights declaration(s)

vivo Mobile Communication Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).