| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0120-v2 |
| --- | --- |

| Title: | Non-EE2: template-based intra MPM list construction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal, Information |  |  |
| Author(s) or Contact(s): | Chuan Zhou Zhuoyi Lv Jinrong Zhang (vivo) | Email: | chuan.zhou@vivo.com zhuoyi.lv@vivo.com |
| Source: | vivo Mobile Communication Co., Ltd |  |  |

_____________________________

- Abstract

This contribution proposes an intra most probable modes optimization method. Firstly, the intra modes derived from the neighbouring blocks and DIMD are sorted with SAD cost using a template, and added to the general MPM list in ascending order of SAD cost after the Planar mode. Then the remaining entries in the general MPM list are the sorted directional modes with added offset. There are two tests, the experimental results are summarized as follows:

On top of ECM-7.0

Test1: This method is disabled when the area of the current CU is greater than 1024.

AI: -0.07% (Y), -0.07% (U), -0.12% (V),  104% (EncT),  103% (DecT)

Test2: To reduce the complexity, this method is disabled when the area of the current CU is greater than 32.

AI: -0.07% (Y), -0.07% (U), -0.13% (V),  103% (EncT),  102% (DecT)

Introduction

In ECM7.0 [1], a general MPM list with 22 entries is constructed first, and then the first 6 entries in this general MPM list are included into the PMPM list, and the rest of entries form the SMPM list. The first entry in the general MPM list is the planar mode. The remaining entries are composed of the intra modes derived from neighbouring blocks and DIMD, the directional modes with added offset from the first two available directional modes of neighbouring blocks, and the default modes.

Proposed Method

In this contribution, the MPM list is constructed by the following three steps:

- The first entry in the general MPM list is always planar mode.

- The next entries are composed of the intra modes of the left (L), above (A), below-left (BL), above-right (AR), and above-left (AL) neighbouring blocks as shown in Figure 1, and DIMD modes, which are sorted in ascending order of SAD cost. Up to 5 modes with the smallest SAD cost are added. The SAD cost is computed between the prediction and the reconstruction samples of the template.

Figure 1. Neighbouring blocks (L, A, BL, AR, AL) used in the derivation of a general MPM list.

- The sorted directional modes with added offset are added into the general MPM list, and then the default modes, until the general MPM list with 22 entries is constructed.

The first 6 entries in this general MPM list are included into the PMPM list, and the rest of entries form the SMPM list.

Regarding the signalling method of the mode in SMPM list, SMPM list is equally divided into four groups and the group index is parsed first. Then a mode index is parsed to indicate which modes in the selected group is used.

Experimental results

The proposed method is tested on top of ECM7.0 under the common test conditions [2].

Table 2. Simulation results of the proposed method (Test1)

|  | All intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.16% | -0.12% | 104% | 102% |
| Class A2 | 0.02% | -0.02% | 0.00% | 104% | 102% |
| Class B | -0.02% | 0.01% | -0.10% | 104% | 103% |
| Class C | -0.11% | -0.06% | -0.21% | 103% | 106% |
| Class E | -0.17% | -0.15% | -0.14% | 103% | 103% |
| Overall | -0.07% | -0.07% | -0.12% | 104% | 103% |
| Class D | -0.15% | -0.25% | -0.09% | 103% | 106% |
| Class F | -0.09% | 0.05% | -0.10% | 102% | 102% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.36% | -0.08% | #NUM! | #NUM! |
| Class A2 | 0.00% | 0.18% | -0.01% | #NUM! | #NUM! |
| Class B | -0.04% | -0.14% | -0.16% | 100% | 100% |
| Class C | -0.05% | 0.00% | -0.02% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | -0.08% | -0.08% | #NUM! | #NUM! |
| Class D | -0.09% | 0.00% | -0.22% | 101% | 101% |
| Class F | -0.11% | -0.29% | -0.09% | 100% | 99% |

Table 3. Simulation results of the proposed method (Test2)

|  | All intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | 0.01% | -0.19% | 104% | 101% |
| Class A2 | -0.03% | -0.09% | -0.05% | 103% | 101% |
| Class B | -0.05% | -0.09% | -0.14% | 104% | 101% |
| Class C | -0.11% | -0.11% | -0.18% | 103% | 104% |
| Class E | -0.10% | -0.04% | -0.08% | 103% | 101% |
| Overall | -0.07% | -0.07% | -0.13% | 103% | 102% |
| Class D | -0.13% | -0.14% | -0.22% | 103% | 105% |
| Class F | -0.10% | 0.09% | 0.04% | 102% | 102% |

References

- M. Coban, F. Le Léannec, M. Sarwer, and J. Ström, “Algorithm description of Enhanced Compression Model 7 (ECM 7),” JVET-AB2025, Oct. 2022.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

Patent rights declaration(s)

vivo Mobile Communication Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).